3GPP TSG-RAN WG2 Meeting #122 	R2-2305110

# Incheon, South Korea, May 22-26, 2023

Agenda item:	7.8.5

Work Item:	NR_UAV-Core

Source: 	Qualcomm Incorporated

Title: 	Remaining aspects of PC5-based BRID and DAA support

Document for:     Discussion/Decision

# Introduction

RAN#99 approved a (revised) WI for NR UAV [1]. One of the objectives is the following:

3. Specify the support for UAV identification broadcast (BRID) in NR PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note:. UAV use of NR PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use.

RAN#99 also approved a parallel WI for LTE UAV [2], where one of the objectives is the following:

1. Specify the support for UAV identification broadcast (BRID) in LTE PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note: UAV use of LTE PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use.

In this contribution, we discuss some remaining aspects on PC5-based Broadcast Remote ID (BRID) and Detect and Avoid (DAA) message delivery for UAV.

# Quick intro of sol#5 from SA2 TR conclusions

Based on TR 23.700-58 section 8.2 and 8.3, SA2 conclusions propose to progress to normative phase PC5-based solutions for the Broadcast Remote ID (BRID) and Detect And Avoid (DAA) in accordance with solution #5 in the SA2 TR. SA2 expects RAN to define necessary enhancements for NR as described in solution #5.

Relevant aspects of solution#5 from SA2 TR are reproduced below. Full details are in TR 23.700-58 section 6.5:

Figure 6.5.2-1: Non-roaming 5G System architecture for U2X communication over PC5 and Uu reference points

-	U2X leverages V2X mechanisms as defined in TS 23.287 [9] to support BRID and direct DAA, with the differences described in this solution:

-	both LTE PC5 as defined in in TS 23.285 [17] and NR PC5 are supported, and the RAT selection is based on the U2XP.

-	Communications modes:

-	Broadcast communication mode is used for BRID.

-	Broadcast communication mode is used for DAA to advertise UAV information. Broadcast over PC5 or unicast over PC5 may be used between two or more UAVs for DAA deconfliction. Unicast over Uu via the U2X AS is not supported in this solution. Groupcast mode for NR based PC5 is not supported in this solution.

-	When NR PC5 is selected, connection-less groupcast communications may be used for DAA. Application layer managed groupcast are not considered in this release due to lack of clear requirements.

-	U2X is supported by an U2X Application Server which interfaces with the operator network via NEF, as in the case of the V2X Application Server.

NOTE 1:	It is expected a dedicated set of services will be defined.

NOTE 2:	Multiple deployment scenarios need to be allowed where the U2X AS and the USS serving a UAV are the same or different entities.

…

-	Similar to V2X, Tx Profiles or NR Tx Profiles are determined based on U2XP mapping ofU2X service types.

…

-	In addition to existing parameters for V2X, the radio parameters per PC5 RAT (i.e. LTE PC5, NR PC5) can be configured with Geographical Area, Altitude Limitation, and Validity timer:

-	This additional information may be needed to enable policing the use of PC5 depending on the specific location of the UAV.

<<skip>>

## Expected RAN2 impacts

SA2 documented the following potential UE impacts of solution#5 in their TR:

1. UE: In addition to the functions defined in TS 23.501 [2], the UE may support the following functions: - Report the U2X Capability (including DAA Capability) and PC5 Capability for U2X to 5GC over N1 reference point. - Indicate U2X Policy Provisioning Request in UE Policy Container for UE triggered U2X Policy provisioning. - Receive the U2X parameters from 5GC over N1 reference point. - Procedures for U2X communication over PC5 reference point. - Configuration of parameters for U2X communication. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over U2X1 reference point from the U2X Application Server.

Note that the above list is not limited to impacts to RAN2 only; there may be impacts to other groups also. From RAN2 point of view, to enable PC5-based BRID and DAA, we think the following would be needed to be specified by RAN2:

- Configuration of parameters for U2X communication for the required scenarios

- Procedures for U2X communication over PC5 reference point.

- UE capability signaling

The required configuration parameters depend on the scenarios to be supported and requirements / traffic characteristics of BRID and DAA transmissions.

The exact definition of the BRID and DAA messages is outside of the scope of RAN2 (or 3GPP); however, BRID messages are expected to be periodically transmitted by the UAVs as higher-layer payload.

BRID and DAA messages are expected to be periodically transmitted by the UAVs as higher-layer payload.

The exact definition of the BRID and DAA messages is outside of the scope of RAN2 (or 3GPP).

RAN2 needs to study traffic characteristics of BRID and DAA messages and whether the current configurations can support them.

Accordingly, RAN2#121 discussed and agreed the following:

Agreements:

-	PC5-U is used to support BRID for UAV

-	Support both in-coverage and out-of-coverage scenarios

-	Mode 2 will be supported.  FFS whether further mode 1 will be supported.

-	FFS whether separate pools are needed

-	FFS whether current configurations can support UAV requirements

Further, RAN2#121bis-e discussed and agreed the following:

Agreements:

- DAA can be supported using the same framework as used for BRID transmission over the LTE and NR PC5 interface, without any specific enhancements.

- LTE PC5 Mode-4 resource allocation is supported, and LTE PC5 Mode-3 is not supported

- NR PC5 mode-1 is not supported

- For LTE PC5, we will follow the NR PC5 framework agreements, unless explicitly identified e.g. a strong technical reason

- RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators

- As a baseline, we will use the existing V2X QoS framework.  FFS whether different resource pools are needed for UAV services

- No further enhancement on PC5 range for A2X broadcast services will be pursued in this release

- We will not investigate interference for BRID and DAA broadcast

In the following, first we focus on the traffic characteristics and scenarios for BRID transmission followed by some discussion on DAA.

# BRID and DAA transmission characteristics

## BRID/DAA message size

Following is from Section IX of FAA final rules [2]:

A remote identification broadcast module must broadcast the following message elements: A unique identifier (the serial number assigned to the remote identification broadcast module); an indication of the unmanned aircraft latitude, longitude, and geometric altitude; an indication of the unmanned aircraft take-off location latitude, longitude, and geometric altitude; an indication of the unmanned aircraft velocity; and a time mark.

As seen above, FAA does not explain about the size of the BRID message but only describes about the content.

A relevant specification reference for message size is ASTM F3411-19 [4]. The standard specifies that a BRID packet may include one or more sub-messages each of size 24 bytes distinguished by their one-byte header. The types of sub-messages are summarized in the following table:

| Msg Name/ Identifier | Content | Size (including 1 Byte header) | Note |
| --- | --- | --- | --- |
| Basic ID msg | ID type, UAV type, UAS ID | 25 bytes | 16 different UAV types (4 bits) |
| Location/Vector msg | Operational status (ground, airborne etc.), flags, direction, speed, vertical speed, Lat, Long, Pressure, Geodetic Alt, height, Hor/ver accuracy, Baro alt accuracy, speed accuracy, timestamp, timestamp accuracy | 25 bytes |  |
| Authentication msg | Auth Type, page num, page count, length, timestamp, auth data/signature | Each page 25 bytes, Upto 5 pages | Optional unless required by location jurisdiction |
| Self-ID msg | Operator (remote pilot) identity/purpose in ASCII text | 25 bytes | Optional unless required by location jurisdiction |
| System msg | Flags, remote pilot lat/long, num# of UAV in area (count), area radius, area ceiling, area floor | 25 bytes | Optional unless required by location jurisdiction |
| Operator msg | Remote pilot ID type, ID | 25 bytes | Optional unless required by location jurisdiction |
| Message pack | Series of multiple of above messages aggregated/concatenated in one msg, num of msg in pack, followed by [hdr][msg][hdr][msg]… | Up to 250 bytes | Optional unless required by location jurisdiction |

Based on the above, while the minimum size of BRID message could be 50 bytes (including only the Basic ID and Location/Vector msg), it is reasonable to assume that typical size of a BRID message is expected to be about 250 Bytes.

As discussed in section 2, the exact definition of the BRID and DAA messages is outside of the scope of RAN2 (or 3GPP). But, as relative comparison, the BRID message content mainly includes the UAS ID content, location, status, etc. whereas DAA message content is expected to include the UAS ID content (which may be similar to BRID broadcast content) and additional collision avoidance/deconfliction specific information.

Typical size of a BRID/DAA message is expected to be about 250 Bytes.

## BRID/DAA message periodicity

Periodicity of BRID message is expected to be at least 1 message per second (1 hertz), See [3] section 5:

The FAA proposed a transmission rate of at least 1 message per second (1 hertz) as the minimum transmission rate for the remote identification message elements for standard remote identification UAS. The proposed transmission rate applied to both the message elements transmitted to a Remote ID USS and broadcast, and is the minimum rate at which the remote identification message would be either broadcast or transmitted to a Remote ID USS by the remote identification equipment.

The FAA is adopting this requirement as proposed with respect to the broadcast message set. As discussed in section VII.A of this preamble, the FAA eliminated the requirement to transmit message elements through the internet to a Remote ID USS. Accordingly, the FAA is adopting this rule without reference to a transmission rate requirement for internet-based transmissions.

Similarly, [4] subclause 5.4.4 indicates that for broadcast messages, dynamic messages shall be sent at least every 1 second, static messages shall be sent at least every 3 seconds and the maximum potential time elapsed since the time of applicability of the dynamic fields in the Location/Vector Message shall be no older than 1 second.

For timely information updates, one can expect that DAA messages should be transmitted at least same or more frequently compared to BRID which depends on the density, speed as well as scenarios involving UAVs right around the corner with non-LOS conditions.

Periodicity of BRID/DAA message is expected to be at least 1 message per second (1 hertz).

Based on above discussion, following is proposed:

Assume typical message size of 250 bytes and periodicity of 1s for changes needed to support PC5-based BRID and DAA broadcast.

# Further considerations on SL resource configuration for UAV

RAN2 briefly discussed whether separate resource pools would be needed for PC5 broadcast of BRID. RAN2 also agreed that PC5 mode 2 will be supported.

## Separate SL resource pool for BRID/DAA

For PC5 resource allocation, 2-stage SCI is used. The second stage SCI format 2-A includes a field about ‘Cast type’. The existing SL resource allocation is only for different Cast types and predefined priority associated with PQI, but no QoS flows or traffic types with requirement of different communication range with interference coordination.

BRID/DAA message transmissions may require different QoS (e.g. different PQI/ packet delay budget/ packet error rate) compared to legacy SL use cases. Therefore, in our view, separate SL resource pool for BRID/DAA broadcast is beneficial.

Due to different potential QoS characteristics, transmit power etc., separate SL resource pool for BRID and DAA broadcast is beneficial.

Once RAN2 agrees on the principal that separate SL resource pool can be configured for BRID, details can be discussed further on how to configure it. Therefore, we propose:

Separate SL resource pool for BRID and DAA broadcast is supported.

## Height dependent SL transmission configuration

Note that for V2X, different (pre)configurations for PSSCH transmission parameters for different speed are currently supported. For low speed and high speed, PSSCH tx parameters including MCS, sub-channel number, retransmission number, max power can be separately (pre)configured.

SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16

SL-PSSCH-TxConfig-r16 ::=        SEQUENCE {

sl-TypeTxSync-r16                SL-TypeTxSync-r16             OPTIONAL,    -- Need R

sl-ThresUE-Speed-r16             ENUMERATED {kmph60, kmph80, kmph100, kmph120,

kmph140, kmph160, kmph180, kmph200},

sl-ParametersAboveThres-r16      SL-PSSCH-TxParameters-r16,

sl-ParametersBelowThres-r16      SL-PSSCH-TxParameters-r16,

...,

[[

sl-ParametersAboveThres-v1650    SL-MinMaxMCS-List-r16         OPTIONAL,    -- Need R

sl-ParametersBelowThres-v1650    SL-MinMaxMCS-List-r16         OPTIONAL     -- Need R

]]

}

V2X supports speed dependent SL transmission configuration.

For UAVs, different heights result in variant channel conditions and interference impact. Therefore, different configurations of PSSCH tx parameters for different UE heights should be supported. Whether to support only one height threshold (and consequently one configuration below and one above threshold) or multiple height thresholds/ranges can be discussed further.

Height-dependent SL transmission configuration is supported. Details FFS.

# Summary

In this contribution, we discussed on PC-5 based transmission of BRID and DAA and made the following observations:

Observation 1.	BRID and DAA messages are expected to be periodically transmitted by the UAVs as higher-layer payload.

Observation 2.	The exact definition of the BRID and DAA messages is outside of the scope of RAN2 (or 3GPP).

Observation 3.	Typical size of a BRID/DAA message is expected to be about 250 Bytes.

Observation 4.	Periodicity of BRID/DAA message is expected to be at least 1 message per second (1 hertz).

Observation 5.	Due to different potential QoS characteristics, transmit power etc., separate SL resource pool for BRID and DAA broadcast is beneficial.

Observation 6.	V2X supports speed dependent SL transmission configuration.

Based on the discussion and observations, we propose:

Proposal 1.	Assume typical message size of 250 bytes and periodicity of 1s for changes needed to support PC5-based BRID and DAA broadcast.

Proposal 2.	Separate SL resource pool for BRID and DAA broadcast is supported.

Proposal 3.	Height-dependent SL transmission configuration is supported. Details FFS.

# References

[1] RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

[2] RP-230783, New WID: Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

[3] Remote Identification of Unmanned Aircraft, FAA, DOT, Final rule, Published on Federal Register on January 1, 2021. Retrieved Feb. 2023 from https://www.govinfo.gov/content/pkg/FR-2021-01-15/pdf/2020-28948.pdf.

[4] F3411-19, Standard specification for Remote ID and Tracking, ASTM International, Dec. 2019