3GPP TSG-RAN WG2 Meeting #122                                                             R2-2305119

Incheon, Korea, May 22 – 26, 2023

Agenda item: 		7.4.2.3

Source: 		Qualcomm Incorporated

Title: 	Dynamic switch in LTM

Document for:	Discussion

# 1	Introduction

In RAN2#121, the following agreements have been achieved [1]:

No consensus to support HARQ continuation (and in order to resume discussion some new input may be needed, e.g. quantitative evidence of a serious problem). To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

This contribution discusses L2 reset at LTM execution.

# 2	Determination of L2 reset at LTM execution

RAN2#121 agreed that the UE determines whether to do L2 reset at LTM execution based on prior RRC configuration. This can be done as follows:

- Step 1: RRC provides partitioning of the candidate LTM configurations into sets during LTM preparation.

- Step 2: The LTM MAC CE indicates a candidate LTM configuration index.

- Step 3: The UE checks if the configuration index of the serving cell group and the configuration index of the candidate cell group belong to same or different sets as per the RRC configuration in Step 1:

  - If same set: the UE does NOT perform L2 reset.

  - If different sets: the UE performs L2 reset.

Proposal 1a: To determine L2 reset, RRC provides partitioning of the candidate LTM configurations into sets during LTM preparation. At LTM execution, the UE performs L2 reset if the source configuration and the target configuration fall into different sets. The UE skips L2 reset otherwise.

L2 reset includes MAC reset, RLC reestablishment and PDCP recovery. Having separate configurations per reset type is more flexible for the network and should be supported:

- This is true for both RLC reestablishment and PDCP recovery.

- For MAC reset:

  - If partial reset is agreed: a separate configuration for MAC reset shall be provided to the UE during LTM preparation that partitions the candidate LTM configurations into sets. The UE performs partial reset if it switches configurations within a set and performs full reset otherwise.

  - If partial reset is not agreed: this implies the UE always performs MAC reset at every LTM execution. No configuration for MAC reset is necessary.

Proposal 1b: Separate RRC configuration partitions the candidate configurations into sets to determine whether to perform RLC reestablishment.

Proposal 1c: Separate RRC configuration partitions the candidate configurations into sets to determine whether to perform PDCP recovery.

Proposal 1d: For MAC reset:

- If partial MAC reset is supported, separate RRC configuration partitions the candidate LTM configurations into sets. The UE performs partial MAC reset if it switches configurations within a set and performs full MAC reset otherwise.

- If partial MAC reset is not supported, full MAC reset is always performed and no corresponding RRC configuration is necessary.

# Conclusion

This contribution discussed L2 reset at LTM execution. The following proposals have been made:

Proposal 1a: To determine L2 reset, RRC provides partitioning of the candidate LTM configurations into sets during LTM preparation. At LTM execution, the UE performs L2 reset if the source configuration and the target configuration fall into different sets. The UE skips L2 reset otherwise.

Proposal 1b: Separate RRC configuration partitions the candidate configurations into sets to determine whether to perform RLC reestablishment.

Proposal 1c: Separate RRC configuration partitions the candidate configurations into sets to determine whether to perform PDCP recovery.

Proposal 1d: For MAC reset:

- If partial MAC reset is supported, separate RRC configuration partitions the candidate LTM configurations into sets. The UE performs partial MAC reset if it switches configurations within a set and performs full MAC reset otherwise.

- If partial MAC reset is not supported, full MAC reset is always performed and no corresponding RRC configuration is necessary.

# References

- Chairman Notes; TSG RAN WG2 Meeting #121