3GPP TSG-RAN WG2 Meeting #122			R2-2305125

Incheon, Korea, May 22-26, 2023

Source:	Qualcomm Incorporated

Title:	FDM IDC Enhancements

WID:	NR_IDC_enh-Core

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss FDM solutions that can be used to enhance the existing IDC solutions in Rel-18 .

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi).

- Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2)

Note: Enhancements to FDM solution is prioritized.

- Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4).
Note: The TDM solution is considered complementary to the FDM solution.

- Specify RRM requirements for TDM solution (RAN4)

Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

In Rel-18, it was agreed to introduce IDC enhancements in WID [1]:

In RAN2 121 the following agreement was reached [2]:

- The NR values of long/short DRX cycle and start offset are used for periodic pattern. RAN2 will not introduce new DRX value for network configuration for IDC purpose. - The slot offset with 1/32ms granularity is included in UEAssistanceInformation-v18xy-IEs for start offset. - Multiple periodic patterns for IDC are not supported in R18. - Per CG pattern is supported for MR-DC. SN can configure the UE to report the TDM assistance information directly to SN, either through SRB 1 (if SRB3 is not configured) or SRB 3. - FFS whether any additional coordination is needed for network to resolve the problem when network receives the reporting from UE. - Slot as time unit for autonomous denial - Agree to send LS to RAN4, indicate the progress in RAN2 - For NR-DC, per CG idc-AssistanceConfigTDM-r18 is introduced to indicate whether TDM assistant information needs to be reported. - For NR-DC, per CG idc-AssistanceConfigFDM-r18 is introduced to indicate whether FDM assistant information needs to be reported. FFS on dependency between FDM and TDM configuration. - The values of drx-onDurationTimer in NR is used as the baseline for active Duration in UE assistant information. FFS on other values. - The same values of validity period and number of denial slots as in LTE is reused. FFS on other values. - The autonomous denial configuration is per CG.

An offline email discussion is ongoing to address leftover issues [3].

# TDM dependence on FDM

One issue that was discussed in [3] is whether FDM configuration and TDM configuration can be configured independently. In our view, We have already provided a TDM configuration in the CR and agreed that TDM capability is separate from FDM capability. The procedural text in the current CR as well does not require FDM to be supported for TDM reporting so it does not seem a spec change is needed from the current CR.

Our motivation to deploy TDM with no FDM is that some deployments like n255 for NTN cannot benefit from FDM solution so in this case this would be a standalone TDM solution.

Observation 1: RAN2 has already agreed on separate configuration and capability for TDM reporting.

Proposal 1: TDM configuration and reporting is independent of FDM.

# Autonomous Denials

The current allowed autonomous denial configuration is not very useful due to two reasons:

- UE cannot request it, gNB must use periodic requests or some other apriori information  to guess if the UE needs autonomous denials

- Configuration is only aperiodic one-time for a maximum of 2000 slots validity period after configuration.

Given that it is impossible for gNB to know when IDC events happen at the UE with no UE aiding except for a periodic pattern request, it does not make sense for gNB to be able to pinpoint when the UE needs that configuration. The more rational course of action is that the gNB would keep configuring autonomous denials for the UE via RRC signalling to “renew” the autonomous denials validity period.

Observation 2: Autonomous denials as an aperiodic configuration that the gNB can provide without UE assistance is not very useful since gNB cannot determine the right validity period.

Thus, in our view, to avoid continuous RRC renewing of autonomous denial validity period via RRC (the deployment that makes sense), we think the gNB can set up an automatically renewing autonomous denial configuration until cancelled. This way, the gNB accepts that some UL denials may happen as long as this configuration is there.

Proposal 2: Network is allowed to make the autonomous denial configuration “recurring”, i.e., configuration renews automatically every validity period until cancelled by RRC signalling.

Another issue that is salient in autonomous denials is whether MN-SN coordination is needed. Since MN can configure SN it needs to tell SN about the active autonomous denial configuration so that the SN will not misinterpret denied UL slots.

Observation 3: When MN configures autonomous denials for SN, SN would need to know of the configuration in order not to misinterpret denied UL slots which would affect PDCCH link adaptation.

Proposal 3: MN should inform SN of an autonomous denials configuration that affects SN UL.

# Periodic Pattern request

The current format of periodic pattern request is stable, however, there is an FFS on all the values the UE can include on cycleLength-r18  in IDC-TDM-Assistance-r18. For this case, we think there are two key values missing for the cycleLength-r18  that the UE may need to report.

- 100ms: The length of WLAN beacon periodicity

- 96 ms: The length of an UWB frame.

We think this would be a straightforward addition to available field values in the IDC TDM assistance without any change to the basic operation.

Proposal 4: Values 100ms and 96ms are added to the possible cycleLength-r18  in IDC-TDM-Assistance-r18 to cover WLAN and UWB, respectively.

# Conclusions

Observation 1: RAN2 has already agreed on separate configuration and capability for TDM reporting.

Proposal 1: TDM configuration and reporting is independent of FDM.

Observation 2: Autonomous denials as an aperiodic configuration that the gNB can provide without UE assistance is not very useful since gNB cannot determine the right validity period.

Proposal 2: Network is allowed to make the autonomous denial configuration “recurring”, i.e., configuration automatically renews every validity period until cancelled by RRC signalling.

Observation 3: When MN configures autonomous denials for SN, SN would need to know of the configuration in order not to misinterpret denied UL slots which would affect PDCCH link adaptation.

Proposal 3: Include information between MN and SN about autonomous denials in Internode coordination.

Proposal 4: Values 100ms and 96ms are added to the possible cycleLength-r18  in IDC-TDM-Assistance-r18 to cover WLAN and UWB, respectively.

# References

- RP-213589: New WI: In-Device Co-existence (IDC) enhancements for NR and MR-DC

- RAN2 121 Chairman Notes

[3] R2-23xxxxx [Post121][655][IDC] Discussion on Leftover issues for IDC (xiaomi)