3GPP TSG-RAN WG2 Meeting #122			R2-2305126

Incheon, Korea, May 22-26, 2023

Agenda Item: 7.3.4

Source:	Qualcomm Incorporated

Title:	IDC UE Capability

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss FDM solutions that can be used to enhance the existing IDC solutions in Rel-18 .

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi).

- Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2)

Note: Enhancements to FDM solution is prioritized.

- Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4).
Note: The TDM solution is considered complementary to the FDM solution.

- Specify RRM requirements for TDM solution (RAN4)

Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

In Rel-18, it was agreed to introduce IDC enhancements in WID [1]:

In RAN2 121 the following agreement was reached [2]:

UE capabilities - Rel-18 IDC UE capability(ies) defined in NR side is/are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF. - In NR side, 3 capability bit is introduced for FDM, periodic pattern and autonomous denial separately. - The pre-requisite of autonomous denial is FDM solution (R16 or R18) or periodic pattern.

An offline email discussion took place [3] with the following outstanding proposals:

Proposal 1: (7 vs 1) The pre-requisite of Rel-18 IDC FDM solution is Rel-16 NR IDC (inDeviceCoexInd-r16). Proposal 2: (6 vs 2) The pre-requisite of Rel-18 IDC periodic pattern is Rel-16 NR IDC (inDeviceCoexInd-r16). Proposal 3: The pre-requisite of Rel-18 IDC autonomous denial is Rel-16 NR IDC (inDeviceCoexInd-r16).

With the current proposed feature list in the running CR [4]:

| inDeviceCoexIndAutonomousDenial-r18 Indicates whether the UE supports IDC autonomous denial as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| inDeviceCoexIndFDM-r18 Indicates whether the UE supports reporting of affected NR carrier frequency ranges in IDC assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndTDM-r18 Indicates whether the UE supports reporting of IDC TDM assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |

This CR has been endorsed offline in RAN2 121 bise [5].

# UE Capability

From our view, the capability issue is quite stable now and RAN2 conclude it for this WI  based on the proposals in  [3].

Proposal 1: RAN2 to confirm that IDC FDM, IDC Periodic patterns, and autonomous denials all have the same pre-requisite which is  inDeviceCoexInd-r16.  No further discussion on UE capability is needed.

# References

- RP-213589: New WI: In-Device Co-existence (IDC) enhancements for NR and MR-DC

- RAN2 121 Chairman Notes

[3] Post121][654][IDC] Capability CRs on IDC (Intel)	Intel Corporation	discussion	Rel-18	NR_IDC_enh-Core

[4] R2-2302979 38.306 running CR for Rel-18 IDC UE capabilities Intel Corporation	draftCR	Rel-18	38.306	17.4.0	B	NR_IDC_enh-Core

[5] RAN2 121 bise Chairman Notes