3GPP TSG-RAN WG2 Meeting #122	R2-2305130

Incheon, Korea, May 22-26, 2023

Agenda item:	    7.23

Source: 		Qualcomm Incorporated

Title: 	Burst Arrival Time (BAT) offset derivation.

Document for:	Discussion

# Introduction

In this contribution we discuss the BAT offset derivation relating to the new Timing Resiliency and URLLC Enh WI introduced in [1] as follows:

3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication. Note 3: Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

RAN2 already discussed this in RAN2 120 and RAN2 121 based on SA2 LS in [2] [3] with no consensus reached. In RAN2 122, the following agreements were reached:

Agreements

=>	We will wait for XR to progress and see if we can use the existing mechanism. RAN2 will strive to re-use existing mechanism or rely on gNB to determine the information (i.e. aim to not introduce new UE specific BAT reporting)

# Problem Description

In order to minimize the User Plane buffering delay of low latency applications, AF and NG-RAN attempt to align their understanding of the burst arrival time, correcting for any timing discrepancy in their synchronization in the process. When AF gets the feedback, that may be reactive or proactive as explained below, it can adjust the application to adapt the BAT time to the transmission opportunity. This can be done at both NG-RAN side for DL traffic and UE side for UL traffic.

For low latency communication, SA2 has agreed to support the following two methods to assist the RAN in adapting DL scheduling to low-latency traffic arrival time:

- Reactive feedback: Based on the observed user plane burst arrivals, RAN can adapt DL resource scheduling to ensure low latency of delivery.

- Proactive feedback: Burst Arrival Time is provided from AF to RAN as part of TSCAI in control signalling. RAN can then adapt resource scheduling to ensure low latency between Burst Arrival Time and RAN scheduled DL resources.

SA2 is then considering whether to extend DL reactive mechanisms to UL, and propose whether some feedback from RRC is required as described in clause 8.4 of TR 23.700-25 as there is no BAT in TSCAI to the RAN in Reactive feedback.

# RAN BAT Derivation

Given the agreement that RAN2 aims not to introduce new signalling, we would like to present our views on how existing signalling can be used to derive BAT at the RAN.

## Using Existing signalling

Given that RAN2 currently is waiting for XR progress, it is useful to assess how the MAC BSR can perform to derive BAT for periodic deterministic traffic.

gNB can use existing BSR signalling to derive BAT. Simply put as shown in the figure, gNB would have a general idea on the BAT based on BSR collection. To obtain more accurate information, gNB can schedule additional BSR occasions to derive BAT estimations with arbitrarily high accuracy.

Observation 1: BSR can be used to derive BAT/BAT offset with any required accuracy.

In our view, BSR is sufficient since the traffic is periodic and deterministic, and RAN only needs a few BSR reports to derive BAT within any realistic accuracy (since the system already has inherent delay floor between the application and User plane buffer). Thus, it would be useful for RAN2 to agree that BSR would be an acceptable baseline.

Note also that compared to new signalling proposed by some companies, BSR is more robust to artifacts such as burst size variation and jitter as explained in our earlier contribution [3].

Proposal 1: RAN2 to take BSR as an acceptable baseline for deriving BAT at RAN. XR enhancements, if found useful, can be adopted as a signalling optimization.

# Conclusion

Observation 1: BSR can be used to derive BAT/BAT offset with any required accuracy.

Proposal 1: RAN2 to take BSR as an acceptable baseline for deriving BAT at RAN. XR enhancements, if found useful, can be adopted as a signalling optimization.

# References

[1]   RP-230754 New WID on NR Timing Resiliency and URLLC enhancements

[2] RAN2-121bis Chair notes

[3] R2-2302933 UL BAT derivation- Qualcomm Incorporated