3GPP TSG-RAN WG2 Meeting #122	R2-2305145

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.16.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	AI/ML Architecture

SID:	FS_NR_AIML_air (Rel-18)

Document for:	Discussion and Decision

# Introduction

A new agenda item was added as of RAN2#121-bis-e for AIML architectural aspects.

7.16.2 AIML methods Explore AIML methods that are expected applicable to this SI and their expected or potential architecture (allocation of functionality to entities), Identification of Models, other framework aspects, impact on RAN2. Most of LCM is in RAN2 scope. Both general aspects and use-cases specific aspects are applicable (for use cases in scope). Aspects of on-line/real-time training are deprioritized at current meeting. Please input to 7.16.2.x 7.16.2.1 Architecture General Model ID: 1a. Attempt to agree a list of cases for which a model ID shall/should be used. 1b. Can discuss also model meta-data that can be useful and the detailed cases/contexts of such usefulness. Should take into account R1 progress if any. At current meeting: No need to discuss whether metadata is a sub-part of a structured model ID or whether we have other IDs, algorithm ID, function ID etc. Mapping of Functionality to entities. 2: Identification of justifications and issues (tangible) that need the definition of architecture, function mapping, and possibly later 3GPP procedure support (e.g. a: for cases of off-line training, is there any reason to specify where training takes place, e.g. b: for cases of network-only models, what support in 3GPP specifications is expected … etc). 3: Review of RAN1 logical/functional architecture (can also consider other inspiration e.g. from R3 SA2), with logical/functional entities their relation etc. 4: At this meeting, expect that the detailed mapping to physical entities is discussed per functionality (for Data Collection, for Model tranfser/delivery, per LCM purpose etc) as below.

In this paper, we will discuss architectural aspects related to the functionality and model identification. Further we emphasize on functionality-based life cycle management (LCM) for ML-enabled features and functionality support indication as well as associated information.

# Discussion

## Frameworks for functionality-based LCM

In the last meeting [5], we presented our view on both the functionality based LCM, as well as the model-ID based LCM frameworks, together with the corresponding modules. As discussed in our last t-doc, the model-ID based LCM requires the management of a large number of physical model(s) which are expected to be deployed to enable the AIML-based features. The management of these models becomes particularly cumbersome in the case of the two-sided models. Seeing the complications associated with the model-ID based LCM, we first focus on studying the functionality-based LCM, which is simpler to enable.

Observation 1: Both the one-sided and two-sided model ID based LCM frameworks are significantly more complex than the functionality-based LCM.

Figure 1 provides an illustration of one-sided functionality based LCM framework for the one-sided use cases. Furthermore, Figure 2 provides an illustration of the LCM framework for two-sided use cases, where the inference and data collection are split between the UE-side and gNB-side. The main blocks for both the frameworks are the data collection, inference, and functionality management blocks.

Figure 1: A simplified framework for one-sided functionality-based LCM.

Figure 2: A simplified framework for two-sided functionality-based LCM.

For the use cases concerning the UE-sided models and the UE-part of the two-sided models, the functionality on UE-side includes two types of functions:

- (ML) Inference: functions required to execute the functionality at the UE-side in inference operating mode.

- Data collection: functions required to enable the data collection and pre-processing of inference data and collection and post-processing of monitoring data.

The functionality management is located at the NW/gNB side and implements the management functions required by the gNB/NW to register, configure, select/activate/switch and monitor the functionalities executed at the UE side. The functionality on the gNB-side includes at least monitoring data collection function(s) i.e., function(s) required to enable the collection and post-processing of monitoring data received from the functionality at the UE-side.

The ML inference and data collection functions located in the same node, NW/gNB or the UE node, communicate with each-other by using implementation specific solutions i.e., there is no need for RAN2 to study these internal protocols.

Observation 2: The functionality inference and functionality data collection when located in the same node NW/gNB or UE node, can communicate with each other using implementation specific solutions i.e., do not require any RAN2 specifications.

In one implementation, data collection can be enabled for both UE-side functionality and gNB-side functionality, to enable higher configuration flexibility of the LCM purposes, e.g., for cases where one gNB-side functionality is interacting with UE-side functionalities located in different UEs.

For UE-side inference, the inference will reside in the UE and interact with functionality management in the gNB. The functionality management may comprise of the following functions:

- Registration: registration in the LCM management module of a given ML-enabled Feature/Functionality based on conditions indicated by the UE capability. This procedure could re-use most the existing UE capabilities reporting framework

- Configuration: configuration of a registered ML-enabled Feature/Functionality using the conditions (configuration parameters) indicated by the UE capability.

- Monitoring: monitoring signaling procedure(s) for configured ML-enabled Feature/Functionality using the conditions (monitoring parameters) indicated by the UE capability.

- Selection, (de)activation, switching: signaling procedure(s) needed to allow the LCM management module operational control of configured ML-enabled Feature/Functionality.

Proposal 1: RAN2 to consider Figure 1 as a baseline for one-sided functionality-based LCM framework.

Proposal 2: RAN2 to consider Figure 2 as a baseline for two-sided functionality-based LCM framework.

Proposal 3: RAN2 to study the protocol specifications required to enable the communication between the functionality management (gNB-side) and the UE-side functionality to ensure inter-operability, at least for registration, configuration, monitoring and selection, (de)activation, switching of Functionality (at) UE-side.

## 2.2	Functionality identification

Figure 3 illustrates how an ML-enabled feature or feature group (FG) is activated with a certain functionality. The functionality is identified by configurations based on conditions reported by UE via UE capabilities. This section provides us a deeper understanding of functionality identification that enables which ML enabled features to be used.

Figure 3: ML-enabled Feature: Proposed relationship between ML-enabled Feature/FG, ML Functionalities enabled by configurations based on applicable conditions (conditions indicated by UE capability), and logical ML Models (set of physical ML Models) associated w

The RRC UE capability exchange procedure will indicate support for conditions that enable AIML features such as beam management, CSI compression, CSI prediction, and positioning. A specific configuration of an AIML-enabled feature is captured by a functionality, which supports a set of conditions. Logical model(s) can be enabled through one or more physical model(s) which are implemented in the device. Moreover, one or more logical model(s) can implement a functionality, which is characterized by a set of conditions. Obtaining a deeper understanding on these additional conditions together with the type of LCM purposes that are applicable to logical models is FFS and waits for RAN1 progress on the matter.

Observation 3: The definition of additional conditions on logical AIML models depends on RAN1 progress, but might include, e.g., applicable scenario.

Proposal 4: Functionalities, defined and identified by sets of conditions, will be signalled through the RRC capability exchange procedure for the beam management, CSI compression, and CSI prediction use cases.

Proposal 5: Functionalities, defined and identified by sets of conditions, will be signalled through the LPP capability exchange procedure for the positioning use-case.

The functionality identification procedure allows us to understand the interaction between the NW and UE. UE signals applicable conditions/conditions and additional conditions. In the following, we listed a few general conditions

Table 2-2-1: General conditions on different configurations.

| Conditions on configurations | Description |
| --- | --- |
| Functionality configuration | - indicates the max number of configured/activated functionalities, - delays in activating/switching of functionalities, and Generalization condition of functionalities for the same ML-enabled Feature/FG, - configuration parameters for the procedure activation/deactivation/switching/fallback procedures, - including corresponding delay budgets and including optional indication of any non-ML operations/algorithms involved in the model functionality e.g., as an indication of the expected delay budget for such operations |
| Performance monitoring | indicates UE support for NW-sided functionality monitoring, and conditions on related configuration options for functionality performance monitoring, including system and intermediary KPIs to be used for functionality-based LCM monitoring purposes |
| Inference input(s) | input data type/source and preparation/pre-processing including an indication of any delay-sensitive ML-specific data processing to be performed e.g., as an indication of the expected delay budget for such operation |
| Inference output(s) | output data and post-processing (optional) including an indication of any delay-sensitive ML-specific output post-processing is performed e.g., as an indication of the expected delay budget for such operation |
| Functionality validation procedure | configuration parameters for the procedure required to validate the performance of the Functionality e.g., after ML Model update or switch (NW-initiated, UE-triggered) |
| Supporting fallback | indicates UE support for one or more fallback features (triggers/events, delays, etc.) |
| Context information, if applicable | indicates UE support for monitoring and reporting conditions for UE-side inference context e.g., radio KPIs not explicitly linked to the ML Functionality, and/or non-radio metrics (position, movement, temperature, etc.) |

The above table can be used as a starting point where we can study the different configurations needed to identify a functionality.

Proposal 6: Use the table to facilitate the configuration and procedures needed for functionality to enable each use case.

## 2.3	Logical model and Logical Model identification

In our understanding, a logical model is a collection of one or more physical models that support conditions defined by one or more functionalities, and additional conditions of an ML enabled functionality. That is, a set of physical models deployed in the UE are trained for the same applicable conditions or a subset of all possible combinations. We can realize this with a simple example shown in Figure 4. The example shows a logical model with two sets of two physical models. The set of physical models is selected by configuration from the gNodeB, e.g., number of top beams to report. Which of the physical models is used could be determined by a KPI internal to the UE. The important thing to note is that each of the physical models that can be autonomously switched between share the same inputs and outputs such that they will be compatible with the configuration provided by the gNodeB.

Figure 4: An illustrative example of logical model. In this example, logical model A is being used for performing the inference function in the beam management use case. The input is SSB-RS for the logical model A is propagated to the appropriate physical model

In our example, Figure 4, the beam management use case is depicted. Assuming that only this logical model A exists, the UE would report in its capabilities for beam management that it supports SSB-RS as the measurement quantity, and that it supports two options for nTopKBeams (e.g., 2 beams and 4 beams). If nTopKBeams is the only configurable parameter, it could be said that the UE supports two functionalities for the beam management feature through the implementation of one logical model. Inside of the logical model are two physical models, but the existence and details of these physical models do not normally need to be exposed to the network. Additionally, more logical models could be present on the UE, and they could carry additional conditions. How these additional conditions could be used by the UE or the network in logical model-based LCM is FFS.

Proposal 7: Use Figure 4 as a starting point for discussing the definition of a logical model.

To support LCM purposes for logical models, a logical model ID would be required such that the gNodeB could address the models, e.g., to switch between logical model 0 and logical model 1. Because logical models enable functionalities, these logical model IDs can be unique per functionality, and any functionality-based LCM purpose should be addressed separately.

Proposal 8: For each of the Functionalities developed in the context of air interface, the Model identification (a process/method of identifying an (logical) ML Model for the common understanding between the NW and the UE), for UE-side models and UE-part of two-sided models, contains the following information elements,

- Logical Model ID(s): a labels/tags/UUIDs which can uniquely identify the logical ML Model(s) (each being set of physical ML models) as defined by RAN1 in [4] either within the ML-enabled Feature (across several potential Functionalities) or only within a specified Functionality, or Feature/FG, and

- The use case specific additional conditions (e.g., dataset ID or context information) as determined/identified between UE-side and NW-side

## 2.4 Meta information

According to last meeting agreement, we have following discussion on meta information:

- R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, - etc. may be required for model management and control, and should, as a starting point, be part of meta - information. Open issue 6: RAN2 to further study the content of meta data, the following bullets can be considered as the starting point for future study: a, model input info; b, model output info; c, model version info; d, model format info; e, required AI capability; f, vendor info; g, applicable scenario, configuration, site information; h, computational complexity: FLOPs, level of pre-/post-processing; i, model complexity: number of real-value model parameter, number of real-value operations; j, model size; k, model performance: Model accuracy, model bias, model variance; l, model functionality; m, use case. Note: The coordination with RAN1 is needed.

RAN2 agreed to have meta information but it is not clear when, why and how this information would be provided. It is apparent that meta information listed may or may not be relevant for all LCM operations and even the proprietary and open format models will have different meta information.  We attempt to list the purposes of different meta information as shown in Table 2-4-1. The list can be exhaustive, and this could make the study even difficult to find all kind of meta information.

Table 2-4-1: Different meta information and association with different purposes.

| Meta information | Purpose |
| --- | --- |
| model version info; model format info; required AI capability; vendor info; | One sided and two-sided physical models to identify more information about a particular model and track for example if it is upgraded. |
| model input info | Only related to the standardized types of physical layer signaling required by the model. |
| model input info model output info; required AI capability; applicable scenario, configuration, site information | Can be part of logical model and used to identify if it will run in a compatible manner to a given network deployment (e.g., in a given cell). |
| model format info; computational complexity: FLOPs, level of pre-/post-processing; model complexity: number of real-value model parameter, number of real-value operations; model size; model performance: Model accuracy, model bias, model variance; | May be needed for open format model which are not clearly discussed. |

Observation 4: The list can be exhaustive, and this could make the study even difficult to find all kind of meta information.

A simple alternative is to split the meta information into two parts:

a) an indicative part where information for model structure and architecture (if not proprietary), version information, dataset information

b) a secured part where vendor specific information which can be encrypted or open

Proposal 9: RAN2 to study meta information further and identify static and dynamic aspects as well as discuss the need for security for such meta information.

# 3	Conclusion

In this contribution we discussed architectural aspects related to the functionality and model identification. Further we emphasized on functionality-based life cycle management (LCM) for ML-enabled features and functionality support indication as well as associated information.

Based on the discussion, the following observations are made:

Observation 1: Both the one-sided and two-sided model ID based LCM frameworks are significantly more complex than the functionality-based LCM.

Observation 2: The functionality inference and functionality data collection when located in the same node NW/gNB or UE node, can communicate with each other using implementation specific solutions i.e., do not require any RAN2 specifications.

Observation 3: The definition of additional conditions on logical AIML models depends on RAN1 progress, but might include, e.g., applicable scenario.

Observation 4: The list can be exhaustive, and this could make the study even difficult to find all kind of meta information.

Based on the discussion, the following proposals are made:

Proposal 1: RAN2 to consider Figure 1 as a baseline for one-sided functionality-based LCM framework.

Proposal 2: RAN2 to consider Figure 2 as a baseline for two-sided functionality-based LCM framework.

Proposal 3: RAN2 to study the protocol specifications required to enable the communication between the functionality management (gNB-side) and the UE-side functionality to ensure inter-operability, at least for registration, configuration, monitoring and selection, (de)activation, switching of Functionality (at) UE-side.

Proposal 4: Functionalities, defined and identified by sets of conditions, will be signalled through the RRC capability exchange procedure for the beam management, CSI compression, and CSI prediction use cases.

Proposal 5: Functionalities, defined and identified by sets of conditions, will be signalled through the LPP capability exchange procedure for the positioning use-case.

Proposal 6: Use the table to facilitate the configuration and procedures needed for functionality to enable each use case.

Proposal 7: Use Figure 4 as a starting point for discussing the definition of a logical model.

Proposal 8: For each of the Functionalities developed in the context of air interface, the Model identification (a process/method of identifying an (logical) ML Model for the common understanding between the NW and the UE), for UE-side models and UE-part of two-sided models, contains the following information elements,

- Logical Model ID(s): a labels/tags/UUIDs which can uniquely identify the logical ML Model(s) (each being set of physical ML models) as defined by RAN1 in [4] either within the ML-enabled Feature (across several potential Functionalities) or only within a specified Functionality, or Feature/FG, and

The use case specific additional conditions (e.g., dataset ID or context information) as determined/identified between UE-side and NW-side

Proposal 9: RAN2 to study meta information further and identify static and dynamic aspects as well as discuss the need for security for such meta information.

# 4	References

[1] “Final Report of 3GPP TSG RAN WG1 #111 v1.0.0, Toulouse, France, 14th – 18th November 2022

[2] “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #112, February 27th – March 3rd, 2023

[3] “Final Report of 3GPP TSG RAN WG1 #110bis-e v1.0.0”, Online, 10th-19th October 2022

[4] R1-2304052, Summary#2 of General Aspects of AI/ML Framework, Moderator (Qualcomm), TSG-RAN WG1 Meeting #112-bis-e, Online, April 17th – April 26th, 2023

[5] R2-2302649, “AI/ML Architecture”, Nokia, Nokia Shanghai Bell.

# 5	Appendix

## 5.1 Working list of terminologies (defined by RAN1)

Table 5-1: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non-real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |