3GPP TSG-RAN WG2 #122 	R2-2305149

Incheon, Korea, 22th –26th May 2023

Agenda Item:	7.5.4.1 BSR enhancements for XR

Source:	NEC

Title:	New BS table(s) and BSR trigger(s)

Document for:	Discussion, Decision

# Introduction

RAN2#121bis-e meeting made good progress on new BSR table(s) topic:

- As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be. - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made. - At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses. - As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size. - Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor). - Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

In this document, firstly we discuss our preference between pre-defined BSR table and RRC configured BSR table. Then we discuss the further details of the new BS table(s) including configuration and MAC CE format. Finally we discuss the potential to introduce new BSR trigger(s).

# Discussion

## Pre-defined vs RRC configured BS table(s).

Both pre-defined and RRC configured new BS table should work. The main argument of supporting pre-defined BS table are for simpler UE implementation and being most straightforward. On the other hand, RRC configured new BS table does not request complex UE implementation neither, because we agreed to use linear distribution. Another concern is how many times the UE needs to regenerate the new table. In principle gNB could reconfigure the new table and consequently UE needs to regenerate the new table, but in practice, gNB will not reconfigure the new table often or not at all during a RRC connection. Because it is not expected that the data rate will be changed dramatically from one range to another range far away. if data rate will changes a lot during RRC connection and request to change the used table often, even pre-configured BS table won’t work well too.

On the other hand, RRC configured new BS table is more future proof,  flexible and potentially achieve lower quantization error.  Besides, by leaving to RRC configuration, we do not need to discuss at this stage how many pre-defined tables should be introduced and which buffer size range each new table should cover. So our preference is RRC configured BS table:

New BS table is generated based on gNB’s configuration

## new BSR table(s) configuration

It is rare that a UE will run two different XR services in uplink and both services have high data rate and request new and different BSR table. Nevertheless, if this is the case, NW can map the traffic to different LCG, and then configure one additional BS table for each LCG respectively (assuming one LCG includes DRB of XR service A, and another LCG includes DRB of XR service B). it would be enough that network configure at most one new BS table a LCH is eligible to use, this will then reduce UE implementation of choosing only between one legacy BS table and one new BS tables. This also will simplify the MAC CE format.

In addition to legacy BS table(s), Network configure at most one additional BS table a LCG is eligible to use. Different BS table can be configured for different LCG

If RRC configured BS table is agreed, network only need to configure the minimum buffer size the BS table covers, and the step size. The maximum buffer size covered by the BS table will be determined by the number of codepoints:

- Network configure Bmin and step size X for each new BS table. (Bmax is equal to Bmin+ X*number of codepoints)

## BSR MAC CE format

Legacy BSR is reported per LCG. LCH with similar priority might be configured into same LCG. we do not expect the circumstance is changed with XR service.

On the other hand, potentially per PDU set/LCH remaining delay report might be supported. If the remaining delay will be reported by extending BSR MAC CE, per LCH/PDU set BSR report might be required.

At least per LCG BS report could be taken as baseline in our understanding:

Per LCG BS report is taken as baseline. Higher granularity than per LCG BS report can be discussed together with remaining delay report.

From different LCG perspective, it is possible that new BS table is required for some LCG(s)( i.e., the LCG include the LCHs carrying XR service) but not for other LCGs(e.g., the LCG for SRBs and DRX not carrying XR traffic). In a long or truncated BSR MAC CE, BS fields of some LCGs should be encode/decode based on legacy BS table if it is not configured with new BS table. It seem not necessary to indicate which table is used for these LCGs never using new BS table, since NW and UE both know whether new BS table is not enabled.

- Observation 1: it should be possible not to apply new BS table for SRB(s) and DRB not for XR traffic

From a given LCG perspective, if It is configured with the new BS table, it is still possible that the buffer size corresponding to this LCG at some point of time, is small, e.g., when only PDCP/RLC control PDU is in buffer or when most of buffered data has been scheduled away. When the buffered data is small, precise BS information is very important for gNB in order to schedule a right size of UL grant. Falling back to use legacy BS table would be preferable. Moreover, if it is agreeable the new BSR table only covers a certain BS range, the buffer size may fall out of the new BS table coverage.

- Observation 2: UE should be able to fall back to legacy BS table(s) for a LCG which is configured/enabled with new BS table (e.g., in case the buffer size becomes small after being scheduled)

hence it is not enough for UE always use new or legacy BS table based on NW RRC configuration, it would be better:

- When the buffer size is in (min, max), use new BS table which provides more precise buffer size

- When the buffer size is out of (min, max), use legacy BS table(s)

To support this dynamic BS table switch, table indication should be introduced in BSR MAC CE, in principle, BS table indication would be only needed if the LCG is configured with additional new BSR table, but it is simple just to introduce table indication for every LCG. Following are example of BSR MAC CR with BS table indication:

- Figure 1: example of BS table indications in long or long truncated BSR MAC CE

Moreover there might be some uncertain period between the reconfiguration message and the reflection of the received new configuration. If this uncertain period may exist for this new BSR mechanism, the BS table indication will be useful. Hence, It is proposed:

BS table indication per LCG is introduced in BSR MAC CE, to enable UE to switch BS table depending on the buffer size

As we discussed here above, it is likely that a long or long truncated BSR MAC CE includes some BS fields encoded based on a legacy 8bits BS table, and some other BS field encoded based on a new BS table, it would be easier to keep all the BS fields with same length, including the BS fields using new BS table.

For a long or long truncated BSR MAC CE, BS field has fixed length of 8 bits

For short or short truncated BSR MAC CE, currently, 5bit BS field/table is used. However, there was proposal to also use 8bits field for finer granularity. Apparently, new BS table is also for finer granularity, this should be discussed together. we could then have 4 options to increase the granularity:

- keep the 5bits BS field but use a new 5bits BS table for higher granularity

- introduce 8bits BS field and use only the legacy 8bit BS table

- introduce 8bits BS field and use only the new 8bit BS table to align with the BS table used for long or long truncated BSR MAC CE

- introduce 8bit BS filed, and UE choose either to use legacy 8bits BS table or new 8bits BS table depends on if the Buffer Size fall in (min, max)

Option 2 and option 3 would request 2bytes of MAC CE format to accommodate 8bits Buffer Size field and 3bits LCG ID field, still 5bits will not be used yet. Instead, with option 4, 1 of the unused 5bits can be used for BS table indication. For option1, it can keep 1byte MAC CE format, but a new (e)LCID would be still required to distinguish from legacy short/short truncated MAC CE. Our preference is to have 8bits field to having same granularity for both short/short truncated BSR MAC CE and long/long truncated BSR MAC CE, with the cost of 1byte more payload.

- Figure 2: example of BS table indication in short or short truncated BSR MAC CE

if new BS table is configured/enabled for a LCG, for a short or short truncated BSR MAC CE, BS field has fixed length of 8 bits too, either legacy or new 8bit BS table could be used.

## New BSR trigger(s)

One target of introducing a new BSR trigger is to update gNB the buffer size especially when the gNB-expected buffer size (based on the last BSR report and scheduled data volume since then) is different from the real buffer size in UE. This difference can be due to more data arrival in the buffer, PDU discarding and so on. Another target of introducing new BSR trigger is simply to update gNB more often in case of XR service where the buffer size and UL grant tend to be big. Following these two targets, RAN2 could discuss a BSR trigger based on unexpected buffer size change for gNB, or we can simply always include BSR if the signalling overhead is not a problem e.g. when the UL grant is big.

New BSR triggers have been proposed by many companies, but unfortunately, these proposals are very different from each other. In the following table, we summarized all proposals from other companies and potential new triggers discussed above, and share our views on each new BSR trigger.

| new BSR triggers | Categories | Our view |
| --- | --- | --- |
| - BSR is triggered if there is remaining data for the current PDU set being transmitted[4] | PDU set-aware BSR trigger | - This will introduce cross-layer interaction, because PDU set would be visible in upper layer but not MAC layer. - For the new/more PDU/burst arrival cases, it can be generalized as unknown buffer size changes for gNB due to new data arrial |
| - BSR is triggered when all data of a whole burst or PDU set arrives [7] |  |  |
| - BSR is triggered when a new data burst starts arriving[5] |  |  |
| - New trigger based on the reception of first PDU of PDU set [11][12] |  |  |
| - BSR is triggered when subsequent data arrival[7] |  |  |
| - BSR is triggered when packet approaching the PDB[10] [12] [13] | Remaining delay budget-based BSR trigger | This can be discussed separately together with remaining delay report. |
| - PDU discard should trigger a regular BSR[8][9] | PDU discard-based BSR trigger | This can be generalized as unknown buffer size changes for gNB due to PDU discard |
| - BSR MAC CE is always included for each TB transmitted with XR packet[6] | XR-service specific BSR trigger | It is not recommended to make BSR mechanism service specific, Because the definition and characteristics of XR service will vary a lot. A function tied to service type is not future proof The underline reason for this trigger probably is potentially big UL grant for XR service. Then we prefer to trigger it based on UL grant size instead of a particular service. |
| - Starting on-duration should trigger a periodic BSR[8] | DRX-based BSR trigger | We do not normally tie DRX(mainly for DL traffic) with BSR(mainly for UL traffic) |
| - Unknown/unexpected buffer size changes for gNB |  | Our preference: because it is decoupled from specific service or PDU set/burst, but focus on the real need of BS update. Please note that new data arrival does not trigger BSR all the time. |

Based on above analysis:

Introduce new BSR trigger based on unknown/unexpected buffer size change(not due to being scheduled, but due to discard or additional data arrival)

Introduce new BSR trigger based on UL grant size is bigger than a threshold

# Summary

Regarding detail of new BS table(s) and possible new BSR trigger(s), we have proposals as follows:

Comparing pre-defined and RRC configured BS table(s), RRC configured BS table is our preference:

- New BS table is generated based on gNB’s configuration

For BS configuration:

In addition to legacy BS table(s), Network configure at most one additional BS table a LCG is eligible to use. Different BS table can be configured for different LCG

Network configure Bmin and step size X for each new BS table. (Bmax is equal to Bmin+ X*number of codepoints)

For BSR MAC CE:

Per LCG BS report is taken as baseline. Higher granularity than per LCG BS report can be discussed together with remaining delay report.

- Observation 1: it should be possible not to apply new BS table for SRB(s) and DRB not for XR traffic

- Observation 2: UE should be able to fall back to legacy BS table(s) for a LCG which is configured/enabled with new BS table (e.g., in case the buffer size becomes small after being scheduled)

BS table indication per LCG is introduced in BSR MAC CE, to enable UE to switch BS table depending on the buffer size

For a long or long truncated BSR MAC CE, BS field has fixed length of 8 bits.

if new BS table is configured/enabled for a LCG, for a short or short truncated BSR MAC CE, BS field has fixed length of 8 bits too, either legacy or new 8bit BS table could be used.

Regarding new BSR trigger(s):

Introduce new BSR trigger based on unknown/unexpected buffer size change(not due to being scheduled, but due to discard or additional data arrival)

Introduce new BSR trigger based on UL grant size is bigger than a threshold

# References

- R2-2209472	BSR enhancement for XR capacity	CATT	discussion	Rel-18	FS_NR_XR_enh

- R2-2210537	Discussion on BSR enhancement for XR-specific capacity improvement	Huawei, HiSilicon	discussion	Rel-18	FS_NR_XR_enh

- R2-2209591	BSR enhancement for XR capacity	MediaTek Inc.	discussion	Rel-18

- R2-2209490	Discussion on feedback enhancements for XR-specific capacity improvements	vivo	discussion	Rel-18	FS_NR_XR_enh

- R2-2209558	BSR for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_NR_XR_enh

- R2-2209636	Enhancements to Buffer Status Reporting for XR Traffic	Intel Corporation	discussion	Rel-18	FS_NR_XR_enh

- R2-2209672	Discussing on UE feedback enhancements for XR capacity	Xiaomi Communications	discussion

- R2-2209691	Discussion on XR-specific feedback enhancements 	InterDigital, Inc.	discussion	FS_NR_XR_enh

- R2-2210024	Discussion on feedback enhancement	OPPO	discussion	Rel-18	FS_NR_XR_enh

- R2-2210599	Discussion on BSR enahancement for timing information in XR	LG Electronics Inc.	discussion	Rel-18	FS_NR_XR_enh