3GPP TSG RAN WG2 #122	                                    		R2-2305167

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.4.2.3

Source:	InterDigital, Inc.

Title:	LTM cell switch MAC CE and triggering.

Document for:	Discussion

# Introduction

The WI on further NR mobility enhancements [1] includes the following objectives for L1/2 based inter-cell mobility:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In the previous meetings, the following agreements were made:

- RAN2#119bis-e - RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. - RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. - FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). - RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. - RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). - FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. - R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed. RAN2#120 - The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch. - LTM cell switch is supervised by a timer - UE arrival in the target cell need to be indicated (somehow) - RAN2 to have the mindset to have a common design for partial MAC reset for different cell change cases in intra-DU scenario (as far as reasonable) - Permanent Identities such as PCI will not be used in L1 L2 signalling, instead L1 L2 signalling will use temporary identities configured by RRC. RAN2#121 - No consensus to support HARQ continuation (and in order to resume discussion some new input may be needed, e.g. quantitative evidence of a serious problem). - To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

The main aim of this objective is to introduce L1/2 triggered cell change, for mobility latency reduction. In this contribution we provide our view on the trigger mechanism.

# Discussion

In general, the contents of the cell switch MAC CE should be limited as much as possible assuming that most of the configuration and procedures have already been provided or performed, and the UE has the necessary information to determine what to perform as part of cell switch.

In addition to the candidate configuration agreed by RAN2, and the SCell states proposed above, RAN1 have agreed the following in the previous meeting:

Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI FFS: the presence of each field (i.e. always present or configurable)

Proposal 1: The content of the cell switch MAC CE at least consists of:

- Candidate configuration ID (already agreed)

- TA related information (agreed by RAN1)

- 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell (agreed by RAN1)

- Active DL and UL BWPs for the target cell (agreed by RAN1)

- FFS RAN1: Triggering of aperiodic TRS transmitted from the target cell

- FFS RAN1: Triggering the CSI acquisition of the target cell and reporting to the target cell

- FFS RAN1: Triggering of aperiodic SRS transmission to the target cell

- FFS RAN1: C-RNTI

FFS: the presence of each field (i.e. always present or configurable)

In addition, we already perform SCell activation/deactivation using MAC. We assume that it needs to be possible to activate/deactivate SCells at the same time as performing SpCell change. Otherwise, we would need to send a first MAC CE in order to activate the candidate configuration, then send a second one to activate the configured SCells. Performing this in a single step would significantly improve latency in terms of being able to perform handover directly to a suitable CA configuration.

It was argued in a previous meeting that this may not be possible for the case of inter-DU cell switch because the source may not know the situation at the target DU. While it may be possible to solve this, a more straightforward approach for inter-DU would be to re-use the RRC configuration sCellState. However, for the intra-DU case there is no reason why SCell activation cannot be performed simultaneously with SpCell switching, therefore we think that this should be supported for at least the intra-DU scenario. If it is supported for the intra-DU scenario, there is perhaps no need to write any limit in the specification – the network can utilise simultaneous SpCell change with SCell activation/deactivation in the relevant scenarios.

One possibility would be to re-use the existing SCell activation/deactivation. However, the current MAC CE refers to current serving cells. Some additional indication such as using the spare bit may be needed in order to indicate that the existing MAC CE is to be applied to the not yet configured serving cells, or NW needs to ensure that the MAC CEs are received in the same TTI and it needs to be specified that the MAC CEs received in the same TTI need to be applied in a specific sequence.

Another possibility would be to define a new MAC CE. Similar to the existing SCell activation/deactivation, the MAC CE could contain a bitmap referring to the indexes of SCells associated with the candidate configuration, for example where every possible SpCell has a candidate configuration then a candidate index can be signalled, with an SCell activation/deactivation bitmap, similar to the following:

Proposal 2: By default the initial SCell state on the target cell after LTM cell switch is based on RRC configuration sCellState. At least for the intra-DU case, NW may indicate a new SpCell candidate configuration index and perform SCell activation/deactivation simultaneously using MAC CE.

Some further discussion is also needed on whether BWP is always indicated in the cell switch command. RAN1 agreed that it may be, but we assume that this is optional and the BWP to use upon cell switch may be pre-configured in at least some (if not most) cases.

Proposal 3: BWP can be indicated in the RRC configuration of candidate cells, MAC CE indication in cell switch command is optional.

Similarly, some further discussion is needed regarding whether the MAC CE may contain a grant or a dedicated RACH resource. In the previous meeting several companies have proposed that this should be possible.

Proposal 4: RAN2 to discuss whether the cell switch MAC CE may contain an indication of RACH resource and/or UL grant for the target cell.

Depending on the overall solution, some further information may need to be conveyed in a separate MAC CE. For example, there may be procedures to be performed before the final cell switch command MAC CE can be issued. This needs further discussion in both RAN1 and RAN2, for example depending on when and how L1 measurement reporting is activated, when DL and/or UL sync is performed, and whether and when RLM and BFD are enabled on target cells.

For example, a MAC CE may be issued in order to trigger the L1 measurements, RLM/BFD, and DL/UL sync of a candidate cell that was detected and reported by the UE, then another MAC CE may be issued (after suitable L1 report) to perform the cell switch itself, and this subsequent MAC CE may include the target configuration ID, beam indication and TA indication.

In particular, RAN1 discussed the L1 measurement configurations for SSB and have asked RAN2 to design the signalling (see [3]). The most efficient option would in our opinion be to define the configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells. This, as noted by RAN1 [3], is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]. Even in this case, the amount of L1 measurement configuration will be significant and it is desirable for the NW to be able to control which ones are in use/active at any point of time, therefore some control is necessary to limit both the UE measurements and the UL resource usage for CSI reporting.

Furthermore, several companies have proposed that it should be possible to perform RLF recovery and BFR on a target cell. It should be further discussed whether it is possible to configure RLM and BFD on a target cell ahead of cell switch, for example for cells which the UE is maintaining DL sync.

Proposal 5: RAN2 to discuss whether, in addition to the cell switch MAC CE, a separate MAC CE is needed for triggering the following functions on target cell(s):

- Enabling L1/CSI measurements for reporting to the serving cell (e.g. configuration IDs of RRC configured L1 report configs, and L1 measurement RSs)

- Triggering DL synch (e.g. candidate configuration ID to trigger SpCell sync)

- Enabling RLM of target candidate cells

- Enabling BFD of target candidate cells

# Conclusion

In this paper we provide the following proposals regarding MAC CE contents and behaviour to consider in the design of L1/2 triggered handover in Release-18.

Proposal 1: The content of the cell switch MAC CE at least consists of:

- Candidate configuration ID (already agreed)

- TA related information (agreed by RAN1)

- 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell (agreed by RAN1)

- Active DL and UL BWPs for the target cell (agreed by RAN1)

- FFS RAN1: Triggering of aperiodic TRS transmitted from the target cell

- FFS RAN1: Triggering the CSI acquisition of the target cell and reporting to the target cell

- FFS RAN1: Triggering of aperiodic SRS transmission to the target cell

- FFS RAN1: C-RNTI

FFS: the presence of each field (i.e. always present or configurable)

Proposal 2: By default the initial SCell state on the target cell after LTM cell switch is based on RRC configuration sCellState. At least for the intra-DU case, NW may indicate a new SpCell candidate configuration index and perform SCell activation/deactivation simultaneously using MAC CE.

Proposal 3: BWP can be indicated in the RRC configuration of candidate cells, MAC CE indication in cell switch command is optional.

Proposal 4: RAN2 to discuss whether the cell switch MAC CE may contain an indication of RACH resource and/or UL grant for the target cell.

Proposal 5: RAN2 to discuss whether, in addition to the cell switch MAC CE, a separate MAC CE is needed for triggering the following functions on target cell(s):

- Enabling L1/CSI measurements for reporting to the serving cell (e.g. configuration IDs of RRC configured L1 report configs, and L1 measurement RSs)

- Triggering DL synch (e.g. candidate configuration ID to trigger SpCell sync)

- Enabling RLM of target candidate cells

- Enabling BFD of target candidate cells

# References

- RP-221799,  “Revised WID on Further NR mobility enhancements”

- 38.133, “NR; Requirements for support of radio resource management”