3GPP TSG-RAN2 Meeting #122	R2-2305184

# Incheon, South Korea, May 22-26, 2023

Agenda Item:	7.11.2.1

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Service continuity, RRC state transitions and notifications

Document for:	Discussion/Decision

# Introduction

Rel-17 MBS added support of multicast services reception by UEs in CM_CONNECTED/ RRC_CONNECTED state only. One of the objectives of Rel-18 WI on enhancements of NR MBS is as follows (highlighting added), see RP-221458:

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include: - Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - <<skip>> Note: collaboration with SA2 is expected in due course for the above objectives.

Previous RAN2 meetings made several agreements related to service continuity, state transitions and notifications. In this contribution, we provide further views on the outstanding issues for multicast reception in RRC_INACTIVE.

# RRC State Transitions

## Transition from RRC_CONNECTED to RRC_INACTIVE

### PTM configuration via RRC Release Msg vs via MCCH

RAN2#121bis-e agreed the following:

Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

The above agreements apply for the case of cell reselection. RAN2#121bis-e also agreed the following:

UE in RRC CONNECTED state is not required to read multicast MCCH to be able to receive multicast MBS service i.e. the UE receives the PTM configuration via dedicated signalling. <<skip>>

RAN2#119bis-e had previously agreed:

Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS).

Consider the case where a UE in RRC_CONNECTED receives PTM configuration in a RRC Release message. At the same time network is also providing the service for UEs in RRC_INACTIVE state and multicast SIB and MCCH is already being broadcasted. RAN2 previously agreed that same or different PHY resources can be used to deliver same service in RRC_CONNECTED vs RRC_INACTIVE in a cell.

Now there are several questions to be addressed. E.g. how does the UE know whether the same or different configurations are provided in multicast SIB/MCCH vs the dedicated configuration in the release message? If they are different, when does the MCCH-provided configuration takes precedence over the dedicatedly provided configuration? It has been agreed that if the UE changes the cell, UE has to read MCCH in the new cell. However, if the UE does not change the cell, the UE is not required to read MCCH in RRC_CONNECTED but it is not clear when to read the MCCH after the UE transmits from RRC_CONNECTED to RRC_INACTIVE. Therefore, RAN2 should discuss when the dedicatedly provided configuration ceases to be valid while the UE is in RRC_INACTIVE state and when the UE is required to read the MCCH to get the PTM configuration if different. Without any indication, the UE may keep using the dedicated PTM configuration even if it is not valid anymore and miss the multicast reception which is not preferred.

If the UE does not change cell but transits from RRC_CONNECTED to RRC_INACTIVE, it is unclear when the dedicatedly provided configuration ceases to be valid and when the UE is required to start using MCCH-provided PTM configuration.

It is beneficial if the UE knows whether the current serving cell is broadcasting MCCH or not. And if MCCH is present, it is beneficial to know whether the PTM config is same between the Release msg and MCCH when the UE moves to RRC_INACTIVE.

Discuss whether and how to indicate presence of MCCH in the cell to the RRC_CONNECTED UEs and whether MCCH-provided PTM configuration is same or different from dedicatedly provided configuration.

When the UE transits from RRC_CONNECTED to RRC_INACTIVE, discuss when the PTM configuration provided in RRC Release msg stops being applicable, when the UE is required to start using the MCCH-provided PTM config, and how the UE knows about it.

### Assistance info for gNB to move UE to RRC_INACTIVE

RAN2 previously agreed the following:

In RAN2#119e

It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision (related to SA2 discussion).

It is assumed the network can choose which UEs receive in RRC INACTIVE and which in RRC Connected and can move UEs between the states for Multicast service reception.

Regarding the “FFS what information gNB may be provided to form such decision (related to SA2 discussion)”, there are two aspects of this:

- Information provided from 5GC to gNB

- Information provided from UE to gNB

Regarding information provided from 5GC to gNB, SA2 has made some conclusions and we understand there is ongoing discussion in RAN3 on what information needs to be passed between CN and RAN.

SA2 sent LS to RAN2 in R2-2301934 (S2-2303407), which indicated the following:

- SA2 agrees that the MBS assistance information for the MBS session sent to NG-RAN consists of an indication that the UE is preferred to be kept in connected when receiving the related MBS session data.

In general, CN gathers such information on whether a particular UE is preferred to be kept in CONNECTED (aka “special UEs”) from the application function and as SA2 indicated, this will be provided to NG-RAN node by the CN. So, RAN2#121bis-e agreed the following:

A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

The FFS on how network can resume such UE is discussed later in section 2.2.1.

Regarding information provided from UE to gNB, in previous RAN2 meetings some companies proposed UE to provide its preference whether to receive a multicast service in RRC_CONNECTED vs RRC_INACTIVE state. In our view, decision on whether a UE should receive multicast service in RRC_INACTIVE state is totally up to RAN decision (while taking into account the recommendation about special UE from the CN if applicable) as long as the UE is capable of receiving multicast in RRC_INACTIVE.

UE indicates capability on whether it supports MBS multicast reception in RRC_INACTIVE. Other than the UE capability, UE does not provide additional indication to RAN about its preference to receive multicast in RRC_INACTIVE state (i.e., it is up to network decision).

## Transition from RRC_INACTIVE to RRC_CONNECTED

### Multicast session activation while UE is in RRC_INACTIVE

Based on the discussions during previous RAN2 meetings, there are at least the following scenarios which the group paging should be able to unambiguously indicate to the multicast UE in RRC_INACTIVE (which has already joined the multicast session and has valid PTM configurations):

- The multicast session was not activated and now it is activated:

  - All applicable UEs continue to receive the service in RRC_INACTIVE, or

  - All applicable UEs move to RRC_CONNECTED to receive the service, or

  - Specific UEs move to RRC_CONNECTED to receive the service (in accordance with the agreement in RAN2#119e)

- The multicast session was already activated but was in temporary data inactivity, now the inactivity is ended:

  - All applicable UEs continue to receive the service in RRC_INACTIVE, or

  - All applicable UEs move to RRC_CONNECTED to receive the service, or

  - Specific UEs move to RRC_CONNECTED to receive the service (in accordance with the agreement in RAN2#119e)

RAN2#121bis-e agreed the following:

Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

Legacy group paging (i.e., Rel-17 group paging) can be used to resume UE to RRC_CONNECTED state.

UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior).

When both enhanced group paging and unicast paging are received by the UE (and targeted for this UE), the UE follows unicast Paging and goes to RRC CONNECTED.

The legacy group paging (Rel-17) behavior is that once a group paging is received, all the UEs receiving a certain session/TMGI to move back to RRC_CONNECTED. Therefore, RAN2 has agreed that group paging needs enhancements to enable (all or some) Rel-18 UEs to stay in RRC_INACTIVE and start monitoring corresponding G-RNTI, but details are FFS. We think a single flag per TMGI should be enough in group paging to indicate the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service (absence of such Rel-18 indication would mean all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED).

A single flag per TMGI should be enough in group paging to indicate the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service (absence of such indication would mean legacy Rel-17 behavior i.e. all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED).

Therefore, we propose the following:

Add a per-TMGI flag in Rel-18 group paging to indicate the UEs (that have valid PTM configuration) continue to receive the service associated with the TMGI in RRC_INACTIVE.

Now, going back to the FFS regarding moving of ‘special UE’ back to RRC_CONNECTED mode: the above agreements and proposals apply and cover all the UEs (including the special UEs), i.e. special UEs can also be moved back to RRC_CONNECTED mode using legacy UE-specific paging. So, no additional special handling would be needed.

No additional special handling is needed regarding how to move ‘special UEs’ back to RRC_CONNECTED.

No additional enhancements are needed specifically for moving ‘special UEs’ back to RRC_CONNECTED.

### Multicast session deactivation/temporary no data while UE is in RRC_INACTIVE

In Rel-17, upon receiving multicast deactivation indication from 5GC, RAN will release all radio resources and inform UEs by releasing MRB configuration via dedicated RRC signaling procedure.

In case of Rel-18, for RRC_INACTIVE UEs, it should be possible for gNB to indicate the multicast session deactivation to multicast RRC_INACTIVE capable UEs by using either Group Paging method or by using MCCH based method.

Both group paging and MCCH-based options are possible to indicate deactivation of multicast session while UE is in RRC_INACTIVE.

RAN2#121bis-e captured the following agreement:

FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

RAN2 previously agreed that MCCH is used to indicate PTM configuration change while the UE is in RRC_INACTIVE. Similarly, deactivation/temporary no data of the session can be indicated by MCCH.

For UEs receiving multicast in RRC_INACTIVE, deactivation/temporary no data of the multicast session is indicated by multicast MCCH.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	If the UE does not change cell but transits from RRC_CONNECTED to RRC_INACTIVE, it is unclear when the dedicatedly provided configuration ceases to be valid and when the UE is required to start using MCCH-provided PTM configuration.

Observation 2.	It is beneficial if the UE knows whether the current serving cell is broadcasting MCCH or not. And if MCCH is present, it is beneficial to know whether the PTM config is same between the Release msg and MCCH when the UE moves to RRC_INACTIVE.

Observation 3.	A single flag per TMGI should be enough in group paging to indicate the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service (absence of such indication would mean legacy Rel-17 behavior i.e. all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED).

Observation 4.	No additional special handling is needed regarding how to move ‘special UEs’ back to RRC_CONNECTED.

Observation 5.	Both group paging and MCCH-based options are possible to indicate deactivation of multicast session while UE is in RRC_INACTIVE.

Based on these observations and discussion above, we have the following proposals:

Proposal 1.	Discuss whether and how to indicate presence of MCCH in the cell to the RRC_CONNECTED UEs and whether MCCH-provided PTM configuration is same or different from dedicatedly provided configuration.

Proposal 2.	When the UE transits from RRC_CONNECTED to RRC_INACTIVE, discuss when the PTM configuration provided in RRC Release msg stops being applicable, when the UE is required to start using the MCCH-provided PTM config, and how the UE knows about it.

Proposal 3.	UE indicates capability on whether it supports MBS multicast reception in RRC_INACTIVE. Other than the UE capability, UE does not provide additional indication to RAN about its preference to receive multicast in RRC_INACTIVE state (i.e., it is up to network decision).

Proposal 4.	Add a per-TMGI flag in Rel-18 group paging to indicate the UEs (that have valid PTM configuration) continue to receive the service associated with the TMGI in RRC_INACTIVE.

Proposal 5.	No additional enhancements are needed specifically for moving ‘special UEs’ back to RRC_CONNECTED.

Proposal 6.	For UEs receiving multicast in RRC_INACTIVE, deactivation/temporary no data of the multicast session is indicated by multicast MCCH.