3GPP TSG-RAN WG2 Meeting #122	R2-2305189

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.5.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	XR SA2 Status

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution gives a short overview of the status of the XR work in SA2.

# 2	Status

Since the last RAN2 meeting, SA2 has agreed several Rel-18 CRs against 23.501 for XR [report]:

-	S2-2306184 on Update of Policy control enhancements to support multi-modal services

-	S2-2306187 on Support of extra traffic characteristics for alternative QoS profile → relevant to RAN specifications (CR allows the SMF to provide MDBV for each alternative QoS profile and RAN to check these extra traffic characteristics parameters).

-	S2-2306188 on Update on Data Rate Monitoring → still relevant to RAN (there is an EN stating that it is pending for RAN WGs to confirm whether providing measured data rate information for QoS Flow to the CN can be included or not in Release-18)

-	S2-2306189 on Resolution of EN for L4S

-	S2-2306241 on Policy Control for L4S

-	S2-2306191 on Update for network exposure for XR services → still relevant to RAN (the CR clarifies how the SMF request RAN to report congestion information)

-	S2-2306194 on Resolve ENs for support of PDU Set handling → relevant to RAN:

-	The notion of QoS flow supporting PDU Set based QoS handling is removed, with PDU Set QoS Parameters now listed as optional for regular QoS flows.

-	All PDU Set QoS parameters are marked as optional.

-	An EN stating that How RAN enforces PSER is up to RAN implementation is added

NOTE: 	The corresponding changes to the Stage 2 CR are incorporated in R2-2305189

-	S2-2306197 on KI#6 text alignment text

-	S2-2306199 on Update about the Packet Delay Variation description and add PDV in QoS monitoring parameters

-	S2-2306201 on EN resolving for KI#8 Except TSCAC and clarifications on AF inputs

-	S2-2306203 on PCF provides the Periodicity to SMF via PCC rules for resolving the EN for KI#8

In addition, for the handling of PDUs not belonging to PDU sets, SA2 has endorsed S2-2306239 which describes that if the UPF receives a PDU that do not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps it to a PDU Set (e.g. it could be a PDU Set with just that PDU) with an EN stating that How PDU Set Importance is applied to the PDU(s) that does not belong to a PDU Set based on Protocol Description is FFS. This is expected to be resolved at the next SA2 meeting.

In addition, SA2 has agreed several Rel-18 CRs against 23.502 for XR [report]:

-	S2-2306186 on Procedures update to support policy control enhancements for multi-modal flows

-	S2-2306192 on Update TS23.502 to reflect conclusion of KI#3 for XRM in TR23.700-60 for the API based congestion exposure option

-	S2-2306195 on PDU Set information provisioning

-	S2-2306198 on Procedures update to support QoS monitoring for the correlated multi-modal flows

-	S2-2306200 on AF Session procedure update to support 5GS Packet Delay Variation monitoring

-	S2-2306244 on Procedure to acquire jitter information

In addition, SA2 has agreed several Rel-18 CRs against 23.503 for XR [report]:

-	S2-2306241 on Policy Control for L4S

-	S2-2306193 on QoS monitoring enhancement

-	S2-2306196 on Support for PDU Set QoS Framework → Note that the SMF shall set the PDU Set Delay Budget, the PDU Set Error Rate and the PDU Set Integrated Handling Information for QoS Flow based on the corresponding parameter in the PDU Set Control Information if available in the PCC rule.

-	S2-2304168 on Policy control support for Packet Delay Variation monitoring and reporting

-	S2-2305262 on Correction on Jitter and Alignment with PDV → replaces jitter by PDV

-	S2-2306202 on Periodicity measurement and reporting for power saving → note that when the PCF does not receive the periodicity information from AF, the periodicity is then part of the PCC rule and defined as the time period between start of two data bursts in UL/DL direction.