3GPP TSG-RAN WG2 Meeting #122	R2-2305205

Incheon, Korea, 22nd-26th May 2023

Agenda item:	7.3.2

Source: 	Fujitsu

Title: 	Discussion on Cell DTX/DRX

Document for:	Discussion and decision

Introduction

In RAN2#121bis-e meeting, RAN2 agreed the followings on the Cell DTX/DRX [1].

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

In this contribution, we would like to discuss SR transmissions and retransmission case on the Cell DTX/DRX.

Discussion

SR transmissions

In RAN2#121bis-e meeting, RAN2 discussed whether an exceptional handling is necessary for SR transmissions during Cell DTX non-active period. However, there is no consensus during the meeting as follows:

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

For the network energy saving efficiency, it is useful that any SR occasions are skipped during Cell DTX non-active period. But it may jeopardize the UE QoS requirements especially a delay sensitive service like an emergency call. Some companies proposed SR RACH can be used to request resources during Cell DTX non-active period, however the SR RACH cannot be ensured to fulfil the requirements due to contention.

For SR transmission, there are some trade-offs between network energy saving gain and scheduling flexibility. Hence, it would be good to be configurable by the network. If the network wants to maximize the network energy saving gain, it can be simply set to “not allowed”. On the other hand, if a delay sensitive service like the emergency call can be accepted in the serving cell, immediate SR transmission should be allowed and indicated by SR configuration. As the SR is supposed to be configured per LCH, it is natural to indicate whether it is allowed per SR configuration.

Observation 1:	It is reasonable that the network allows an SR transmission during Cell DRX non-active period for a high priority traffic (e.g., emergency).

Proposal 1:	The network indicates per SR configuration whether an SR transmission is allowed during Cell DRX non-active period.

If the SR is transmitted during the Cell DRX non-active period, it is natural for the network to send a corresponding PDCCH for a new transmission immediately after the SR reception. Because the SR is triggered by the high priority traffic then it is urgent to allocate radio resources to archive the UE QoS requirement. Therefore, in this case, the network should not be delayed transmitting a scheduling grant until the next Cell DTX active period.

The UE behaviours are the same as the current C-DRX mechanism, i.e., the active period of the C-DRX is started after sending SR PUCCH and monitor the PDCCH for a new transmission during C-DRX active period even during Cell DTX non-active period.

Proposal 2:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH after the SR transmission regardless of Cell DTX configuration.

Proposal 3:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a new transmission during C-DRX active period even during Cell DTX non-active period.

Retransmission case

Due to the lack of time, a retransmission case is also not agreed in the last meeting.

FFS how to deal with retransmissions

As same as the new transmission case, a PDCCH for dynamic UL retransmission of high priority traffic should be monitored by the UE regardless of the Cell DTX non-active period. There is no technical reason to restrict the PDCCH monitoring for a dynamic UL retransmission during the Cell DTX active period. Otherwise, the UL data completion may be delayed then the QoS requirements of the traffic is not met.

The UE just follows the C-DRX mechanism for the retransmission case, as it is already considered a dynamic UL retransmission.

Observation 2:	To meet the QoS requirements for a high priority traffic, it is important to complete UL data transmissions regardless of Cell DTX active period.

Proposal 4:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH for a dynamic UL retransmission by C-DRX mechanism.

Proposal 5:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a retransmission during C-DRX active period even during Cell DTX non-active period.

In addition, if the SR with high priority is not transmitted, the UL traffic is considered as delay tolerance data then it can be delayed to the next Cell DTX active period. Therefore, the UE does not have to monitor the PDCCH during Cell DTX non-active period.

Proposal 6:	If the SR with high priority is not transmitted, the UE should monitor a PDCCH for a dynamic UL retransmission during Cell DTX active period.

Proposal 7:	If the SR with high priority is not transmitted, the network is limited to schedule a dynamic UL grant for a retransmission during Cell DTX active period.

Conclusion

In this contribution, we have some discussions on the Cell DTX/DRX issues, and the following proposals are made:

SR transmissions:

Proposal 1:	The network indicates per SR configuration whether an SR transmission is allowed during Cell DRX non-active period.

Proposal 2:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH after the SR transmission regardless of Cell DTX configuration.

Proposal 3:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a new transmission during C-DRX active period even during Cell DTX non-active period.

Retransmission case:

Proposal 4:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH for a dynamic UL retransmission by C-DRX mechanism.

Proposal 5:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a retransmission during C-DRX active period even during Cell DTX non-active period.

Proposal 6:	If the SR with high priority is not transmitted, the UE should monitor a PDCCH for a dynamic UL retransmission during Cell DTX active period.

Proposal 7:	If the SR with high priority is not transmitted, the network is limited to schedule a dynamic UL grant for a retransmission during Cell DTX active period.

References

- R2-2304203

- R2-2303604