3GPP TSG-RAN WG2 Meeting #122	 R2- 2305217

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	7.9.3

Source:	Xiaomi

Title:	Discussion on service continuity enhancement

Document for:	Discussion

Introduction

In this contribution, we discuss how to support the service continuity enhancements for L2 UE-to-network relay.

Discussion

Event Z2

Event Z1 is introduced in R18. But it’s FFS whether Z2 is introduced. According to RAN1/RAN4 reply LS, the comparison between SL-RSRP and SD-RSRP requires additional work to resolve the power control difference. Therefore, it’s difficult to compare the SL-RSRP and SD-RSRP.

Proposal 1: Not introduce comparison between SL-RSRP and SD-RSRP based event Z2.

However, there is no technical problem to compare SD-RSRP from serving relay and candidate relay. Serving relay UE may still perform discovery transmission to discover other remote UEs and remote UE may still monitor discovery message to evaluate the candidate relay UE. Therefore, there is no additional effort for remote UE to obtain the SD-RSRP of serving relay UE.

Observation 1: Remote UE can obtain the SD-RSRP of serving relay UE without additional effort.

Event Z1 is fulfilled when serving relay UE becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2. If there is candidate relay which is much better than serving relay UE, remote UE would not trigger report as long as serving relay UE is not worse than threshold1. NW can’t switch remote UE to the candidate relay UE. Event Z2 can trigger the report in such case, if candidate relay UE is offset better than serving relay UE. It is important for remote UE to find another relay UE which could provide better service than current serving relay UE.

Proposal 2: Introduce SD-RSRP based event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE.

Lossless delivery

In last meeting, U3 and U5 are agreed as candidate solutions to ensure uplink lossless delivery. In U3, Remote UE may have discarded the data confirmed by relay UE reception. After handover, remote UE can’t retransmit such data to target cell. Lossless can’t be ensured. To avoid such data loss, longer discard timer, e.g. infinity, is expected to be configured to UE. But such long discard timer would require large UE storage, which may not be supported by low capability UE. Therefore, U3 is not preferred. On the other hand, U5 doesn’t require special UE handling and can apply to all types of UE.

Proposal 3: U5 is selected to ensure uplink lossless delivery.

In last meeting, D3/4/5 are agreed as candidate solutions to ensure downlink lossless delivery. D3/D5 are implementation based. But D4 requires enhancement on Xn interface, which is not in the scope of RAN2. Therefore, D3/D5 can be used to ensure the downlink lossless delivery.

Proposal 4: D3/D5 is selected to ensure uplink lossless delivery. It’s up to RAN3 to decide whether support D4.

As discussed above, RAN3 should be informed about RAN2 decision to support U5 and discuss whether support D4. LS can be sent to RAN3.

Proposal 5: Send LS to RAN3 about RAN2 decision to support U5 and ask whether support D4.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: Not introduce comparison between SL-RSRP and SD-RSRP based event Z2.

Observation 1: Remote UE can obtain the SD-RSRP of serving relay UE without additional effort.

Proposal 2: Introduce SD-RSRP based event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE.

Proposal 3: U5 is selected to ensure uplink lossless delivery.

Proposal 4: D3/D5 is selected to ensure uplink lossless delivery. It’s up to RAN3 to decide whether support D4.

Proposal 5: Send LS to RAN3 about RAN2 decision to support U5 and ask whether support D4.

Reference

[1] 38.331