3GPP TSG-RAN WG2 Meeting #122	 R2- 2305219

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	7.9.5

Source:	Xiaomi

Title:	Discussion on SL DRX in U2N relay

Document for:	Discussion

Introduction

In [1], one of the objectives is as following,

4.	With a low priority, study the gains and, if needed, specify signalling between gNB and relay UE in sidelink mode 2 to assist the determination of the sidelink DRX configuration used for remote UE in Layer-2 UE-to-Network sidelink relay operation [RAN2]

In this contribution, we analysis the potential gain of DRX enhancement and give our proposals.

Discussion

Remote UE in RRC_IDLE/RRC_INACTIVE would establish unicast connection with relay UE after relay selection. SL DRX may be configured on the unicast connection for power saving. Furthermore, if the serving relay UE indicates Uu RLF or HO, remote UE may trigger RRC reestablishment via the same relay UE, if same relay UE is selected after relay selection. So, when remote UE trigger RRC establishment/resume/reestablishment procedure via relay UE, SL DRX may already be configured on the connection between remote UE and relay UE.

Observation 1: when remote UE trigger RRC establishment/resume/reestablishment procedure via relay UE, SL DRX may already be configured to save power on the connection between remote UE and relay UE

After transmission of the initial message, i.e. RRCSetupReqeust/RRCResumeRequest/RRCReestablishmentRequest, remote UE would start timer, i.e. T300/T319/T301. If the timer expires, remote UE would enter RRC_IDLE. On Uu, the NW would response before timer expires. However, remote UE may enter inactive due to sidelink DRX and the response message reception may be delayed. Relay UE may have to delay the transmission of response message to remote next active period. If the DRX cycle is longer than the timer length, i.e. T300/T319/T301, remote UE would enter RRC_IDLE due to timer expiry.

Observation 1: Remote UE’s RRC connection establishment/resume/reestablishment may fail due to SL DRX inactive.

Note TX UE would stay in active after transmission of DCR until reception of the RRCReconfigurationCompleteSidelink message. Similar procedure could be reused to avoid the failure for remote UE’s RRC establishment/resume/reestablishment. Remote UE shall stay in SL DRX active between transmission of RRCSetupReqeust/RRCResumeRequest/RRCReestablishmentRequest and reception of RRCSetup/RRCResume/RRCReestablishment/RRCReject/RRCRelease. The key point is to keep remote UE and relay UE synchronized of remote UE’s active time. From remote UE point of view, it can acknowledge the content of the RRC message. There is no problem for remote UE to decide the active time. From relay UE point of view, although it can’t acknowledge the content of the message directly, the message can be differentiated by the SL-RLC. The first RRC message from remote UE is transmitted via SL-RLC0, which could be RRCSetupReqeust/RRCResumeRequest/RRCReestablishmentRequest. The response message from NW can be transmitted via SL-RLC1, i.e. RRCSetup/RRCResume/RRCRelease, or via SL-RLC0, i.e. RRCReject. Relay UE can stay active between reception of the first message via SL-RLC0 and transmission of the first message via SL-RLC1 or SL-RLC0. So, both remote UE and relay UE can decide the active time in a synchronized way.

Observation 2：Relay UE can consider remote UE in SL DRX active between reception of message via SL-RLC0 and transmission of the first response message via SL-RLC1 or SL-RLC0.

Proposal 1: Remote UE shall stay in SL DRX active between transmission of RRCSetupReqeust/RRCResumeRequest/RRCReestablishmentRequest and reception of RRCSetup/RRCResume/RRCReestablishment/RRCReject/RRCRelease.

Note the response message from gNB may not occur immediately after remote UE’s initial message transmission. If remote UE and relay UE can delay entering active, the power saving gain could be further improved without delay the message transfer.

Conclusion

Based on the discussion in section 2, we have following proposals:

Observation 1: Remote UE’s RRC connection establishment/resume/reestablishment may fail due to SL DRX inactive.

Observation 2：Relay UE can consider remote UE in SL DRX active between reception of message via SL-RLC0 and transmission of the first response message via SL-RLC1 or SL-RLC0.

Proposal 1: Remote UE shall stay in SL DRX active between transmission of RRCSetupReqeust/RRCResumeRequest/RRCReestablishmentRequest and reception of RRCSetup/RRCResume/RRCReestablishment/RRCReject/RRCRelease.

Reference

[1] RP-213585 New WID on NR sidelink relay enhancements