3GPP TSG-RAN WG2 Meeting #122	R2-2305229

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.15.4

Source: 	Xiaomi

Title:  	Discussion on resource allocation for SL-U

Document for:	Discussion and Decision

1	Introduction

At RAN2#121 meeting, RAN2 discussed resource selection for SL unlicensed and reached some agreements [1]:

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

In addition, RAN1 sent LS to RAN2 to ask some question related to approaches to implement/achieve MCSt for SL-U communication in R1-2304257 [2].

In this contribution, we would like to discuss about issues related to resource selection and provide corresponding proposals.

2	Discussion

2.1 Resource allocation from MCSt

Based on the LS from RAN1, there are in total 3 approaches to achieve MCSt for single TB and multiple TBs of the same UE.

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA).

- Approach 1: “best effort for multiple TBs”

For approach 1, MAC triggers resource selection for different TB separately and delivers one set of parameters for each TB separately, which is purely R16/17 UE behaviour. With the set of parameter, PHY delivers single slot candidate resource set for each TB and then MAC performs resource selection from the delivered resource set for each TB separately. In this case to achieve MCSt for multiple TBs, MAC should select resource according to some consecutive-slots criterion from each set of candidate resource set for each TB and ensure each selected resource for each TBs consecutive.

Actually from MAC point of view, it is feasible to select the resources for multiple TBs consecutively. MAC just performs independent resource selection per TB from the delivered candidate resource set but during the selection procedure, MAC should try to guarantee the selected resource for different TB “consecutive”. Therefore the impact on MAC for approach 1 is to introduce some requirement on resource selection to ensure the selected resource for multiple TBs are consecutive. However how to determine if the two adjacent resources for two TBs are “consecutive” enough should base on RAN1 input or leave to UE implementation.

Observation 1: The impact on MAC for approach 1 is MAC should select resource according to some consecutive-slots criterion from each set of candidate resource set for each TB and ensure each selected resource for each TBs are consecutive.

- Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs”

For approach 2, MAC triggers resource selection for different TB separately and delivers one set of parameters for each TB separately, however, different from R16/17, additional parameter “number of slots for MCSt” should be delivered to PHY. With the set of parameter, PHY delivers candidate resource set for each TB, however different from R16/17, the candidate resource is not single-slot but multi-slot resource. Then MAC performs resource selection from the delivered resource set for each TB separately as in R16/17 and the selected resource is multi-slot for each TB. In this case, the key point to achieve the MCSt is solved in PHY, i.e., PHY derives and delivers the “multi-slot” resource set to MAC. From MAC point of view, the impact is to deliver an additional parameter “number of slots for MCSt” to PHY and there is no difficulty since this parameter can be determined based on the CAPC of the generated MAC PDU, which is also known by MAC.

Observation 2: The impact on MAC for approach 2 is MAC should deliver an additional parameter “number of slots for MCSt” to PHY.

In this case, for a single TB, the selected resource is multi-slot, which is consecutive and can be regarded as some kind of repetition. However for different TB, if MAC just follows legacy R16/17 behaviour, the selected different sets of “multi-slot” resource for different TBs may not be consecutive. But if similar enhancement on MAC as in approach 1 is adopted, then MAC may ensure the “multi-slot” resource for different TBs are consecutive. However, different from approach 1, the resource for different TB are different sets of “multi-slot” resource, then it seems more complicated to judge whether the “multi-slot” resource for different TB is consecutive or not, e.g., rely on the interval between the last slot for a certain TB and the first slot for another  TB etc. In addition, the “multi-slot” resource for different TB maybe interlaced, which will increase the complexity of the judgement of “consecutive”.

Observation 3: With approach 2, the MCSt for a single TB is guaranteed, while for multiple TB, if MAC behaves as in R16/17, the “multi-slot” resource for multiple TBs may not be consecutive.

Observation 4: Compared with approach 1, it is more complicated to judge whether the “multi-slot” resource for different TB is consecutive or not in approach 2.

- Approach 3: “guarantee MCSt for multiple TBs”

For approach 3, MAC triggers resource selection for different TB only one time and delivers only one set of parameters for different TB. This is different from R16/17 and the impact on the specification may be significant, since in R16/17, for each TB resource selection is triggered independently and the set of parameters is delivered for each TB independently. In addition, it remains unclear and needs further discussion on how to derive a single set of parameters for multiple TBs, especially when the QoS requirement of multiple TBs are quite diverse.

Similar as approach 2, additional parameter “number of slots for MCSt” should be delivered to PHY. But different from approach 2, since there is only one set of parameters for multiple TBs, this additional “number of slots for MCSt” is derived from the CAPC of multiple TBs. With the set of parameter, PHY delivers one multi-slot candidate resource set for multiple TBs and then MAC performs resource selection from the delivered resource set as in R16/17 and the selected resource is multi-slot for multiple TBs. In this case, for multiple TB, the selected resource is multi-slot, which is consecutive.

Observation 5: The impact on MAC for approach 3 is to trigger resource selection for multiple TBs only one time and deliver only one set of parameters including an additional parameter “number of slots for MCSt” to PHY.

Observation 6: The impact on MAC may be significant since the existing resource selection trigger and parameter deliver is operated per TB independently.

Observation 7: It remains unclear and needs further discussion how to derive a single set of parameters for multiple TBs considering different QoS requirements.

Therefore, based on the above analysis, we think we can reply the questions from RAN1 as below.

Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs) Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

Proposal 1: RAN2 to discuss and agree the following reply to RAN1 on MCSt.

Reply to Question 1: RAN2 considers it is feasible for MAC to select the resources for multiple TBs consecutively for approach 1. For approach 2, it is more complicated to judge whether the “multi-slot” resource for different TB is consecutive or not, e.g., whether interlace is allowed or not.

Reply to Question 2: RAN2 considers it may introduce significant impact on MAC since the existing resource selection trigger is operated per TB independently. It is also unclear how to derive a single set of parameters for multiple TB considering different QoS requirement.

Reply to Question 3: RAN2 considers it is feasible for MAC to provide the new parameter “number of slots for MCSt” to PHY.

2.2 Resource allocation from the LBT impact

Actually we have sent LS to RAN1 to inform RAN1 the conclusion from RAN2 on both inter-UE case and intra-UE case. However during the discussion in RAN1, the majority view is that both inter-UE case and intra-UE case should be discussed by RAN1.

- Approach 1 (4): OPPO, ZTE, LGE, Spreadtrum - No duplicate work between RAN1/RAN2 on same issue, optimization issue, no need to prioritize this issue, respect to RAN2 decision - Approach 2 (13): CATT/GH, QC, Intel, Futurewei, DCM, MediaTek, vivo, Samsung, CMCC, HW/HiSi, Lenovo - RAN1/RAN2 solution scope is different, LBT parameters and resource selection are RAN1 domain

In addition, there was some candidate options to handle the LBT blocking issue from another UE (inter-UE case), see below. At least option 1, which is to avoid selection of consecutive resources that overlaps with the LBT of the reserved resource, can be easily extended to handle the intra-UE case, i.e., similar approach can be applied to MAC when selecting resources from the candidate resource set to avoid the LBT blocking issue. Therefore, we think to avoid the duplicated discussion in different WGs and considering RAN1 is still doing down selection among the candidate solutions for inter UE case, it is better to wait for more RAN1 input even for intra-UE case.

To resolve the Type 1 LBT blocking issue, where one UE performing a Type 1 LBT procedure for using its own selected/reserved resource(s) is blocked by another UE’s SL transmission at least in a slot preceding to the selected/reserved resource and causing the LBT to fail, further study the following options in a future meeting. - Option 1: - UE avoid selection of N consecutive resource(s) before a reserved resource with high priority when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource. - UE avoid selection of N consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection - FFS: whether / how to achieve this in RA mode 1 - FFS: How to determine value of N - Option 2: - UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource). - UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the COT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource). - FFS whether / how to achieve this in RA mode 1. - Option 3: UE selects extra / more resources than required for transmitting a TB (i.e., overbooking) to accommodate potential Type 1 LBT failures. FFS how to determine/preconfigure the number of extra selected resources. - Option 4: The expected LBT duration is determined firstly, then resource selection takes into account of the expected LBT duration is performed. - Option 5: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection. - Option 6: UE excludes frequency resources (if any) previously reserved via SCI by other SL UEs in the corresponding slot, when estimating the detected power within a sensing slot duration in Type 1 channel access. - Option 7: SL UE deems channel busy only if the UE detects transmission other than SL transmission occupying the channel (e.g., exceeding the energy detection threshold), i.e., the energy detection for EDT checking in LBT procedure does not take into account the energy from SL transmissions. - Option X: No solution is needed. To avoid inter-UE blocking from performing Type 1 LBT can be handled based on UE implementation (e.g., as the start timing to perform LBT sensing is determined by each UE).

Proposal 2: RAN2 to wait for more RAN1 input on type 1 LBT blocking issue even for intra-UE case.

2.3 Resource allocation from COT sharing

With the introduction of IUC, UE shall randomly select the resources within the intersection of the received preferred resource set and the resources indicated by the physical layer when preferred resource set is received and UE has its own sensing result. Similarly, even the detailed procedure of resource allocation is still under discussion in RAN1, from MAC perspective, some mechanism that impact the selection procedure in higher layer can be considered by RAN2. Since less effort on LBT is required for resources within the shared COT, when the UE is the responding UE and operates in mode 2, it is beneficial to firstly select resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection. If there is no remaining resource within the shared COT that can be selected, then the UE randomly select resources from the resources indicated by the physical layer as in legacy.

Proposal 3: When the UE is the responding UE and operates in mode 2, the UE firstly selects resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection.

3	Conclusion

In this contribution, we discussed about resource allocation for SL-U and have the corresponding observation and proposals:

Observation 1: The impact on MAC for approach 1 is MAC should select resource according to some consecutive-slots criterion from each set of candidate resource set for each TB and ensure each selected resource for each TBs are consecutive.

Observation 2: The impact on MAC for approach 2 is MAC should deliver an additional parameter “number of slots for MCSt” to PHY.

Observation 3: With approach 2, the MCSt for a single TB is guaranteed, while for multiple TB, if MAC behaves as in R16/17, the “multi-slot” resource for multiple TBs may not be consecutive.

Observation 4: Compared with approach 1, it is more complicated to judge whether the “multi-slot” resource for different TB is consecutive or not in approach 2.

Observation 5: The impact on MAC for approach 3 is to trigger resource selection for multiple TBs only one time and deliver only one set of parameters including an additional parameter “number of slots for MCSt” to PHY.

Observation 6: The impact on MAC may be significant since the existing resource selection trigger and parameter deliver is operated per TB independently.

Observation 7: It remains unclear and needs further discussion how to derive a single set of parameters for multiple TBs considering different QoS requirements.

Proposal 1: RAN2 to discuss and agree the following reply to RAN1 on MCSt.

Reply to Question 1: RAN2 considers it is feasible for MAC to select the resources for multiple TBs consecutively for approach 1. For approach 2, it is more complicated to judge whether the “multi-slot” resource for different TB is consecutive or not, e.g., whether interlace is allowed or not.

Reply to Question 2: RAN2 considers it may introduce significant impact on MAC since the existing resource selection trigger is operated per TB independently. It is also unclear how to derive a single set of parameters for multiple TB considering different QoS requirement.

Reply to Question 3: RAN2 considers it is feasible for MAC to provide the new parameter “number of slots for MCSt” to PHY.

Proposal 2: RAN2 to wait for more RAN1 input on type 1 LBT blocking issue even for intra-UE case.

Proposal 3: When the UE is the responding UE and operates in mode 2, the UE firstly selects resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection.

4	Reference

[1] RAN2#121 meeting minutes.

[2] R1-2304257 LS on MCSt resource (re-)selection, RAN1

5	Annex

3GPP TSG-RAN WG2 Meeting #122	R2-230xxxx

Incheon, Korea, May 22-26, 2023

Title:	[Draft] LS response on MCSt resource (re-)selection

Response to:	R2-2304618(R1-2304257)

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	Xiaomi [To be RAN2]

To:	RAN1

Cc:	-

Contact person:	Li Zhao

zhaoli6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

RAN2 would like to thank RAN1 for the LS on MCSt resource (re-)selection. RAN2 discussed about the questions and agreed to reply the questions as below.

Reply to Question 1: RAN2 considers it is feasible for MAC to select the resources for multiple TBs consecutively for approach 1. For approach 2, it is more complicated to judge whether the “multi-slot” resource for different TB is consecutive or not, e.g., whether interlace is allowed or not.

Reply to Question 2: RAN2 considers it may introduce significant impact on MAC since the existing resource selection trigger is operated per TB independently. It is also unclear how to derive a single set of parameters for multiple TB considering different QoS requirement.

Reply to Question 3: RAN2 considers it is feasible for MAC to provide the new parameter “number of slots for MCSt” to PHY.

- Actioins

To RAN1

RAN2 respectfully asks RAN1 to take into account the above information in RAN1’sfuture work.

- Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #123	August 21 – August 25, 2023	Toulouse, FR