3GPP TSG-RAN WG2 Meeting #122	           R2-2305234

Incheon, Korea, May 22th– May 26th, 2023

Agenda item:	7.9.3

Source:	China Telecom

Title:	Discussion on lossless delivery solution for inter-gNB path switching

Document for:	Discussion

- Introduction

Regarding the UL/DL lossless delivery for inter-gNB path switching, during RAN2#121bis e-meeting, RAN2 have the following agreements [1]:

For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305. Other solutions are not pursued. For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

In this paper, we would like to further discuss the UL/DL lossless delivery solutions for inter-gNB path switch scenarios.

- Discussion

2.1	UL Lossless data delivery

- Solution U3: Remote UE’s PDCP retransmission based on DL PDCP Status Report from target gNB

- Solution U5: Source Relay UE continues to transmit UL data to source gNB and gNB forwards to the target gNB

Regarding solution U5, it requires that the source gNB still keeps the remote UE’s/relay UE’s context even after the remote UE switches to the target gNB. A new end marker from the relay UE to the source gNB would be needed to indicate the end of relaying data for the remote UE after all the packets received from the remote UE are successfully received by the source gNB. The new end marker could be RLC layer or adaptation layer control information, such as control PDU. Furthermore, the source gNB should make sure that the remote UE’s/relay UE’s context are released after receiving the new end maker.

Observation 1: Regarding solution U5, a new end marker from the relay UE to the source gNB is needed to indicate the end of relaying data for the remote UE after all the packets received from the remote UE are successfully received by the source gNB, in order to ensure UL lossless delivery.

Regarding solution U3, since the target gNB has the HFN and corresponding SN for UL provided by the source gNB as legacy HO, the target gNB can send PDCP status report to the remote UE after the remote UE switch to the target gNB, and the remote UE can retransmit PDCP SDUs which are not acknowledged by the PDCP status report. Solution U3 can ensure UL lossless data delivery during inter-gNB i2x path switching without unnecessary retransmission, and has no additional constraint with the timing to reconfigure the relay UE to release the relaying configuration. Moreover, it has small specification impact. Since RAN2 has agreed that solutions based on the PDCP status report mechanism are the baseline during RAN2 121 meeting [2], we suggest to adopt solution U3 to following the previous agreements and minimize the specification impact.

Proposal 1: Adopt solution U3 to resolve UL lossless delivery for inter-gNB i2x path switch.

According to the legacy PDCP status report reception behaviour, the UE discard the successfully delivered PDCP SDUs which are confirmed by the PDCP status report, but not retransmit the PDCP SDUs which have not been successfully delivered. The control PDU for PDCP status report can be enhanced to indicate the UE to retransmit the PDCP SDUs which have not been successfully delivered after receiving the PDCP status report. Two possible options are listed as the following:

- Option 1: one reserved filed ‘R’ is used to indicate retransmission when an enhanced PDCP status report is received.

- Option 2: one of the reserved values for the filed ‘PDU type’ is used for the enhanced PDCP status report.

Proposal 2: The control PDU for PDCP status report can be enhanced to indicate the UE to retransmit the PDCP SDUs which have not been successfully delivered.

Obviously, this mechanism also applies to the intra-gNB i2x cases. Since the specification impact is the same, it’s not necessary to exclude the intra-gNB i2x path switch cases.

Proposal 3: Reuse the UL lossless delivery solution for the inter-gNB i2x path switch to resolve UL lossless delivery for intra-gNB i2x path switch.

2.2	DL Lossless data delivery

- Solution D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

- Solution D3: A new PDCP status report sent from Remote UE to the source gNB

- Solution D5: Proactive Data forwarding from source gNB to target gNB

Regarding solution D3, it requires to specify a new trigger for PDCP status report before the remote UE perform path switching. As indicated by the email discussion rapporteur in R2-2304305 [3], solution D3 can only work if the source gNB can receive an accurate PDCP status report before the SN status transfer, and assumes the the source gNB can send the required data to the target gNB during path switch. Since the path switch decision implies that the source PC5 link condition is poor, it is probably that the PDCP status report will not be accurately received. Thus it is an unreliable solution.

Regarding solution D5, the source gNB voluntarily forwards all the buffered data to the target gNB based on the source gNB implementation. This solution leads to unnecessary forwarding a lot of data, which is inefficient.

Regarding solution D4, after the remote UE switch to the target gNB, the target gNB can configure the remote UE to send PDCP status report. Then the target gNB can request the source gNB to forward the PDCP SDUs which have not been acknowledged by the PDCP status report from the remote UE for DL lossless delivery. This solution ensures DL lossless delivery during path switch without unnecessary data forwarding. And it is consistent with the previous agreements that solutions based on the PDCP status report mechanism are the baseline. The Xn data forwarding details can be decided by RAN3.

Proposal 4: Adopt solution D4 to resolve DL lossless delivery for inter-gNB i2x path switch.

If solution D4 is adopted, we suggest to send LS to RAN3 to inform our decision and ask RAN3 to consider Xn data forwarding details.

Proposal 5:  If solution D4 is adopted, RAN2 send LS to RAN3 to inform RAN2’s decision and ask RAN3 to consider Xn data forwarding details.

3	Conclusions

Based on the discussion above, we have the following proposals.

Observation 1: Regarding solution U5, a new end marker from the relay UE to the source gNB is needed to indicate the end of relaying data for the remote UE after all the packets received from the remote UE are successfully received by the source gNB, in order to ensure UL lossless delivery.

Proposal 1: Adopt solution U3 to resolve UL lossless delivery for inter-gNB i2x path switch.

Proposal 2: The control PDU for PDCP status report can be enhanced to indicate the UE to retransmit the PDCP SDUs which have not been successfully delivered.

Proposal 3: Reuse the UL lossless delivery solution for the inter-gNB i2x path switch to resolve UL lossless delivery for intra-gNB i2x path switch.

Proposal 4: Adopt solution D4 to resolve DL lossless delivery for inter-gNB i2x path switch.

Proposal 5: If solution D4 is adopted, RAN2 send LS to RAN3 to inform RAN2’s decision and ask RAN3 to consider Xn data forwarding details.

4	References

- [1]	R2-2304204, Report from session on positioning and sidelink relay, Session Chair (MediaTek).

- [2] R2-2301904, Report from session on positioning and sidelink relay, Session Chair (MediaTek).

- [3] R2-2304305, Summary of [AT121bis-e][432]Candidate solutions for lossless delivery, NEC.