3GPP TSG-RAN WG2 Meeting #122                                                                  R2-2305242

Incheon, Korea, 22 – 26 May 2023

Source:	CATT

Title:	Discussion on Rel-18 UL Tx Switching

Agenda Item:	7.25.2‎

Document for:	Discussion and Decision

- Introduction

In RAN2#119-e meeting, Rel-18 UL Tx switching was discussed and made the following conclusions:

- As a baseline, RAN2 reuse Rel-16/17 UL Tx switching band combination list (i.e. BandCombinationList-UplinkTxSwitch-r16) for Rel-18 UL Tx switching capability reporting. - As a baseline, uplink bands for Rel-18 UL Tx switching are configured as in legacy way, i.e. by UplinkConfig. - RAN2 waits for RAN1/4 input and then addresses the potential issues according to RAN1/4 indication, e.g.: – whether the switching period is configured per band pair or per band combination on UE capability reporting. – whether the switching option (i.e. switchedUL or dualUL) is configured per band pair or per band combination on UE capability reporting. – how RRC configures a period location for each band pair within three or four bands on RRC configuration. – how to configure a state of Tx chains after the UL Tx switching is not unique in Rel-18 framework on RRC configuration.

In RAN2#120 meeting, agreements were agreed as below:

- R2 assumes For UE capability to report applicability of DL interruption for Rel-18 UL Tx switching, RAN2 reuses uplinkTxSwitching-DL-Interruption-r16 (no spec impact). - R2 assumes to reuse the per band per BC capability, uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17, on UL-MIMO coherence for the 2Tx-capable UL band(s) for Rel-18 UL Tx switching (fallback description FFS).

In RAN2#121 meeting[2], further agreements were agreed as below:

- For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching. - configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination) - For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band. - For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

In RAN2#121bis-e meeting[3], further agreements were agreed as below:

- P2: RAN2 reuse uplinkTxSwitching-DualUL-TxState-r17 to indicate the state of Tx chains for dualUL mode. - P4: RAN2 introduce an optional list of bands in CellGroupConfig, in which the priority is configured by the order (or similar equivalent change, TBD CR disc). - P5: RAN2 introduce a per-band-pair report of bands that can be transmitted while the other Tx chain is switching across that band pair. Absence of this field means there is interruption in all bands during the switching. - P3-1: R2 assumes that the network ensures the UE supports dualUL for a band and its associated band (config restriction) - we send an LS to RAN1, ask to confirm RAN2 understanding (below) When the UE is indicated to switch from two bands to one different band (e.g. A+B => C), follow below logic when determine the switched Tx: - If network indicates 1port transmission on band C, and uplinkTxSwitching-DualUL-TxState is set to oneT, and the associated band is configured to band C: ---- Switch 1Tx chain to band C and switch another Tx chain to associated band; - Else if network indicates 1port transmission on band C, but uplinkTxSwitching-DualUL-TxState is not configured or is set to twoT, or associated band is not configured to band C: ---- Switching 2Tx chains to band C. - In support of RAN4 agreement, RAN2 intend to introduce support for two per-band-pair UE capabilities, a length of a switching period, for 1Tx-2Tx switching (like Rel-16) and that for 2Tx-2Tx switching (like Rel-17). - FFS if the UE supports 1T-2T, whether the UE need to report this capability for every case (or whether it could/should be inferred from R1617 reporting). - FFS if the absence of 2Tx-2Tx per-band-pair UE capability (switching period) means the UE does not support 2Tx-2Tx UL Tx switching. - For Rel-18 Tx switching, wait for more input from RAN4 on whether to introduce a separate capability for UE transmitting on the Tx chain switched first during the time gap of different switching periods.

In this contribution, we discuss the leftover issues for Rel-18 UL Tx switching across 3 or 4 bands.

- Discussion

  - Associated band configuration

In RAN1#111 meeting[1], RAN1 agreed to introduce a new RRC parameter to indicate the Tx switching associated band:

Agreement In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives. - An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band) - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}. - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C) - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D) If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled

Also, in RAN2#121 meeting[2], RAN2 agreed associated band configuration:

- For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band.

Further in RAN2#121bis meeting[3], RAN2 clarified gNB configures associated band of a band only for the band pair supports dualUL.

- P3-1: R2 assumes that the network ensures the UE supports dualUL for a band and its associated band (config restriction)

However, based on RAN2’s agreements, there is only condition for gNB not configuring the associated band, which means gNB may not configure associated band for a band even if UE supports concurrent transmission on the band and another band. However, from our understanding on RAN1’s agreements highlighted in blue, gNB does not configure an associated band for the band when there is one band where concurrent transmission with any other band is not supported, which means gNB shall always configure an associated band for the band if there is one band where dualUL with the band is supported by UE. The different understandings for configuring the associated band will affect the description of this IE in 331. RAN2 can discuss this issue, there are two alternatives:

- Alt. 1: align with RAN1’s agreement to clarify gNB should configure an associated band for a band if one band where dualUL with this band is supported by UE.

- Alt. 2: keep RAN2’s agreements, i.e., gNB ensures the UE supports dualUL for a band and its associated band, whether configure an associated band for the band is up to gNB implementation.

Proposal 1: RAN2 discusses whether gNB should always configure an associated band for the band if there is at least one band where dualUL with this band is supported by UE.

- Alt. 1: align with RAN1’s agreement to clarify gNB should configure an associated band for a band if there is one band where dualUL with this band is supported by UE.

- Alt. 2: keep RAN2’s agreements, i.e., gNB ensures the UE supports dualUL on a band and its associated band, whether configure an associated band for the band is up to gNB implementation.

  - Fallback issues

In RAN4#106bis-e meeting, RAN4 sent an LS [4] to RAN1 and RAN2 for informing the following agreements on the fallback of Rel-18 Tx switching to Rel-16/17 Tx switching:

RAN4 has also discussed the fallback of Rel-18 Tx switching to Rel-16/17 Tx switching, and reached the following agreement and understanding: - The indication of supported switching band combinations of the UE capability can also imply the capabilities of the fallbacks in accordance with the current 38.331 fallback framework. - Rel-18 Tx switching capability will include all sufficient capabilities for two-band switching. In addition, UE may also declare two-band Tx switching capabilities with Rel-16 and/or Rel-17 capabilities. If UE is configured for two-band Tx switching only and UE declares those Rel-16/17 capabilities, then Rel-16/17 capabilities apply for this two band Tx switching.

Basing on above RAN4’s agreements, Rel-18 UL Tx switching band combination of UE capability follows current fallback framework and Rel-18 UL Tx switching capability will include all sufficient capabilities for two-band switching. That means UE will report UL Tx switching capabilities for the band pairs supported by Rel-16/17 in Rel-16/17 UL Tx switching band combination and report UL Tx switching capabilities supported by Rel-18 in Rel-18 UL Tx switching band combination. The band pairs involved in the Rel-16/17 Tx switching band combinations should also be included in the band pair list of the Rel-18 Tx switching band combination. And the switching options supported by Rel-16/17 are also supported by Rel-18 on those band pairs.

Proposal 2: UE reports UL Tx switching capabilities for the band pairs supported by Rel-16/17 in Rel-16/17 UL Tx switching band combination, and report UL Tx switching capabilities supported by Rel-18 in Rel-18 UL Tx switching band combination respectively.

Referring to agreement of last RAN2 meeting, two per-band-pair UE capabilities for Rel-18 tx switching are intended to be introduced including a length of a switching period for 1Tx-2Tx switching and that for 2T-2T switching. But it is not clear when UE needs to report those capabilities for Rel-18. In our perspective, if UE supports 2T-2T Tx switching (i.e., also support 1T-2T Tx switching) on a band pair for Rel-18, UE needs to report 2T-2T switching period for Rel-18, otherwise, i.e., UE only supports 1T-2T Tx switching on the band pair for Rel-18, UE should report 1T-2T switching period for Rel-18.

Proposal 3:  UE reports 2T-2T switching period for a band pair for Rel-18 only if UE supports 2T-2T Tx switching on the band pair for Rel-18, otherwise, UE reports 1T-2T switching period for the band pair for Rel-18.

On the other hand, the overlapping capabilities between Rel-16/17 Tx switching and Rel-18 Tx switching give room for RAN2 to design the UE capability message to reduce the signalling overhead. The straightforward method is using Rel-16/17 switching capability between 2 bands to indicate the capabilities for Rel-18 UL Tx switching if the capabilities are the same for Rel-16/17 and Rel-18.

Proposal 4: For saving the signal, RAN2 to discuss using Rel-16/17 switching capabilities between 2 bands to report the capabilities for Rel-18 UL Tx switching if the capabilities are the same for Rel-16/17 and Rel-18.

In our view, if the period for 1T-2T tx switching for a band pair supported by Rel-18 is the same as the period for 1T-2T tx switching supported by Rel-17 for the same band pair, the new 1T-2T switching period can be omitted.

Proposal 5: UE can omit reporting the switching period for Rel-18 1T-2T Tx switching on a band pair if it is the same as the switching period for Rel-17 1T-2T Tx switching on the same band pair.

Since 2T-2T tx switching on a band pair may not be supported, indicator on supporting 2T-2T Tx switching across 3/4 bands is needed. However, RAN2 cannot decide whether to use the existence of 2T-2T Tx switching period to indicate UE supporting 2T-2T switching on a band pair for Rel-18. And RAN2 has consulted RAN4 for that issue. RAN2 can wait for RAN4’s reply to consider signalling design with respect to reporting Rel-18 2T-2T Tx switching period.

Proposal 6: RAN2 waits for RAN4’s reply to consider signalling design with respect to reporting Rel-18 2T-2T Tx switching period.

- Conclusion

In this contribution, we discuss the leftover issues for Rel-18 UL Tx switching across 3 or 4 bands, based on which we have the following proposals.

Proposal 1: RAN2 discusses whether gNB should always configure an associated band for the band if there is at least one band where dualUL with this band is supported by UE.

- Alt. 1: align with RAN1’s agreement to clarify gNB should configure an associated band for a band if there is one band where dualUL with this band is supported by UE.

- Alt. 2: keep RAN2’s agreements, i.e., gNB ensures the UE supports dualUL on a band and its associated band, whether configure an associated band for the band is up to gNB implementation.

Proposal 2: UE reports UL Tx switching capabilities for the band pairs supported by Rel-16/17 in Rel-16/17 UL Tx switching band combination, and report UL Tx switching capabilities supported by Rel-18 in Rel-18 UL Tx switching band combination respectively.

Proposal 3:  UE reports 2T-2T switching period for a band pair for Rel-18 only if UE supports 2T-2T Tx switching on the band pair for Rel-18, otherwise, UE reports 1T-2T switching period for the band pair for Rel-18.

Proposal 4: For saving the signal, RAN2 to discuss using Rel-16/17 switching capabilities between 2 bands to report the capabilities for Rel-18 UL Tx switching if the capabilities are the same for Rel-16/17 and Rel-18.

Proposal 5: UE can omit reporting the switching period for Rel-18 1T-2T Tx switching on a band pair if it is the same as the switching period for Rel-17 1T-2T Tx switching on the same band pair.

Proposal 6: RAN2 waits for RAN4’s reply to consider signalling design with respect to reporting Rel-18 2T-2T Tx switching period.

- Reference

- Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0

- Minutes report, RAN2 Chairman, RAN2#121

- Minutes report, RAN2 Chairman, RAN2#121bis-e

- R4-2306623, LS on Rel-18 Tx switching across 3/4 bands, to RAN1 and RAN2, RAN4#106bis-e