3GPP TSG RAN2 Meeting #122                                                              R2-2305252

22nd May – 26th May 2023, Incheon, South Korea

Agenda item:	7.3.5

Source:	Samsung

Title:	Discussion on Connected mode mobility for NES

Document for:	Discussion & Decision

Introduction

During the previous RAN2 #121 and #121bis-e meetings, RAN2 discussed whether NES techniques are required to be supported for CHO enhancements utilizing serving and target cell NES status. And RAN2 have the following agreements:

Agreements

1.	Study whether CHO enhancements are needed for the purpose of turning off the cell

2.	Continue discussing CHO in the context of different NES techniques.

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

Moreover, within the offline discussion [1], RAN2 has discussed the related issue and have the following remained proposals:

Proposal 1: [13/ 25] Cell DTX/ DRX and cell switch off are considered separate NES techniques. FFS if there is any difference from a UE's perspective when network decides to conditionally handover this UE to another cell. Proposal 3: [22/ 25] RAN2 assumes that NES Mode toggling is stable/ slow (in seconds or minutes) but does not preclude more frequent NES mode changes. Any optimizations for more frequent NES mode changes, should be further discussed e.g., on a case by case basis. Proposal 6: The exact triggers to start RF measurement condition evaluation and execution needs further discussion. Proposal 7: [21/ 25] Legacy measurement events A3, A4 and A5 are considered as NES CHO conditions. Proposal 8: RAN2 to discuss following two options to ensure that the UE can find a suitable target cell: - Option 1: [11/ 25] NW implementation to reconfigure candidate cells, i.e., no spec impact - Option 2: [9/ 25] Network provides additional prioritization for candidate cells that UE can take into consideration. Proposal 9: [13/ 25] RAN2 further discuss if legacy connection re-establishment is an appropriate way to handle the case when no suitable candidate it found, and the source cell is about to sleep.

In this contribution, we discuss the possible enhancements of UE mobility especially when the UE is in an NES cell.

Discussion

For the CHO triggering conditions and associated conditions, we may consider various scenarios for UE to determine whether the serving cell is switching to NES mode and the target cell is not in the NES mode. As an example, a UE may acknowledge that the serving or candidate cell is switched to NES mode if the preconfigured pattern indicates so. As another example, a UE may acknowledge that the serving or candidate cell will switch to NES mode if the UE receives a signal which indicate so. Since there are enough signals and information, we propose the new triggering condition for the conditional handover (CHO) would utilize such information.

Hence, from the source cell, we believe a CHO configuration could have more than one of the following events:

- A3

- A4

- A5

- A new condition of “NES activation signal reception”

Proposal 1. RAN2 needs to discuss and determine the conditions in a CHO configuration, which could be one or more than one of conditions of : A3, A4, A5, and a new condition of “NES activation signal reception”

In order for the NES UE to decide on a target cell among candidate cell(s) that have fulfilled handover condition(s), additional information such as NES status of each candidate cell, needs to be provided by the source cell to the UE.

Considering each cell operating with L1 activation/deactivation signal, now it seems impossible for a UE to detect the cell’s NES status without assistance of the serving cell.

Therefore, we believe such additional information needs to be defined and shared from the candidate cells to the UE, via source cell or via broadcasting signal.

Proposal 2. For a UE to decide the CHO target cell, RAN2 needs to discuss the need of candidate cell’s NES status information, especially when there are L1 (or L2) activation/deactivation signals within each cell.

In case for a CHO failure case, if there is no candidate cell which is good enough to trigger CHO, then the CHO shall not be triggered. CHO is only for the quick handover within the network configured conditions met.

Even if the CHO is not triggered earlier, eventually the UE can trigger CHO having signal strength based conditions such as A3, so no need to introduce such new behavior.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. RAN2 needs to discuss and determine the conditions in a CHO configuration, which could be one or more than one of conditions of : A3, A4, A5, and a new condition of “NES activation signal reception”

Proposal 2. For a UE to decide the CHO target cell, RAN2 needs to discuss the need of candidate cell’s NES status information, especially when there are L1 (or L2) activation/deactivation signals within each cell.

Reference

[1] R2-2304353 Summary of [AT121bis-e][303][NES] Conn mode mob (Lenovo)