3GPP TSG RAN2 Meeting #122                                                                R2-2305269

Incheon, Korea, May 22-26, 2023

Agenda item:	7.21.3

Source:	NEC

Title:	UP Impacts for Further NR Coverage Enhancements

Document for:	Discussion & Decision

Introduction

In this contribution, we will discuss UP impacts for PRACH coverage enhancements based on the agreements of RAN1 and RAN2.

Discussion

CFRA

In RAN2 #121bis [1], the agreement whether MSG1 repetition can be applicable to CBRA and CFRA is as following.

Agreements RAN2 assumes that MSG1 repetition can be applicable to all 4-step CBRA procedures (FFS for SI request) CFRA support is FFS

In this agreement, whether MSG1 repetition can be applicable to CBRA procedure of SI request is FFS. In our understanding, for SI request, it is beneficial to apply MSG1 repetition when the UE is at cell edge, so there is no necessary to exclude this scenario, i.e. MSG1 repetition can be applicable to all 4-step CBRA procedures.

For CFRA procedure, there are the following four triggering scenarios according to TS 38.300 [2]:

- Request by RRC upon synchronous reconfiguration (e.g. handover); - To establish time alignment for a secondary TAG; - MSG1 based SI request; - Beam failure recovery;

For MSG1 based SI request, even if there is no SSB above a threshold, CFRA procedure is still performed, which may result in RACH failure when the channel condition is bad. In this case, it is beneficial to transmit multiple PRACHs. For time alignment of STAG, when the UE is at cell edge, the channel condition may be poor. It is helpful to apply MSG1 repetition to increase the probability of access success. Besides, for BFR and HO procedure, though CFRA is performed only when there is SSB above a threshold, MSG1 repetition can increase the successful probability of PRACH transmission when the UE is at cell edge. Therefore, for the above CFRA procedures, we think it is useful to apply MSG1 repetition. MSG1 repetition for CFRA procedure should be supported.

proposal 1: MSG1 repetition can be applicable to all 4-step CBRA procedures.

Proposal 2: MSG1 repetition can also be applicable to CFRA procedures.

RA-RNTI

In RAN1 #112bis [3], the following agreement for starting point of RAR window was reached.

Agreement The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

Firstly, for MSG1 repetition, in order to reduce UE power consumption and blind decoding, RA-RNTI should be only associated with single RO in one RO group. Secondly, according to the current agreement, RAR window is started after the last symbol of the last valid RO in the RO group. Then it is very natural to calculate RA-RNTI based on the last RO of the RO group.

Proposal 3: RA-RNTI is associated with the last valid RO in the RO group.

Multiple PRACH occasions

The UE can select the number of PRACH transmissions at least for the first RACH attempt based on the configured SSB-RSRP threshold based on RAN1’s agreement. When the number of PRACH transmissions is selected, the UE shall select  one SSB. In the following, the UE selects the available PRACH occasion(s) based on the selected number of PRACH transmission and SSB. In the current TS 38.321 [4], only one PRACH occasion is selected and informed to the lower layer. Then for MSG1 repetition, a simple way is to select multiple PRACH occasions once in the procedure of random access resource selection. For multiple PRACH occasions, how to inform the lower layer, the following two options may be considered:

Option 1: All PRACH occasions in an attempt are indicated to the lower layer together.

Option 2: All PRACH occasions are separately indicated to the lower layer.

For option 1, since multiple PRACH occasions are indicated to lower layer together, the procedure below is performed once. Then for power ramping, it is not applied within one RACH attempt according to the current agreement. In our understanding, the current SPEC may be reused. While for the calculation of PREAMBLE_RECEIVED_TARGET_POWER and RA-RNTI, the current SPEC may be updated based on the future discussion. So option 1 seems to have a smaller impact on SPEC.

The MAC entity shall, for each Random Access Preamble: 1> if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and 1> if the notification of suspending power ramping counter has not been received from lower layers; and 1> if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and 1> if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1. 1> select the value of DELTA_PREAMBLE according to clause 7.3; 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA; 1> except for contention-free Random Access Preamble for beam failure recovery request, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted; 1> instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER.

For option 2, since each PRACH occasion is separately indicated, whether PRACH resource selection and preamble transmission procedure can be separately performed for each MSG1 repetition needs to be considered. If so, there will be a big impact on SPEC.

Therefore, in our understanding, it is necessary to discuss how to inform multiple PRACH occasions.

Proposal 4: RAN2 to discuss the following ways of indicating all PRACH occasions within one attempt:

- All PRACH occasions in one attempt are indicated to the lower layer together

- All PRACH occasions are separately indicated to the lower layer

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposal:

proposal 1: MSG1 repetition can be applicable to all 4-step CBRA procedures

Proposal 2: MSG1 repetition can also be applicable to CFRA procedure.

Proposal 3: RA-RNTI is associated with the last valid RO in the RO group.

Proposal 4: RAN2 to discuss the following ways of indicating all PRACH occasions within one attempt:

- All PRACH occasions in one attempt are indicated to the lower layer together

- All PRACH occasions are separately indicated to the lower layer

References

[1] Draft_R2_121bis_meeting_report_v1.

[2] TS 38.300 h40.

[3] Draft_Minutes_report_RAN1#112b-e_v010.

[4] TS 38.321 h40.