3GPP TSG-RAN WG2 Meeting #122	R2-2305272

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item		:	7.4.2.2

Source		:	LG Electronics Inc.

Title		:	Discussion on L1 measurement configuration

Document for		:	Discussion and Decision

1.	Introduction

In RAN2#121bis-e meeting, RAN2 made the following intital agreements about L1 measurement [1]:

- Initial agreements, from RAN2 point of view (may be dep on RAN1 progress). The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS. RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration). RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). Chair: the agreements above may need to be further evaluated, e.g. wrt subsequent LTM switches. RAN2 assumes that whether filtering, hysteresis, and time-to-trigger are needed for LTM specific L1 measurements is up to RAN1. FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status. Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

2.	Discussion

In RAN2#121bis-e meeting, the location of RS configuration for SSB-based measurements of candidate cells and the location of L1 reporting configuration are discussed and concluded as follows in terms of the current UE configuration:

The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS.

RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

From subsequent LTM perspective, the current UE configuration comes from candidate configurations, i.e., candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration and then the complete candidate configuration is applied and becoming the current UE configuration by LTM cell switch. So, RS configuration for SSB-based measurements of candidate cells and L1 reporting configuration are also included in candidate configurations (i.e. reference or candidate delta configuration) in order to support subsequent LTM.

Whether RS configuration is included in the reference or candidate delta configuration

Even if the serving cell changes due to LTM cell switch, the measurement RS for the remaining candidate cells may not change. In other words, RS configuration of candidate cells is independent on serving cell change by LTM cell switch. Therefore, it makes sense that RS configuration of candidate cells is included in the reference configuration rather then each candidate delta configuration. If RS configuration is included in each candidate delta configuration, there may exist too many duplicated RS configurations across candidate delta configurations, which can result in signaling redundancy.

Consider the case that two candidate cells are configured for LTM cell switch (i.e., Cell#1 and #2) and the current serving cell is Cell#0. If RS configuration is included in each candidate delta configuration, then RS configuration is provided as described in Figure 1. Otherwise RS configuration is included in the reference configuration as described in Figure 2, signaling redundancy due to duplicated configurations can be reduced.

| Candidate delta config for Cell#0 { RS config set for LTM candidates { RS config [Cell#1] RS config [Cell#2] } … } | Candidate delta config for Cell#1 { RS config set for LTM candidates { RS config [Cell#0] RS config [Cell#2] } … } | Candidate delta config for Cell#2 { RS config set for LTM candidates { RS config [Cell#1] RS config [Cell#0] } … } |
| --- | --- | --- |

Figure 1. RS configuration is included in each candidate delta configuration. Duplicated RS configurations across candidate delta configurations are green-highlighted.

Reference config { RS config ser for LTM candidates { RS config [Cell#0] RS config [Cell#1] RS config [Cell#2] } … }

Figure 2. RS configuration is included in the reference configuration. Signaling redundancy due to duplicated configurations can be reduced.

Based on above, we propose that RS configuration of candidate cells is included in the reference configuration.

Proposal 1. For subsequent LTM without reconfiguration, RS configuration of candidate cells is included in the reference configuration.

Whether L1 reporting configuration is included in the reference or candidate delta configuration

Unlike RS configuration, L1 reporting configuration depends on serving cell change by LTM cell switch because, for example, when to report and UL resource for reporting are necessary to be configured for each candidate cell. This implies that there may be no duplicated configurations across L1 reporting configuration of candidate cells. So, there is little benefit to be gained by including the L1 reporting configuration in the reference configuration. Since RAN2 agreed that L1 reporting configuration is placed inside the ServingCellConfig of current UE configuration, it is straighforward that L1 reporting configuration of candidate cells is included in the candidate delta configuration of each candidate cell.

Proposal 2. For subsequent LTM without reconfiguration, L1 reporting configuration of candidate cells is included in the candidate delta configuration of each candidate cell.

3.	Conclusion

In this paper, we have made the following proposals:

Proposal 1. For subsequent LTM without reconfiguration, RS configuration of candidate cells is included in the reference configuration.

Proposal 2. For subsequent LTM without reconfiguration, L1 reporting configuration of candidate cells is included in the candidate delta configuration of each candidate cell.

4.	References

[1] Chair Notes, RAN2 Chairman (MediaTek), 3GPP TSG RAN2 Meeting #121bis-e, 17 – 26 April, 2023