3GPP TSG-RAN WG2 Meeting #122	R2-2305273

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item		:	7.13.3

Source		:	LG Electronics Inc.

Title		:	Discussion on MDT override protection

Document for		:	Discussion and Decision

1.	Introduction

In this paper, we discuss cross-RAT signaling for MDT override protection.

2.	Discussion

In RAN2#121 meeting, Solution 2 is chosen for further specification work, where override protection by cross-RAT signaling but no cross-RAT reporting of LTE logged MDT report from NR to LTE. RAN2 made the following agreement for Solution 2:

Agreements For solution 2: - Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information - NR signaling is needed to inform the gNB that signaling based MDT is configured by E-UTRA. - Try to reuse R17 NR signaling by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

In our view, based on above agreement, Rel-18 MDT override protection procedure would be similarly designed with Rel-17 MDT override protection procedure. In Rel-17 MDT override protection procedure, during RRC connection (re)establishment/ RRC reconfiguration/ RRC connection procedure, the UE set sigLogMeasConfigAvailable to true or false in the corresponding RRC complete message if the UE has logged measurements available for NR and the logged measurements is signaling based MDT configured by NR. That is, the inclusion of sigLogMeasConfigAvailable implies the UE is configured with signaling based MDT by NR and the value of sigLogMeasConfigAvailable (i.e. true or false) indicates whether T330 timer is running or not. Another available indicator (i.e. logMeasAvailable) indicates whether the UE stores logged measurement or not. Possible cases from the combination of sigLogMeasConfigAvailable and logMeasAvailable are summarized as follows:

| sigLogMeasConfigAvailable | logMeasAvailable | Interpretation |
| --- | --- | --- |
| True | included in RRC complete message | - UE has logged measurements. - UE is configured with signaling based MDT by NR. - UE will perform measurements of signaling based MDT configured by NR when UE goes into IDLE mode. |
| True | not included in RRC complete message | - UE does not have logged measurements. - UE is configured with signaling based MDT by NR. - UE will perform measurements of signaling based MDT configured by NR when UE goes into IDLE mode. |
| False | included in RRC complete message | - UE has logged measurements available for NR. - UE is configured with signaling based MDT by NR. - Because of UE memory outage, UE will not perform measurements of signaling based MDT configured by NR when UE goes into IDLE mode. |
| False | not included in RRC complete message | Erroneous case! logMeasAvailable should be included if sigLogMeasConfigAvailable is set to false. |
| not included in RRC complete message | included in RRC complete message | - UE has logged measurements. - UE is not configured with signaling based MDT by NR. |
| not included in RRC complete message | not included in RRC complete message | - UE does not have logged measurements. - UE is not configured with signaling based MDT by NR. |

It is simple to follow the legacy principle for designing Rel-18 MDT override protection. Based on Agreement 1, LTE LoggedMeasurementConfiguration is extended to include sigLoggedMeasType in order to indicate signaling based MDT is configured by LTE (i.e. E-UTRA). Then, the UE follows the legacy procedure for MDT override protection during inter-RAT handover from LTE to NR (i.e. RRCReconfigurationComplete to gNB).

Proposal 1. LTE LoggedMeasurementConfiguration is extended to include sigLoggedMeasType in order to indicate signaling based MDT is configured by E-UTRA.

During inter-RAT handover from LTE to NR, the UE includes sigLogMeasConfigAvailable with the value of true or false in the RRCReconfigurationComplete message if the UE is configured with signaling based MDT by E-UTRA and T330 is not expired (i.e. T330 is running or stops). If sigLogMeasConfigAvailable-r17 is reused for cross-RAT signaling to prevent MDT override, the NR network cannot distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA without negotiating core network. Consequently, the NR network may initiate cross-RAT reporting by initiating the UE information procedure.

Observation 1. The reuse of sigLogMeasConfigAvailable-r17 IE and Rel-17 MDT override protection procedure can result in cross-RAT reporting (i.e. the UE reports logged measurements of signaling based MDT configured by E-UTRA to gNB), which violates the agreement that Solution 2 is chosen for further specification work.

To avoid the violation, the following approaches can be considered:

Approach 1: Reuse sigLogMeasConfigAvailable-r17 IE (i.e. no new IE addition). The setting of sigLogMeasConfigAvailable-r17 follows Rel-17 MDT override protection procedure. If the gNB receives the RRCReconfigurationComplete message including sigLogMeasConfigAvailable-r17 IE, the gNB should negotiate with core network in order to check whether the signaling based MDT set on the UE is configured by NR or E-UTRA

Approach 2: Define new IE indicating the UE is configured with signaling based MDT by E-UTRA (e.g., EUTRA-sigLogMeasConfigAvailable-r18 IE). The setting of EUTRA-sigLogMeasConfigAvailable-r18 follows Rel-17 MDT override protection procedure. The gNB can distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA upon receiving the RRCReconfigurationComplete message.

Approach 3: Reuse sigLogMeasConfigAvailable-r17 IE and define new IE indicating signaling based MDT is configured by E-UTRA (e.g., EUTRA-sigBasedMDT-r18). The setting of sigLogMeasConfigAvailable-r17 follows Rel-17 MDT override protection procedure, and EUTRA-sigBasedMDT-r18 is included in the RRCReconfigurationComplete message only if signaling based MDT is configured by E-UTRA. The gNB can distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA upon receiving the RRCReconfigurationComplete message.

Proposal 2. RAN2 consider the following approaches for cross-RAT signaling for MDT override protection:

- Approach 1: Reuse sigLogMeasConfigAvailable-r17 IE (i.e. no new IE addition). The setting of sigLogMeasConfigAvailable-r17 follows Rel-17 MDT override protection procedure. If the gNB receives the RRCReconfigurationComplete message including sigLogMeasConfigAvailable-r17 IE, the gNB should negotiate with core network in order to check whether the signaling based MDT set on the UE is configured by NR or E-UTRA

- Approach 2: Define new IE indicating the UE is configured with signaling based MDT by E-UTRA (e.g., EUTRA-sigLogMeasConfigAvailable-r18 IE). The setting of EUTRA-sigLogMeasConfigAvailable-r18 follows Rel-17 MDT override protection procedure. The gNB can distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA upon receiving the RRCReconfigurationComplete message.

- Approach 3: Reuse sigLogMeasConfigAvailable-r17 IE and define new IE indicating signaling based MDT is configured by E-UTRA (e.g., EUTRA-sigBasedMDT-r18). The setting of sigLogMeasConfigAvailable-r17 follows Rel-17 MDT override protection procedure, and EUTRA-sigBasedMDT-r18 is included in the RRCReconfigurationComplete message only if signaling based MDT is configured by E-UTRA. The gNB can distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA upon receiving the RRCReconfigurationComplete message.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Proposal 1. LTE LoggedMeasurementConfiguration is extended to include sigLoggedMeasType in order to indicate signaling based MDT is configured by E-UTRA.

Observation 1. The reuse of sigLogMeasConfigAvailable-r17 IE and Rel-17 MDT override protection procedure can result in cross-RAT reporting (i.e. the UE reports logged measurements of signaling based MDT configured by E-UTRA to gNB), which violates the agreement that Solution 2 is chosen for further specification work.

Proposal 2. RAN2 consider the following approaches for cross-RAT signaling for MDT override protection:

- Approach 1: Reuse sigLogMeasConfigAvailable-r17 IE (i.e. no new IE addition). The setting of sigLogMeasConfigAvailable-r17 follows Rel-17 MDT override protection procedure. If the gNB receives the RRCReconfigurationComplete message including sigLogMeasConfigAvailable-r17 IE, the gNB should negotiate with core network in order to check whether the signaling based MDT set on the UE is configured by NR or E-UTRA

- Approach 2: Define new IE indicating the UE is configured with signaling based MDT by E-UTRA (e.g., EUTRA-sigLogMeasConfigAvailable-r18 IE). The setting of EUTRA-sigLogMeasConfigAvailable-r18 follows Rel-17 MDT override protection procedure. The gNB can distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA upon receiving the RRCReconfigurationComplete message.

- Approach 3: Reuse sigLogMeasConfigAvailable-r17 IE and define new IE indicating signaling based MDT is configured by E-UTRA (e.g., EUTRA-sigBasedMDT-r18). The setting of sigLogMeasConfigAvailable-r17 follows Rel-17 MDT override protection procedure, and EUTRA-sigBasedMDT-r18 is included in the RRCReconfigurationComplete message only if signaling based MDT is configured by E-UTRA. The gNB can distinguish whether the signaling based MDT set on the UE is configured by NR or E-UTRA upon receiving the RRCReconfigurationComplete message.

4.	References