3GPP TSG-RAN WG2 Meeting#122                                                         R2-2305276

Incheon, KR, 22nd – 26th May, 2023

Source:              CATT

Title:	             Consideration on the time length for DRX timers

Agenda Item:	6.7.2

Document for:	Discussion and Decision

Introduction

In this contribution, we mainly focus on the postponed issue on the timer length definitions for drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL when sidelink configured grant Type 1 is applied.

Discussion

Based on the current spec, the timer lengths for drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL are defined as below in section 6.3.2 of TS 38.331.

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL Value in number of symbols of the BWP where the PDCCH was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL Value in number of slot lengths of the BWP where the PDCCH was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

It can be seen that the reference BWP of above two timers is not clear for the case of sidelink configured grant Type 1, because the sidelink configured grant Type 1 is configured by the RRC message and there is no PDCCH, the RRC message may be occasionally transmitted in several slots on different carriers if the RRC message is segmented, it is not clear which PDCCH will be applied. So, some companies proposed to give a clear description on the definitions of the BWP, two options are proposed as follows:

- Option 1: the BWP refers to active DL BWP.

- Option 2: referring to the DL BWP on which the PDCCH transmission( maybe the first PDCCH transmission if RRC message segment is considered) indicating the PDSCH carrying the RRCReconfiguration containing sidelink configured grant Type 1

For above options, option1 looks simple, but option 2 apply a unified rule for both sidelink configured grant Type 1 and other cases, which are all associated to the BWP of PDCCH, this PDCCH is the PDCCH used to activate the SL configure grant Type 2, to indicate SL grant, or to schedule   RRC message for sidelink configured grant Type 1.

Besides, as in Annex, regarding to Uu DRX timers drx-HARQ-RTT-TimerUL/DL and drx-RetransmissionTimerUL/DL, the timer length is associated to the BWP where the transport block was transmitted, so the BWP referring to the BWP where the transport block was transmitted could be a feasible option for sidelink configured grant Type 1 and also align to the Uu rule.

In our view, according to the history discussion, some companies proposed the network could solve the issue by receiving/transmitting the HARQ feedback information/data for different BWP selection, and there is no outstanding one amongst the above feasible options, so we prefer to leave it as UE implementation, and a note is preferred to be added to clarify this case.

Proposal 1:	A note is added to clarify that it is UE implementation to derive the symbol length for the drx-HARQ-RTT-TimerSL timer and the slot length for the drx-RetransmissionTimerSL timer for SL configured grant Type 1 case.

The corresponding CR is in [2].

Conclusion

According to the analysis in section 2, we have the following proposal:

Proposal 1:	A note is added to clarify that it is UE implementation to derive the symbol length for the drx-HARQ-RTT-TimerSL timer and the slot length for the drx-RetransmissionTimerSL timer for SL configured grant Type 1 case.

Reference

- Report from session on NR SL for RAN2#121bis-e, OPPO.

- R2-2305277 Correction on the time lengh for DRX timers.

Annex

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received. drx-HARQ-RTT-TimerDL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerDL (without suffix) for SCS 480 kHz and 960 kHz.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted. drx-HARQ-RTT-TimerUL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerUL (without suffix) for SCS 480 kHz and 960 kHz.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimerDL Value in number of slot lengths of the BWP where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.