3GPP TSG-RAN WG2 Meeting #122                                                        R2-2305279

Incheon, Korea, 22nd – 26th May, 2023

Source:              CATT

Title:	             Discussion on U2U Relay

Agenda Item:	7.9.2

Document for:	Discussion and Decision

Introduction

In the RAN2#121bis-e meeting, the U2U relay was discussed and reached the below agreements:

Agreements:

For the integrated-discovery case, the relay UE forwards the discovery message for DCR message with integrated Discovery case only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.  FFS if there is separate impact for this agreement from the relay selection functionality.

Both mode-1 and mode-2 resource allocation can be supported on both remote UE and relay UE in U2U relay case.  No impact to legacy resource allocation procedures is expected.

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.  This does not imply support of all PC5-RRC procedures between the remote UEs.

Each remote UE (source or destination) can trigger relay selection based on the direct link quality.  FFS interaction between discovery and selection.

Postpone replying to the SA2 LS on authorization.

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID and egress RLC Channel for a particular pair between source remote UE and target remote UE.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS  how configurations for e2e SL-SRBs are supported.

WA: E2E bearer ID (i.e., configuration index in the list of SLRB configurations) is used as input for the L2 U2U relay ciphering and deciphering at PDCP.

LS to SA3 to confirm the feasibility of using the configuration index.

In the current contribution, the below two aspects are further discussed:

- Leftover issues of U2U relay discovery and (re)selection;

- General control plane aspects for U2U relay.

Discussion

Leftover issues of U2U relay discovery and (re)selection

In this section, we focus on the key leftover issues of U2U relay discovery and (re)selection.

    - Conditions for U2U relay discovery message transmission

In RAN2#120 meeting, whether to introduce AS condition for U2U relay discovery message transmission was summarized but not discussed due to time limitation. In this section, this issue will be analyzed by scenario:

- Scenario#1: The sidelink U2U relay discovery for remote UEs without PC5 connection;

- Scenario#2: The sidelink U2U relay discovery for remote UEs with direct PC5;

- Scenario#3: The sidelink U2U relay discovery for remote UEs with indirect PC5 connection.

In the online discussion at RAN2#121bis-e meeting, companies raised that the RSRP threshold limitation for model A/B discovery may only apply to the remote UEs, following this approach, the below description is from the perspective of remote UEs:

For Scenario#1, it is SA2 scope to deal with the triggering of relay discovery for initial discovery procedure, hence there is no AS layer condition needed.

For Scenario#2, in Rel-17 U2N relay, as captured in TS 38.300, the remote UE evaluate Uu quality to decide whether it can transmit the relay discovery message.

For U2U relay, similar mechanism can be introduced in order to avoid unnecessary relay discovery message transmission. If the U2U remote UE does not support dual PC5 connectivity (e.g., both direct PC5 link and U2U relay discovery message transmission), remote UE can only transmit discovery message when the SL-RSRP/SD-RSRP of the current direct PC5 link between a pair of directly connected remote UEs is below a threshold.

Proposal 1: Remote UE can trigger U2U relay discovery message transmission when SL-RSRP/SD-RSRP of the current direct PC5 link is below a threshold.

For Scenario#3, the remote UEs already had one U2U relay connection, and due to some reason (e.g., one hop of current relay link became worse), the remote UE should reselect another relay UE. Regarding to this scenario, in the previous RAN2 meeting, the below agreement was agreed:

UE-to-UE relay reselection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) between a remote UE and the relay UE falling below a threshold.  FFS which remote UE (or both) can trigger relay reselection.  FFS if/how the second hop between the relay UE and the peer UE is considered.

The above agreement is related to the triggers of relay reselection. The left issue is whether to introduce separate threshold for U2U relay discovery.

- Option 1: Relay discovery is triggered by relay reselection. I.e., no separate threshold is configured to trigger relay discovery.

- Option 2: Before relay reselection, relay discovery can be triggered in advance in order to detect the candidate relay UEs.

Option1 is more suitable if the U2U relay service can tolerate longer interruption time, while option 2 can reduce the interruption time. In fact, even the service continuity is not requested for U2U relay, according to the current PDB’s requirement, using option2 can achieve better service effectiveness, besides considering the AS layer condition for U2N relay discovery had already been captured, it had better to inherit this rule in Rel-18 U2U relay. If option 2 is adopted, the PC5 RSRP threshold for triggering the sidelink relay discovery should be higher than the PC5 threshold for triggering the relay reselection.

Proposal 2: U2U relay discovery can be triggered when the SL-RSRP/SD-RSRP between a remote UE and the current serving relay UE is below a threshold.

Proposal 3: Two different thresholds can be configured for triggering U2U relay discovery message transmission and U2U relay (re-)selection. The threshold used for triggering the U2U relay discovery message transmission can be higher than the threshold used for triggering the U2U relay (re)selection.

    - Remote UE behavior when there are multiple suitable candidates U2U relay UEs which meet both the AS-layer and high layer criteria

In RAN2#121 meeting, RAN2 reached the following agreement:

For relay UE selection, the remote UE uses SL-RSRP measurements towards peer remote UE to trigger relay UE selection when there is data transmission on direct link.

For relay UE reselection, the remote UE uses SL-RSRP measurements towards the relay UE to trigger relay UE reselection when there is data transmission on the indirect link.

In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

Further, in case there is more than one candidate U2U relay UEs are detected, which one is chosen can be left to remote UE implementation following the same principle in Rel-17 U2N relay.

Proposal 4: If multiple suitable candidates U2U relay UEs which meet both the AS-layer and higher layer criteria are available, it is up to remote UE implementation to choose which U2U relay UE.

Control plane for U2U relay

    - SRAP Header

The Rel-17 U2N relay’s SRAP function includes bearer mapping and remote UE identification. In the TR 38.836[1], it also records that for U2U relay, the SRAP function includes bearer mapping and remote UE identification. For U2N relay, the mapping information is configured by the gNB for relay UE and remote UE, the remote UE identification is included in the adaptation layer header for further identification. The difference between the U2N relay uplink and U2N relay downlink is the certainty of the destination. For U2N relay uplink, the destination is relay UE’s serving gNB without doubt, there is unnecessary to use the destination information in relay UE. But for U2N relay downlink, relay UE needs the destination information (remote UE identification) to achieve the correct mapping. When it comes to U2U relay, end-to-end RB ID and UE ID is also needed in the adaptation layer header especially for multi-hop cases. Furthermore, the destination L2 ID of target remote UE and the source L2 ID of source remote UE are all needed.

In the previous meeting, the below FFS was added:

FFS whether both UE IDs are included in the header or the relay UE does a mapping.

If source remote UE L2 ID and target remote UE L2 ID are all included in SRAP header, the SRAP header overhead will be high. In order to achieve the purpose of saving the overhead of the adaptation layer header, a mapping from the pair of source remote UE L2 ID and target remote UE L2 ID to a shorter link identifier is needed.

Proposal 5: For U2U relay, PC5 adaptation layer header should include: source remote UE L2 ID, target remote UE L2 ID and BEARER ID. Considering the overhead, a mapping from the pair of source remote UE L2 ID and target remote UE L2 ID to a local shorter link identifier is needed in Rel-18 U2U relay.

Proposal 6: For Rel-18 U2U relay, relay UE is responsible for assignment of the local shorter link identifier.

    - QoS Split

For U2U relay QoS part, two questions will be discussed:

- Which QoS parameter(s) should be split between the two hops?

Refer to Rel-17 U2N relay, the end-to-end PDB parameter was split between the two hops. Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. With the introduction of U2U relay, the End-to-end link is split into two PC5 links, it is obvious the PDB should be split between the two PC5 links to ensure the overall requirement. Besides, Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). In order to meet the end-to-end PER requirement, the target PER of at least one of the PC5 should be lower than the end-to-end PER. That is to say, the PER should be split between the two PC5 links. More information from SA2, in the latest TR23.752, it records in the Solution#31 that PER is also needed to be split for U2U relay case. Hence, we propose that:

Proposal 7: The end-to-end PDB and PER parameters needs to be split between two PC5 links.

- Which UE is in charge of handing the QoS parameter split between the two hops?

For U2U relay, considering the connection is initiated by the source remote UE, it is nature that the source remote UE is in charge of handling end-to-end QoS to hop-by-hop QoS for U2U relay. On the other hand, if the relay UE is in charge of  splitting the end-to-end QoS to hop-by-hop QoS, then the end remote UE needs to transfer the end-to-end QoS to relay UE. Considering the relay UE is just one router, it is suitable to transfer the end-to-end QoS to relay UE. Additionally, this will increase the unnecessary signaling overhead. Hence, we propose that remote UE is in charge of splitting the end-to-end QoS to hop-by-hop QoS for U2U relay.

Proposal 8: The remote UE is in charge of splitting the end-to-end QoS to hop-by-hop QoS for U2U relay.

    - End-to-end SL-SRBs configuration

In the last RAN2 meeting, we reached the below agreements for U2U PC5-RRC connection establishment:

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.  This does not imply support of all PC5-RRC procedures between the remote UEs.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

Currently, the specified configuration for SL SRB is utilized for direct communication in PC5. As for U2U relay, the existing mechanism can be considered. Besides, to enable the exchange of E2E PC5-S signaling (for DCR/DCA and SMC messages) following per-hop link establishment, the AS-layer settings for the corresponding SL-SRBs need to be prepared beforehand. It seems reasonable to adopt the traditional approach by utilizing the specified configuration for PDCP/RLC/MAC. Regarding the SRAP configuration, since we have agreed that SRAP is also present for the SL SRBs, the same principle applies, meaning the specified configuration for SRAP can be employed for the SL SRBs. For Rel-17 U2N relay, the legacy SRAP configuration consists of local ID and RB-to-PC5 mapping information. But for Rel-18 U2U relay, there are two PC5 hops which are different to U2N relay, it is nature to introduce one new SRAP configuration, especially the end-to-end UE pair identification which consists of source and destination remote UE IDs should be considered as baseline. Furthermore, the E2E RB-to-PC5 hop mapping should also be extended to two PC5 hops.  In Rel-17 U2N relay, for the delivery of remote UE’s SRB0 RRC message, specified (fixed) configuration is used for the configuration of PC5 RLC channel. For the delivery of remote UE’s SRB1 RRC message such as RRCResume and RRCReestablishment message, default configuration is used for the configuration of PC5 RLC channel which can be reconfigured by network. Since RAN2 decided that explicit end-to-end PC5 RRC establishment is not needed, then in order to transfer the E2E PC5-S messages for PC5-S connection establishment and the first E2E PC5-RRC signalling, all the related E2E SL-SRBs(SL-SRB0,1,2,3) should be specified, then the following end-to-end DRBs can be further configured based on E2E SL-SRB3.

Proposal 9: Using the specified configuration for E2E SL-SRBs.

Proposal 10: In order to support the specified configuration for E2E SL-SRBs, the mapping between the two PC5 RLC channels used for the E2E SL-SRBs should be specified.

Proposal 11: In order to support the specified configuration for E2E SL-SRBs, new per-hop SL-RLCs (e.g. SL-RLC3/4/5/6) should be introduced.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Remote UE can trigger U2U relay discovery message transmission when SL-RSRP/SD-RSRP of the current direct PC5 link is below a threshold.

Proposal 2: U2U relay discovery can be triggered when the SL-RSRP/SD-RSRP between a remote UE and the current serving relay UE is below a threshold.

Proposal 3: Two different thresholds can be configured for triggering U2U relay discovery message transmission and U2U relay (re-)selection. The threshold used for triggering the U2U relay discovery message transmission can be higher than the threshold used for triggering the U2U relay (re)selection.

Proposal 4: If multiple suitable candidates U2U relay UEs which meet both the AS-layer and higher layer criteria are available, it is up to remote UE implementation to choose which U2U relay UE.

Proposal 5: For U2U relay, PC5 adaptation layer header should include: source remote UE L2 ID, target remote UE L2 ID and BEARER ID. Considering the overhead, a mapping from the pair of source remote UE L2 ID and target remote UE L2 ID to a local shorter link identifier is needed in Rel-18 U2U relay.

Proposal 6: For Rel-18 U2U relay, relay UE is responsible for assignment of the local shorter link identifier.

Proposal 7: The end-to-end PDB and PER parameters needs to be split between two PC5 links.

Proposal 8: The remote UE is in charge of splitting the end-to-end QoS to hop-by-hop QoS for U2U relay.

Proposal 9: Using the specified configuration for E2E SL-SRBs.

Proposal 10: In order to support the specified configuration for E2E SL-SRBs, the mapping between the two PC5 RLC channels used for the E2E SL-SRBs should be specified.

Proposal 11: In order to support the specified configuration for E2E SL-SRBs, new per-hop SL-RLCs (e.g. SL-RLC3/4/5/6) should be introduced.

Reference

- TR 38.836, Study on NR sidelink relay

- R2-2301933 Reply LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U

- R1-2304211  Reply LS on comparison of SL-RSRP and SD-RSRP measurements