3GPP TSG-RAN WG2 Meeting #122                                                        R2-2305281

Incheon, Korea, 22 – 26 May 2023

Source:	CATT

Title:	Discussion on Multi-path Scenario 1

Agenda Item:	7.9.4‎

Document for:	Discussion and Decision

Introduction

In RAN2#121bis-e meeting, some basic agreements of multi-path were made.

In this contribution, we further discuss the leftover issues of multi-path for scenario 1 and give our preference.

Discussion

  - Cell Group for MP

In RAN3#119 meeting, inter-DU case was agreed to be considered for MP relay.

RAN3 to consider inter-DU case for MP transmission. RAN3 to confirm the following WA as agreements: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

RAN2 had agreed that the PCell only on the direct path when a remote UE is configured with multi-paths. Thus, the direct path should be always configured in MCG of the remote UE when the remote UE is configured with multi-path.

Proposal 1: The direct path of the multi-path is always configured in MCG of the remote UE.

For cell group of indirect path, there are two cases as follows:

- Case 1: The serving cell of indirect path and direct path are in same DU.

- Case 2: The serving cell of indirect path and direct path are in different DUs.

For case 1, the indirect path should also be configured in MCG of the remote UE. For case 2, indirect path can be configured in SCG of the remote UE.

Proposal 2: The indirect path of the multi-path is configured in MCG of the remote UE when both indirect path and direct path are in same gNB-DU.

Proposal 3: The indirect path of the multi-path can be configured in SCG of the remote UE when indirect path and direct path are in different gNB-DUs.

In addition, MAC entities are different for case 1 and case 2. For case 1, one MAC entity is used for both indirect path and direct path. For case 2, two MAC entities are used for indirect path and direct path, i.e., one MAC entity for direct path and the other MAC entity for indirect path.

Proposal 4: One MAC entity is used for both indirect path and direct path when both paths are in same gNB-DU.

Proposal 5: Two MAC entities are used for indirect path and direct path when the paths are in different gNB-DU, i.e., one MAC entity for direct path and the other MAC entity for indirect path.

If two MAC entities are used for multi path, the agreement made in RAN2#120 meeting should be reviewed:

Proposal 3 [Easy] In principle, Mode 1 RA can be supported for the remote UE configured with multi-path in Scenario 1.

The mode 1 SL resource can’t be scheduled cross different MAC entities. Even if indirect path is in another DU, remote UE can request SL grant from DU of direct path. And DU of direct path can schedule the mode 1 SL grant to the remote UE directly which is similar to Rel-16 V2X SL mode 1 scheme.

Proposal 6: The remote UE configured with multi-path requests Mode 1 RA via direct path, DU of direct path schedules the SL mode 1 grant to the remote UE for both intra-DU and inter-DU multi path cases. From RAN2’s perspective, no enhancement is needed.

  - Signaling procedure of MP

In the SI stage, RAN2 has agreed that the following use cases are supported in Rel-18.

- A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

- B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

- C.	The remote UE operating in multi-path releases the indirect path;

- D.	The remote UE operating in multi-path releases the direct path;

- E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB;

- G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

In the following, we will discuss the detailed signaling procedures for each case:

For Case A, the signaling procedure is in Figure-1.

Figure-1 Addition of indirect path on top of direct path (Case A)

Based on Figure-1, the detailed steps are as below:

1.	The remote UE reports one or multiple candidate relay UE(s) to gNB via direct path.

2.	The gNB decides to add the indirect path to remote UE and selects the relay UE.

3. 	The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message can include at least relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated indirect/split bearer configuration. For the indirect bearer, it may reuse the SL-SRAP-Config for bearer mapping between remote UE’s RB and PC5 RLC channel.

4.	The remote UE establishes PC5 connection with the relay UE.

5.	If the relay UE is in RRC_IDLE/RRC_INACTIVE state, the remote UE will trigger it enter RRC_CONNECTED state.

6.	If the relay UE is in RRC_IDLE/RRC_INACTIVE state, it performs RRC setup/resume procedure.

7.	The remote UE sends the RRCReconfigurationComplete message to the gNB.

8.	The gNB sends an RRCReconfiguration message to the relay UE, which can include at least remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

9.	The relay UE sends the RRCReconfigurationComplete message to the gNB.

Step 8 and 9 could precede or overlap with step 7. If the target relay UE is in RRC_CONNECTED state, step 8 and 9 could be at any time after step 2.

10.	The data traffic from the indirect bearer or split bearer may be delivered via indirect path between the remote UE and the gNB.

Proposal 7: For Case A of multi-path Scenario 1, capture the indirect path addition signaling procedure in Figure-1 in TS 38.300.

For Case B, the signaling procedure is clear based on the following agreements made in RAN2#121 meeting:

Agreements: As a baseline, direct path addition for multi-path is a path switch procedure in which the target configuration contains both paths. Upon direct path addition for multi-path, one of the serving cells of the added direct path is configured as PCell for the remote UE.

For Case C and D, regarding to the signaling flow, the following open issues need to be discussed:

-	Issue 1: Whether RB reconfiguration from the indirect/direct path to the direct/indirect path is needed?

-	Issue 2: Whether PDCP data recovery for the remote UE’s AM DRBs is needed?

Regarding to Issue 1, for Case C and D, when indirect or direct path is released, the RB configured on the released path should be reconfigured from the released path to another path in order to avoid the service interruption.

Regarding to Issue 2, for AM DRBs, PDCP data recovery is performed to retransmit all the PDCP Data PDUs previously submitted to the re-established or released AM RLC entities but they are successful delivered. For Case C and D, PDCP data recovery can be reused in order to ensure the service continuity. The same mechanism can also be applied to Case E and G.

Proposal 8: For Case C and D of multi-path Scenario 1, when indirect or direct path is removed, the RB reconfiguration may be performed from the indirect/direct path to the direct/indirect path.

Proposal 9: For Case C, D, E and G of multi-path Scenario 1, PDCP data recovery can be reused for the remote UE’s AM DRBs.

For Case E, it was already agreed to support a single procedure for this case:

Agreement: Change of direct path while keeping the indirect path can be done with a release-and-add in a single RRC message. This does not exclude a gNB implementation from using separate release and add procedures instead.

For Case G, it was agreed to support a single procedure:

Agreement: Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message. This does not exclude a gNB implementation from using separate release and add procedures instead.

The signaling procedure of case G is in Figure 2.

Figure-2 Change to a new relay UE for the indirect path while keeping the direct path under the same gNB (Case G)

Based on Figure-2, the detailed steps are as below:

1.	The remote UE reports one or multiple candidate relay UE(s) to gNB.

2.	The gNB decides to change the indirect path of remote UE to target relay UE.

3. 	The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message can include at least target relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated indirect/split bearer configuration. For the indirect bearer, it may reuse the SL-SRAP-Config for bearer mapping between remote UE’s RB and PC5 RLC channel.

4.	The remote UE establishes PC5 connection with the target relay UE.

5.	If the target relay UE is in RRC_IDLE/RRC_INACTIVE state, the remote UE willtrigger it enter RRC_CONNECTED state.

6.	If the relay UE is in RRC_IDLE/RRC_INACTIVE state, it performs RRC setup/resume procedure.

7.	The remote UE sends the RRCReconfigurationComplete message to the gNB.

8.	The gNB sends an RRCReconfiguration message to the target relay UE, which can include at least remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

9.	The target relay UE sends the RRCReconfigurationComplete message to the gNB.

Step 8 and 9 could precede or overlap with step 7. If the target relay UE is in RRC_CONNECTED state, step 8 and 9 could be at any time after step 2.

10.	The gNB sends an RRCReconfiguration message to the source relay UE to release remote UE's configuration.

11.	The source relay UE sends the RRCReconfigurationComplete message to the gNB.

12.	The data traffic from the indirect bearer or split bearer may be delivered via indirect path between the remote UE and the gNB.

Proposal 10: For Case G of multi-path Scenario 1, capture the indirect path modification signaling procedure in Figure-2 in TS 38.300.

  - Details of Multi-path

Failure or HO report

In RAN2#121bis-e meeting, it was agreed that remote UE should inform Uu RLF of relay UE to gNB upon reception of Uu RLF indication from the relay UE when SRB1 is available on the direct path and not suspended:

Agreements: WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE. A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment. FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE. This agreement does not imply any conclusion on non-split SRB1 on indirect path. A remote UE in multipath that is released to RRC_IDLE/RRC_INACTIVE can apply legacy cell/relay selection behaviour, thus moving to single-path operation on either path according to implementation.

First, RAN2 can confirm the working assumption.

Proposal 11: RAN2 confirms the working assumption:

WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

Whether the remote UE needs to inform handover of relay UE and expiry of dataInactivityTimer of relay UE to gNB is FFS.

For handover of relay UE, handover command is come from gNB. Therefore, gNB can reconfigure or release the indirect path of the remote UE to avoid unexpected indirect path failure. Since inter-gNB multi path mobility is not supported in Rel-18, the gNB is expected to release the multipath configuration related to this relay at the remote UE before it reconfigures the relay UE to perform inter-gNB handover. For intra-gNB handover, the gNB is expected to reconfigure or release the indirect path related to this relay at the remote UE before it reconfigures the relay UE which can be up to gNB implementation.

Proposal 12: For Rel-18 multi-path, the gNB is expected to release the multipath configuration related to this relay at the remote UE before it reconfigures the relay UE to perform inter-gNB handover.

Proposal 13: For Rel-18 multi-path, the gNB is expected to reconfigure or release the multipath configuration related to this relay at the remote UE before it reconfigures the relay UE to perform intra-gNB handover.

For expiry of dataInactivityTimer of relay UE, it happens in the relay UE. The remote UE can aware it upon reception of relayUE-Uu-RRC-Failure indication from the relay UE. Therefore, the remote UE needs to inform this information to gNB if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.

Proposal 14: For Rel-18 multi-path, A remote UE in RRC_CONNECTED, upon reception of relayUE-Uu-RRC-Failure indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.

Furthermore, In RAN2#121 meeting, it was agreed that remote UE can report path failure to gNB upon detection of 3GPP-defined RLF failure in one path. Which messages are used to carry Uu-RLF, PC5-RLF and Uu-RLF/Uu RRC failure of relay UE is FFS.

Agreements: In case of Uu-RLF, at least for split SRB1, if SRB1 is available on indirect path not suspended, trigger report to network via indirect path to report the failure via a RRC message. Otherwise, RRC Re-establishment is initiated. RAN2 is requested to discuss whether the RRC message is the existing message e.g. MCGFailureInformation or a new message. In case of PC5-RLF, if SRB1 is available on direct path not suspended, trigger report to network via direct path to report the failure via a RRC message. FFS if an alternative case exists and what would be done in that case. FFS which message is used. The existing PC5-RRC Notification Message procedure is reused for the relay UE to inform the remote UE about Uu failure of the relay UE as currently specified in 38.331.

The signaling used for direct path failure and indirect path failure reporting are FFS. There are two options were proposed in company’s contributions in the last meeting：

- Option 1: Remote UE reports direct path failure via MCGFailureInformation message, and reports indirect path failure via SidelinkUEInformation message.

- Option 2: Remote UE reports direct path failure and indirect path failure via MCGFailureInformation or SCGFailureInformation message.

For option 1, SidelinkUEInformation message is not suitable to be used for remote UE reports indirect path failure since indirect path failure includes two cases: 1) PC5-RLF between remote UE and relay UE, and 2) Uu-RLF or Uu RRC failure of relay UE. SidelinkUEInformation message can be only used for case 1) since Uu-RLF or Uu RRC failure of relay UE is not related to sidelink.

In Rel-17 U2N relay, the relay UE sends SidelinkUEInformation message to gNB while PC5-RLF is detected. For the remote UE, it should not send SidelinkUEInformation message since there isn’t RRC connection when PC5 connection with the relay UE is RLF.

Observation 1: In Rel-17 U2N relay, the remote UE should not send SidelinkUEInformation message when PC5 connection with the relay UE is RLF since the RRC connection between the remote UE and gNB is failure.

Furthermore, UE shall report all sidelink information in the SidelinkUEInformation message when it is initiated no matter it is changed or not, the signaling overhead of option 1 is higher than option 2. Consideration on SidelinkUEInformation message can’t be used to report indirect path failure for scenario 2, MCGFailureInformation/SCGFailureInformation message can be used for both scenario 1 and scenario 2. Therefore, option 2 is preferred.

Proposal 15: For Rel-18 multi-path, remote UE reports direct path failure via MCGFailureInformation message and reports indirect path failure via MCGFailureInformation or SCGFailureInformation message based on the serving cell of relay UE is in MCG or SCG of remote UE.

In addition, new failure types of SL RLF, Uu RLF of relay UE and Uu RRC failure of relay UE need to be introduced in MCGFailureInformation and SCGFailureInformation message.

Proposal 16: Introducing new failure types (SL RLF, Uu RLF of relay UE, Uu RRC failure of relay UE) in MCGFailureInformation and SCGFailureInformation message.

Path activation/deactivation

In RAN2#121bis-e meeting, it was agreed to reuse primary path and primary RLC entity for MP split bearer.

Agreements: The concept of the ‘primary path and primary RLC entity’ is adopted for each MP split bearer configuration according to the existing definition.

For legacy CA, SCells are configured by gNB via RRC signaling, and can be activated or deactivated via MAC CE in order to achieve power saving benefit. However, for multi-path scenario, direct path and indirect path can be in the same cell. Hence, the cell activation/deactivation concept is not suitable any more. Therefore, it is needed to introduce path activation/deactivation mechanism. Since Uu MAC CE can’t be transmitted to the remote UE via the indirect path. Path activation/deactivation can be done via MAC CE over direct path.

Proposal 17: Path activation/deactivation is introduced for multi-path.

Proposal 18: Introduce Path Activation/Deactivation MAC CE over direct path to control the activation/deactivation of direct path and/or indirect path.

Duplication activation/deactivation

For legacy DC/CA, Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE are used to activate or deactivate duplication of DRBs. It can be reused for multi-path relay scenario.

Proposal 19: Reuse Duplication Activation/Deactivation MAC CE or Duplication RLC Activation/Deactivation MAC CE over direct path to control the duplication activation/deactivation of DRBs.

Whether non-split SRBs can be configured on the indirect path

In RAN2#121bis-e meeting, it was agreed that SRB is up to network implementation whether to configure SRB1 and SRB2 with same or different bearer types.

Agreements: The bearer type configuration is provided per SRB. It is up to network implementation whether to configure SRB1 and SRB2 with same or different bearer types (within the bearer types that are supported). FFS if there are cases where the configuration of non-split SRBs over indirect path is useful.

For the useful of non-split SRBs over indirect path, indirect path may be also in the PCell. Non-split SRB1 over indirect path doesn’t conflict with the Pcell on direct path. Furthermore, which path is more reliable depends on the channel quality. When the remote UE is located in the cell edge, indirect path may be more reliable than direct path.

Proposal 20: For multi-path scenario 1, non-split SRBs can be configured on the indirect path.

Trigger of the RRC_IDLE/RRC_INACTIVE target relay UE to establish/resume the RRC connection.

Similar as Rel-17 U2N relay, the serving relay UE of the remote UE configured with multi-path should be in RRC_CONNECTED. Additionally, when the remote UE performs path switch, the target relay UE can be in any RRC state. If the target relay UE is in RRC_IDLE /RRC_INACTIVE, it should enter RRC_CONNECTED triggered by the first RRC message received from the remote UE. In Rel-18 multi-path relay, the target relay UE in RRC_IDLE /RRC_INACTIVE can’t enter into RRC_CONNECTED if the SRB1 of the remote UE configured on direct path only.

In RAN2#121bis-e meeting, this issue was discussed, the agreements were listed below:

Agreements: When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1. When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition. - when primary RLC entity of split SRB1 is on direct path - when non-split SRB1 is configured on direct path When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2. When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2. - when primary RLC entity of split SRB1 is on direct path - when non-split SRB1 is configured on direct path

Besides split SRB1 with duplication, there are two configurations can be used for RRCReconfigurationComplete message via indirect path to trigger the idle/inactive relay UE to RRC_CONNECTED.

-	when primary RLC entity of split SRB1 is on indirect path;

-	when non-split SRB1 is configured on indirect path.

Proposal 21: When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the indirect path for Scenario 1.

-	when primary RLC entity of split SRB1 is on indirect path;

-	when non-split SRB1 is configured on indirect path.

For the agreement of “A PC5-RRC trigger is specified at least for other cases.”, since the solution is only used for triggering IDLE/INACTIVE relay UE to enter CONNECTED state, the remote UE doesn’t know relay UE’s RRC state. If PC5-RRC is used, to avoid redundancy PC5-RRC message, the remote UE should acquire the target relay UE’s RRC state from the relay UE or gNB. Furthermore, consideration on the compatibility, gNB should configure SRB1 on the indirect path if the target relay UE is a Rel-17 UE in IDLE/INACTIVE. Since gNB does not know the release of IDLE UE, even if UE release will be introduced in UE capability, remote UE should acquire the release of the relay UE and indicate it to gNB.

In summary, the additional issues should be discussed for triggering the idle/inactive relay UE to RRC_CONNECTED via PC5-RRC message:

- How to avoid the new trigger in PC5-RRC message cannot be interpreted by Rel-17 relay UE;

- How to avoid redundancy trigger message to CONNECTED relay UE.

Therefore, the additional enhancements should be considered:

- Remote UE should acquire the release of the relay UE and indicate it to gNB;

- Remote UE can acquire the target relay UE’s RRC state from the relay UE or gNB.

Proposal 22: RAN2 discuss the following enhancements for triggering the idle/inactive relay UE to RRC_CONNECTED via PC5-RRC message:

- Remote UE should acquire the release of the relay UE and indicate it to gNB;

- Remote UE should acquire the target relay UE’s RRC state from the relay UE or gNB.

T304/T420:

In the summary [2], it is proposed to discuss whether to reuse T304/T420 timer for MP configuration procedure or to introduce new timer(s) and what UE should do after timer expiry.

For indirect path addition/modification procedure, T420 can be reused.

| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |
| --- | --- | --- | --- |

Proposal 23: Reuse T420 timer for MP indirect path addition or indirect path modification configuration procedure.

However, the stop condition and UE behavior after expiry of T420 are not suitable for MP. Since RRCReconfigurationComplete may be sent on direct path, there isn’t RRCReconfigurationComplete message on the indirect path. There are 3 options for the MP remote UE stop T420:

- Option1: Upon successfully sending RRCReconfigurationComplete message on indirect path.

- Option2: Upon PC5 connection establishment successfully.

- Option3: Upon successfully receiving RRCReconfigurationCompleteSidelink message.

For option 1, it is reused the stop condition of T420 in Rel-17. Furthermore, the split SRB1 should be configured and the primary RLC entity should be reconfigured to the RLC entity of indirect path. Some field description may be needed to clarify the configuration.

For option 2, there is high risk of indirect path setup failure only based on PC5-S connection establishment successfully.

For option 3, since the remote UE shall send RRCReconfigurationSidelink message to the relay UE to establish PC5 Relay RLC channels when reception of the RRCReconfiguration message to add the indirect path. This message can be used to trigger the IDLE/INACTIVE relay UE enter RRC_CONNECTED state. Therefore, the remote UE can consider the indirect path is established successfully when it received RRCReconfigurationCompleteSidelink message.

Proposal 24: For Rel-18 multi path, the remote UE shall stop T420 upon successfully receiving RRCReconfigurationCompleteSidelink message.

For indirect path addition procedure, when T420 expired, indirect path addition failure. The remote UE shall continue using the configuration used prior to the reception of RRCReconfiguration message. For indirect path modification procedure, target indirect path establishment failure. The remote UE can continue using the direct path configuration before. But for the source indirect path, it may be released. Therefore, for both indirect path addition and modification procedure, the remote UE shall fallback to continue using the direct path configuration used prior to the reception of RRCReconfiguration message. Furthermore, the remote UE should inform network about the reconfiguration with sync failure.

Proposal 25: For Rel-18 multi path, the remote UE shall fallback to continue using the direct path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration with sync failure when T420 expired.

For direct path addition/modification procedure, T304 can be reused.

| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync for the MCG which does not include sl-PathSwitchConfig, or upon reception of RRCReconfiguration message including reconfigurationWithSync for the SCG not indicated as deactivated in the NR or E-UTRA message containing the RRCReconfiguration message or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| --- | --- | --- | --- |

Proposal 26: Reuse T304 timer for MP direct path addition or indirect path modification configuration procedure.

The stop condition of T304 can be reused for MP.

Proposal 27: For Rel-18 multi path, the remote UE shall stop T304 upon successful completion of random access on the corresponding SpCell.

For both direct path addition and modification procedure, the remote UE shall fallback to continue using the indirect path configuration used prior to the reception of RRCReconfiguration message. Furthermore, the remote UE should inform network about the reconfiguration with sync failure.

Proposal 28: For Rel-18 multi path, the remote UE shall fallback to continue using the indirect path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration with sync failure when T304 is expired.

Conclusion

According to the above discussion, the proposals on the above issue are as follows:

Proposal 1: The direct path of the multi-path is always configured in MCG of the remote UE.

Proposal 2: The indirect path of the multi-path is configured in MCG of the remote UE when both indirect path and direct path are in same gNB-DU.

Proposal 3: The indirect path of the multi-path can be configured in SCG of the remote UE when indirect path and direct path are in different gNB-DUs.

Proposal 4: One MAC entity is used for both indirect path and direct path when both paths are in same gNB-DU.

Proposal 5: Two MAC entities are used for indirect path and direct path when the paths are in different gNB-DU, i.e., one MAC entity for direct path and the other MAC entity for indirect path.

Proposal 6: The remote UE configured with multi-path requests Mode 1 RA via direct path, DU of direct path schedules the SL mode 1 grant to the remote UE for both intra-DU and inter-DU multi path cases. From RAN2’s perspective, no enhancement is needed.

Proposal 7: For Case A of multi-path Scenario 1, capture the indirect path addition signaling procedure in Figure-1 in TS 38.300.

Proposal 8: For Case C and D of multi-path Scenario 1, when indirect or direct path is removed, the RB reconfiguration may be performed from the indirect/direct path to the direct/indirect path.

Proposal 9: For Case C, D, E and G of multi-path Scenario 1, PDCP data recovery can be reused for the remote UE’s AM DRBs.

Proposal 10: For Case G of multi-path Scenario 1, capture the indirect path modification signaling procedure in Figure-2 in TS 38.300.

Proposal 11: RAN2 confirms the working assumption:

WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

Proposal 12: For Rel-18 multi-path, the gNB is expected to release the multipath configuration related to this relay at the remote UE before it reconfigures the relay UE to perform inter-gNB handover.

Proposal 13: For Rel-18 multi-path, the gNB is expected to reconfigure or release the multipath configuration related to this relay at the remote UE before it reconfigures the relay UE to perform intra-gNB handover.

Proposal 14: For Rel-18 multi-path, A remote UE in RRC_CONNECTED, upon reception of relayUE-Uu-RRC-Failure indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.

Observation 1: In Rel-17 U2N relay, the remote UE should not send SidelinkUEInformation message when PC5 connection with the relay UE is RLF since the RRC connection between the remote UE and gNB is failure.

Proposal 15: For Rel-18 multi-path, remote UE reports direct path failure via MCGFailureInformation message and reports indirect path failure via MCGFailureInformation or SCGFailureInformation message based on the serving cell of relay UE is in MCG or SCG of remote UE.

Proposal 16: Introducing new failure types (SL RLF, Uu RLF of relay UE, Uu RRC failure of relay UE) in MCGFailureInformation and SCGFailureInformation message.

Proposal 17: Path activation/deactivation is introduced for multi-path.

Proposal 18: Introduce Path Activation/Deactivation MAC CE over direct path to control the activation/deactivation of direct path and/or indirect path.

Proposal 19: Reuse Duplication Activation/Deactivation MAC CE or Duplication RLC Activation/Deactivation MAC CE over direct path to control the duplication activation/deactivation of DRBs.

Proposal 20: For multi-path scenario 1, non-split SRBs can be configured on the indirect path.

Proposal 21: When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the indirect path for Scenario 1.

-	when primary RLC entity of split SRB1 is on indirect path;

-	when non-split SRB1 is configured on indirect path.

Proposal 22: RAN2 discuss the following enhancements for triggering the idle/inactive relay UE to RRC_CONNECTED via PC5-RRC message:

- Remote UE should acquire the release of the relay UE and indicate it to gNB;

- Remote UE should acquire the target relay UE’s RRC state from the relay UE or gNB.

Proposal 23: Reuse T420 timer for MP indirect path addition or indirect path modification configuration procedure.

Proposal 24: For Rel-18 multi path, the remote UE shall stop T420 upon successfully receiving RRCReconfigurationCompleteSidelink message.

Proposal 25: For Rel-18 multi path, the remote UE shall fallback to continue using the direct path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration with sync failure when T420 expired.

Proposal 26: Reuse T304 timer for MP direct path addition or indirect path modification configuration procedure.

Proposal 27: For Rel-18 multi path, the remote UE shall stop T304 upon successful completion of random access on the corresponding SpCell.

Proposal 28: For Rel-18 multi path, the remote UE shall fallback to continue using the indirect path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration with sync failure when T304 is expired.

Reference

- Draft_R2_121bis_meeting_report_v1

- R2-2301925	[Pre121][407] Summary of AI 8.9.4 on Multi-path relaying	LG Electronics