3GPP TSG-RAN WG2 Meeting #122                                                          R2-2305282

Incheon, KR, 22nd – 26th May, 2023

Source:              CATT

Title:	             Leftover Issues on Multi-path Scenario 2

Agenda Item:	7.9.4

Document for:	Discussion and Decision

Introduction

In this contribution, we will further analyze the leftover open issues for multi-path scenario 2.

Discussion

Indirect path change (Case G)

In the study phase, the following cases were discussed for multi-path scenario 2:

- A. The remote UE configured only on the direct path adds the indirect path under the same gNB;

- B. The remote UE configured only on the indirect path adds the direct path under the same gNB;

- C. The remote UE configured with multi-path releases the indirect path;

- D. The remote UE configured with multi-path releases the direct path;

- E. The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

- F. The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

- G. The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

During the last RAN2 meetings, it has been agreed that Case A/C are supported, and Case B/D/E/F are not supported. The only remaining issue is whether Case G can be supported. In the RAN2#120 meeting, the following agreement was reached for Case G:

Agreements: For Scenario 2, whether to support Case G is discussed in normative phase, but RAN2 will not do additional work to enable it for Scenario 2 over Scenario 1.

In [1], there are some discussions on whether more than one relay UE can be supported for one remote UE in multi-path scenario 2. For scenario 2, the interface between relay UE and remote UE are ideal. Hence, they should be co-located or connected via fiber. In this case, there is no strong motivation to use more than one relay UE for a remote UE. Hence, it is unnecessary to support Case G.

Proposal 1: For scenario 2, more than one relay UE for a remote UE is not supported.

Proposal 2: For scenario 2, Case G (The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB) is not supported.

Protocol stack

For scenario 2, the following agreement and working assumption were reached in the RAN2#119bis-e meeting:

Agreements: Proposal 9A (modified): Do not specify adaptation layer over UE-to-UE link for scenario 2 in RAN2. Proposal 9B: Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

Based on the above, the following protocol stack can be used for scenario 2.

Figure-1 User plane protocol stack of multi-path (scenario 2)

Figure-2 Control plane protocol stack of multi-path (scenario 2)

Proposal 3: For scenario 2, take Figure-1 and Figure-2 as the baseline UP/CP protocol stack and capture them in TS38.300.

Relay connection setup procedure

In the RAN2#119-e meeting, it was agreed that the relation between remote UE and relay UE in scenario 2 is preconfigured or static and how the relation is pre-configured or static is out of the 3GPP scope. Hence, relay discovery procedure is not needed for scenario 2. Correspondingly, the relay selection/reselection is unnecessary for scenario 2.

Proposal 4: Relay discovery/selection/reselection procedure is not needed for scenario 2.

The relation between remote UE and relay UE in scenario 2 is preconfigured or static, but gNB does not know this relation. Hence, in order to establish the relay connection, how does the gNB aware this relation should be discussed. In RAN2#121bis-e meeting, the following agreement was reached:

Agreements: In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition. Need for reporting in the idle/inactive case can be further discussed. FFS what ID is used.

Based on the above agreement, there are still two issues needs further discussion:

- Issue 1: Whether relay UE in IDLE/INACTIVE should be considered?

- Issue 2: Which relay UE ID should be reported?

For Issue 1, as discussed in section 2.1, only Cases A/C are supported. There is no need to consider relay UE in IDLE/INACTIVE. How to trigger the relay UE enters RRC_CONNECTED state can be left to implementation.

For Issue 2, considering the interface between relay and remote UE is ideal, there is no PC5 L2 ID. Hence, it had better use the legacy UE identification, e.g., C-RNTI.

Proposal 5: For scenario 2, enhancements for relay UE in RRC_IDLE/INACTIVE state is not considered.

Proposal 6: For scenario 2, remote UE should report the relay UE’s C-RNTI to gNB for indirect path addition.

After the gNB knows the association between relay and remote UE, it can further configure the bearer mapping. In RAN2#121 meeting, it was agreed that:

Agreements: gNB provides bearer mapping information to relay UE through dedicated signalling.

But the detailed configuration procedure is still unclear. In our understanding, the most direct way is that gNB can configure the 1:1 mapping between Uu RLC channel and the end to end bearer based on the association between relay UE and remote UE. The whole signaling flow is shown in the following Figure-3.

Figure-3 Relay connection setup procedure

The description of main step in Figure-3 is as below:

- Step 1: The remote UE reports the relay UE ID to gNB.

- Step 2/4: Once gNB is aware of the relay UE, gNB should perform RRC reconfiguration to both remote UE and relay UE.

- For relay UE

- In the UL, gNB configures the 1:1 mapping between the E2E bearer and the Uu RLC channel to relay UE through RRC reconfiguration procedure to help the relay UE to perform UL data forwarding. Since the interface between relay UE and remote UE is ideal, when relay UE receives UL data, how it acquires which E2E bearer the data belongs to can be left to implementation.

- In the DL, since the interface between relay UE and remote UE is ideal, configuration from gNB is unnecessary.

- For remote UE

- In the UL, gNB configures the 1:1 mapping between E2E bearer and Uu RLC channel of relay UE through RRC reconfiguration procedure.

- In the DL, configuration from gNB is unnecessary.

Proposal 7: For UL, gNB needs to configure the 1:1 mapping between E2E bearer and the Uu RLC channel to remote UE and relay UE separately by RRC reconfiguration procedure.

PDCP duplication

Regarding to PDCP duplication, some agreements have been reached in the past RAN2 meetings. But there are still some open issues:

- Issue 1: How many legs can be supported for PDCP duplication in scenario 2?

- Issue 2: Whether PDCP duplication activation/deactivation can be supported?

For Issue 1, according to the current TS38.331, at most 4 legs can be supported for CA+DC duplication. For the multi-path scenario 2, there are three options:

- Option 1: Only two legs can be supported.

In this option, one leg is on the direct path and another leg is on the indirect path.

- Option 2: Three legs can be supported.

In this option, two legs on the direct path considering CA is support, and one leg on the indirect path.

- Option 3: Four legs can be supported.

In this option, two legs on the direct path and two legs on the indirect path considering both CA is supported both on the direct path and the PC5 interface of the indirect path.

In our understanding, considering SL CA is still under discussion, we can first assume only one leg is supported on the indirect path at this stage. For the direct path, it already supports CA. Therefore, two legs can be considered on the direct path. Based on the above analysis, Option 2 is slightly preferred.

Proposal 8: For scenario 2, three legs can be supported for PDCP duplication.

For Issue 2, it is obvious that PDCP duplication activation/deactivation should be supported. Similarly as Uu, MAC CE can be used for PDCP duplication activation/deactivation.

In Uu, there are two kinds of MAC CE: Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE. The Duplication Activation/Deactivation MAC CE is used to indicate whether PDCP duplication is activated or deactivated for each DRB. The Duplication RLC Activation/Deactivation MAC CE is used to indicate which whether each seondary RLC entities of one DRB is activated or deactivated. The two kinds of MAC CE formats are as below:

Figure-4 Duplication Activation/Deactivation MAC CE

Figure-5 Duplication RLC Activation/Deactivation MAC CE

The same MAC CE formats can be applied for multi-path. If the duplication leg for multi-path is less than the CA+DC case, some field can be ignored in the Duplication RLC Activation/Deactivation MAC CE.

Furthermore, considering MAC layer is hop by hop, hence, it had better restrict the MAC CEs can be only transmitted on the direct path.

Proposal 9: The legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE can be reused as baseline for multi-path scenario 2.

Proposal 10: The current Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE can only be transmitted on the direct path.

UE-UE link failure

Regarding to the UE-UE link failure, the following agreements were reached in the RAN2#121bis-e meeting:

Agreements: If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path. Details of the reporting mechanism can be further discussed.

Based on the above agreement, it is already agreed that remote UE can report UE-UE link failure to gNB. But it is still FFS on the following aspects:

- Issue 1: How does the remote UE detect the UE-UE link failure?

- Issue 2: How does the remote UE report the UE-UE link failure to gNB?

- Issue 3: Whether relay UE can also report UE-UE link failure to gNB?

For Issue 1, considering the interface between remote UE and relay UE is ideal, RLF is a rare case. If it happens, how to detect the UE-UE link failure can be based on remote UE implementation.

Proposal 11:  For scenario 2, the UE-UE link failure detection is based on remote UE implementation.

For Issue 2, there are two options for the UE-UE link failure reporting:

- Option 1: Introduce a new RRC signaling;

- Option 2: Reuse the legacy MCGFailureInformation or SCGFailureInformation.

For Option 1, a new RRC signaling, e.g., indirect path failure reporting message can be introduced. It will introduce additional specification effort. For Option 2, the legacy RRC message can be reused and a new failure type can be introduced, e.g., UE-UE link failure. In Option 2, whether MCGFailureInformation or SCGFailureInformation can be used depends on the serving cell of relay UE is in the MCG or SCG of the remote UE.

Proposal 12: For scenario 2, remote UE can report UE-UE link failure via MCGFailureInformation or SCGFailureInformation message, and which message is used depends on the serving cell of relay UE is in the MCG or SCG of the remote UE.

For issue 3, in [1], some companies proposed either the remote UE or the relay UE can report the UE-UE link failure to gNB. Considering it was already agreed that the remote UE can report the UE-UE link failure to gNB in the last RAN2 meeting, it should further discuss whether the relay UE can also report the UE-UE link failure to gNB. Assuming SRB1 is used to report the UE-UE link failure, the following two cases should be considered:

- Case 1: Split SRB is not configured;

- Case 2: Split SRB is configured.

In Case 1, it was agreed in the last RAN2 meeting non-split SRB1 over indirect path is not supported for scenario 2. Hence, relay UE reports UE-UE link failure is not supported in Case 1.

For Case 2, if PDCP duplication is not activated and data amount is lower than the threshold, it can only be transmitted through the primary path. Even if the data amount is higher than the threshold, it can be transmitted either through the primary path or secondary path. For scenario 2, considering it was agreed that the primary path of the split SRB1 is always on direct path. Hence, in most cases, relay UE report the UE-UE link failure is not possible.

Proposal 13: For scenario 2, relay UE reports the UE-UE link failure is not supported.

Resource allocation between relay and remote UE

In scenario 1, for remote UE, in the PC5 hop of UL direction, both mode 1 and mode 2 can be used in PC5 in case of multi-path is configured since there is direct path.

In scenario 2, different from scenario 1, the interface between remote UE and relay UE is ideal. Hence, how to perform the UL/DL transmission should be considered:

- For UL, the straight forward method is that for the direct link, the UL data is scheduled by gNB, and for the indirect link, the UL resource of Uu link of relay UE is scheduled by gNB, once get the UL grant, relay UE can transmit the data already stored in it if any or relay UE can get data from remote UE.

- For DL, similarly, relay UE can receive data from network and how to send the data to remote UE depends on relay UE implementation.

Proposal 14: For scenario 2, UL/DL transmission of relay UE in Uu link are both scheduled by network and how to perform the data transmission between relay UE and remote UE depends on relay/remote UE implementation.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: For scenario 2, more than one relay UE for a remote UE is not supported.

Proposal 2: For scenario 2, Case G (The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB) is not supported.

Proposal 3: For scenario 2, take Figure-1 and Figure-2 as the baseline UP/CP protocol stack and capture them in TS38.300.

Proposal 4: Relay discovery/selection/reselection procedure is not needed for scenario 2.

Proposal 5: For scenario 2, enhancements for relay UE in RRC_IDLE/INACTIVE state is not considered.

Proposal 6: For scenario 2, remote UE should report the relay UE’s C-RNTI to gNB for indirect path addition.

Proposal 7: For UL, gNB needs to configure the 1:1 mapping between E2E bearer and the Uu RLC channel to remote UE and relay UE separately by RRC reconfiguration procedure.

Proposal 8: For scenario 2, three legs can be supported for PDCP duplication.

Proposal 9: The legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE can be reused as baseline for multi-path scenario 2.

Proposal 10: The current Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE can only be transmitted on the direct path.

Proposal 11:  For scenario 2, the UE-UE link failure detection is based on remote UE implementation.

Proposal 12: For scenario 2, remote UE can report UE-UE link failure via MCGFailureInformation or SCGFailureInformation message, and which message is used depends on the serving cell of relay UE is in the MCG or SCG of the remote UE.

Proposal 13: For scenario 2, relay UE reports the UE-UE link failure is not supported.

Proposal 14: For scenario 2, UL/DL transmission of relay UE in Uu link are both scheduled by network and how to perform the data transmission between relay UE and remote UE depends on relay/remote UE implementation.

Reference

- R2-2301925 [Pre121][407] Summary of AI 8.9.4 MP

- TS38.331  NR; Radio Resource Control (RRC) protocol specification (Release 17)