3GPP TSG-RAN WG2 Meeting#122                                                         R2-2305285

Incheon, KR, 22nd – 26th May, 2023

Source:              CATT

Title:	             Consideration on CAPC and LBT impacts

Agenda Item:	7.15.4

Document for:	Discussion and Decision

Introduction

In this contribution, we mainly focus on the below remaining issues on CAPC and LBT, our views will be given based on the analysis.

- Issue 1: How to determine the SLRB’s CAPC for IDLE/INACTIVE/OOC/Connected UE if the SLRB’s CAPC is not configured in SIB/Preconfiguration/RRCsignalling?

- Issue 2: Whether the working assumption on multiple PSFCH occasions can be confirmed and whether there is other impact of introducing multiple PSFCH occasions per PSSCH/PSCCH transmission?

Discussion

SLRB’s CAPC for IDLE/INACTIVE/OOC/Connected UE

For operation with NR-U, SRB0, SRB1 and SRB3 are assigned with the highest priority Channel Access Priority Class (CAPC), (i.e. CAPC = 1) while CAPC for SRB2 and DRB are configurable. In NR-U, as in annex, the gNB could configure the CAPC in LogicalChannelConfig IE to UE for each logical channel associate to SRB2 or DRB.

In past RAN2 meetings, the following agreements were reached.

RAN2#120 Agreement on SL CAPC for RRC inactive/idle/OOC UE 1: For an IDLE/INACTIVE/OOC UE, if the QoS flow of non-standardized PQI can be mapped to a non-default SLRB, the UE determines the CAPC of this non-standardized PQI using the CAPC of this SLRB. RAN2#121 Agreement on SL CAPC mapping rule: 1: For an IDLE/INACTIVE/OOC UE, if a QoS flow cannot be mapped to a non-default SLRB: 1) if the per-bearer CAPC is configured in SIB/Pre-configuration, the UE use the configured CAPC; 2) else, select CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized QoS flow based on one or more QoS characteristics. For a standardized QoS flow, CAPC is directly derived from CAPC table.

For IDLE/INACTIVE/OOC UE in SL-U, the CAPC for a QoS flow could be determined based on above agreement, the ‘best-match’ issue for QoS flow was solved in [1]. But the CAPC for default SLRB without configured CAPC is not discussed so far. In our view, Tx UE could determine the default SLRB’s CAPC by two ways:

- Option 1: Depends on UE implementation. E.g., Tx UE will determine the default SLRB’s CAPC based on the CAPC information of all QoS flow(s) in a default SLRB,

- Option 2: Specify that the CAPC is determined by the lowest CAPC priority among the CAPC for all QoS flow(s) in a default SLRB from the view of fairness.

For the non-default SLRB, RAN2 has agreed that the CAPC value is (pre)configurable per-DRB as in NR-U. But it is still unclear the CAPC for non-default SLRB will be always configured or not. If its CAPC is not configured in the SIB/Pre-configuration, the IDLE/INACTIVE/OOC UE could also apply above option to determine the non-default SLRB’s CAPC.

RAN2#119bis Agreement on SL CAPC 2: For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U.

For RRC Connected UE, gNB could configure the CAPC for SLRB. But in NR-U, as in annex, it is optional to configure the CAPC for a NR-U RB. Hence, it is also possible that network doesn’t configure the CAPC of SLRB by RRC signaling, in this case, the connected UE will apply the same CAPC determination option as for IDLE/INACTIVE/OOC UE.

Proposal 1: For IDLE/INACTIVE/OOC/connected UE, if the CAPC of the SLRB is not configured in SIB/Pre-configuration/RRC signaling, it is suggested to do down-selection from the following two options:

- Option 1: The CAPC of the SLRB will be determined by UE implementation based on the CAPC of all QoS flow(s) in the SLRB.

- Option 2: The CAPC of the SLRB will be determined by the lowest CAPC priority among the CAPC of all QoS flow(s) in the SLRB.

Multiple PSFCH occasions per PSCCH/PSSCH

Regarding to the multiple PSFCH occasions, the following working assumption is reached in RAN2#121 meeting:

3a:	Working assumption: If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.

3b: If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

In RAN1#112 meeting, the following agreement was reached:

Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

Based on the above agreement, it is obvious that more than 1 PSFCH occasion per PSCCH/PSSCH transmission is already supported in RAN1. Hence, RAN2 working assumption can be confirmed with modification accordingly.

Proposal 2: Confirm the working assumption on multiple PSFCH occasion with some modification, as below:

- In case of multiple PSFCH occasion per PSCCH/PSSCH, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.

- In case of multiple PSFCH occasion per PSCCH/PSSCH, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

In addition, with the introduction of multiple PSFCH occasions per PSCCH/PSSCH, it also has impact on SL RLF judgement. According to the current spec, when numConsecutiveDTX reaches sl-maxNumConsecutiveDTX, UE will indicate HARQ-based Sidelink RLF detection to RRC. If cases of multiple PSFCH occasions are introduced per PSCCH/PSSCH, some PSFCH occasions may have no PSFCH transmission due to LBT, but according to the current MAC spec, the numConsecutiveDTX will still be accumulated. This will increase probability of SL RLF. In order to avoid unnecessary trigger of SL RLF, it is proposed that for multiple PSFCH occasions can be regarded as one unit, and only no HARQ FB is received on all the PSFCH occasions in this unit, the numConsecutiveDTX can be added by 1.

Proposal 3: Only when no HARQ FB is received on all the PSFCH occasions which corresponding to one PSCCH/PSSCH is received, the number of HARQ DTX can be added by 1.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Proposal 1: For IDLE/INACTIVE/OOC/connected UE, if the CAPC of the SLRB is not configured in SIB/Pre-configuration/RRC signaling, it is suggested to do down-selection from the following two options:

- Option 1: The CAPC of the SLRB will be determined by UE implementation based on the CAPC of all QoS flow(s) in the SLRB.

- Option 2: The CAPC of the SLRB will be determined by the lowest CAPC priority among the CAPC of all QoS flow(s) in the SLRB.

Proposal 2: Confirm the working assumption on multiple PSFCH occasion with some modification, as below:

- In case of multiple PSFCH occasion per PSCCH/PSSCH, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.

- In case of multiple PSFCH occasion per PSCCH/PSSCH, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

Proposal 3: Only when no HARQ FB is received on all the PSFCH occasions which corresponding to one PSCCH/PSSCH is received, the number of HARQ DTX can be added by 1.

Reference

- Report from session on NR SL for RAN2#121bis-e, OPPO.

Annex

- LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- Cond PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                   OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, ms0p01-v1700, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                              OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                                 OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}               OPTIONAL,    -- Need R

[[

allowedCG-List-r16                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL,   -- Need S

allowedPHY-PriorityIndex-r16        ENUMERATED {p0, p1}                                                 OPTIONAL    -- Need S

]],

[[

logicalChannelGroupIAB-Ext-r17      INTEGER (0..maxLCG-ID-IAB-r17)                                      OPTIONAL,   -- Need R

allowedHARQ-mode-r17                ENUMERATED {harqModeA, harqModeB}                                   OPTIONAL    -- Need R

]]

}                                                                                                       OPTIONAL,   -- Cond UL

...,

[[

channelAccessPriority-r16           INTEGER (1..4)                                                      OPTIONAL,   -- Need R

bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}                                   OPTIONAL    -- Need R

]]

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP