3GPP TSG-RAN WG2 Meeting #122      	                          R2-2305292

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.4.2.1

Source: 	OPPO

Title:  	Discussion on early TA acquisition and maintenance for LTM

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 has triggered the discussion regarding to PDCCH ordered early RACH based on RAN1 LS in. And the following agreements have been reached:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch).

R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18.

In this contribution, we would like to provide our views on the potential RAN2 impacts on TA acquisition, maintenance for LTM.

Discussion

To reduce the latency caused by synchronization procedure, RAN1 has agreed to support early synchronization for TA acquisition of a candidate cell before cell switch command is received. PDCCH ordered-CFRA is performed for early synchronization to candidate cell(s) and whether RAR needs to be received is configured by RRC. With RAN1 agreements, RAN2 further concludes that RAR reception from a candidate cell is not supported in R18 to avoid additional data interruption.

RAR reception from serving cell is configured

For serving cells, maintenance of uplink time alignment relies on a TA timer that is associated with a TAG. UE starts/restarts the corresponding TA timer if TA is indicated by NW. For PDCCH ordered-RACH for candidate cell(s), RAR including TA information will be provided to UE if RAR reception is configured. As the TA value is associated to a candidate cell, how to store and maintain the TA should be studied.

- One way is to reuse TAG for TA indication and maintenance. Similar as serving cell TA maintenance, UE starts or restarts the timeAlignmentTimer associated with this TAG when a timing advance command is received in a random access response message for a candidate cell belonging to a TAG. In that way, TAG ID should be unique at least among serving cell(s) and candidate cells.

- Another way is to use separate way to maintain the TA for serving cell and candidate cell. For example, UE stores the received TA in early sync procedure of a candidate cell . Upon LTM execution completion, UE needs re-associate the TA to the TAG indicated in candidate cell config if the candidate cell becomes to the serving cell.

During MAC partial reset discussion, numerous companies believe that TA maintenance can be continued upon reception of LTM cell switch command. To be simple and clear, a common way to indicate and maintain TA for both serving and candidate is preferred.

TA timer associated with TAG is reused for candidate cell TA maintenance.

TAG ID is unified among serving cell(s) and candidate cell(s).

TA timer is started/restarted if timing advance command is received in a random access response message for a candidate cell belonging to a TAG.

Currently, there is not agreements on the number of candidate cells that UE can perform early sync procedure, that is, UE may acquire the TA of candidates in addition to target cell. It is questionable on how to handle the TA of other candidate cells during LTM execution. In our understanding, TA timer(s) associated with candidate cell (TAG) can be maintained for subsequent LTM execution. If TAG is used for candidate TA maintenance, the timer can be restarted if TA command is received for the TAG. And upon expiration of the TA timer, no action is needed for UE if it only associates with LTM candidate cell.

TA timer associated with other candidate cell (TAG) is not stopped upon LTM execution.

In addition to TA, legacy RAR also includes the fields to indicate TC-RNTI and UL grant. While the candidate cell configuration provided to UE already includes C-RNTI for the candidate cell, there is no necessary to provide redundant RNTI to UE. Regarding to UL grant of candidate cell, it is waste of resources to provide candidate cell resource before cell switch since UE will only perform data transmission with current serving cell instead of candidate cell. Thus, the field of UL grant (candidate cell) and TC-RNTI in RAR for the RACH triggered for early synchronization is not necessary.

For RACH triggered by early sync, UL grant (candidate cell) and T-CRNTI in RAR are not needed. FFS on whether to introduce new RAR format.

In conventional contention-free RACH procedure, UE declares the RACH completion by the reception of RAR with matched RAPID. Similar as legacy, RACH procedure which is triggered by early sync is considered as successfully complete if RA response is received. Otherwise, the RACH procedure is declared as failed if the maximum number of attempts is reached and UE will indicate a RACH problem to upper layer. Other than RACH towards to serving cell, early sync to candidate failure is not equivalent to poor radio link condition of serving cell connection. So, the serving cell connection should not be impacted due to early sync failure, i.e., RACH problem is not indicated to upper layer if the RACH is triggered for early synchronization.

For RACH triggered for early synchronization, if RACH failure is declared, the MAC entity considers the RACH procedure is unsuccessfully completed without indicating upper layer.

“without RAR” is configured

RAN1 has also introduced RAR-skip based early synchronization, and it is still questionable on whether and how to support subsequent preamble transmission without RAR reception. According to the latest RAN1 agreements, UE autonomous retransmission is not supported for PDCCH-ordered RACH without RAR.

In this case, UE only support one-shot preamble transmission based on PDCCH order. Upon the transmission of preamble transmission, UE consider the RACH procedure triggered by early sync is completed. And if preamble is not successful received, NW may trigger the PRACH transmission again by PDCCH order.

For PDCCH-ordered RACH, if reception of RAR is not configured, UE considers the RACH is completed upon transmission of preamble.

While there is one potential issue on power ramping for subsequent PRACH transmission in one shot PRACH transmission case. Legacy RACH procedure, UE ramps the transmission power of PRACH transmission based on the pre-configured PREAMBLE_POWER_RAMPING_STEP. If subsequent PRACH transmission relies on the PDCCH order, UE will always consider the PRACH transmission as initial transmission, i.e. the transmit power is determined by preambleReceivedTargetPower configured by RRC without ramp. To avoid the PRACH transmission failure due to low transmit power, further study is required on how UE determines the transmit power of subsequent PRACH triggered by PDCCH order, for example, inclusion of power info by DCI or support UE autonomous power ramping.

RACH collision

Another open issue for early sync is about RACH collision. In legacy, if a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure. For LTM, RACH procedures toward serving cell and candidate cells may cause RACH storm. Stop the ongoing RACH towards serving cell may cause service interruption, and candidate cell early TA acquisition failure will not impact the service continuity. Thus, RAN2 may need to specify the UE behavior clearly since the priority of RACH towards serving cell and candidate cell is different. For example:

- If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell.

- If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH procedure.

- If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure.

If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell.

If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH trigger.

If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new RACH procedure.

Conclusion

Based on the discussion in section 2 we have following observations:

Proposal 1	TA timer associated with TAG is reused for candidate cell TA maintenance.

Proposal 2	TAG ID is unified among serving cell(s) and candidate cell(s).

Proposal 3	TA timer is started/restarted if timing advance command is received in a random access response message for a candidate cell belonging to a TAG.

Proposal 4	TA timer associated with other candidate cell (TAG) is not stopped upon LTM execution.

Proposal 5	For RACH triggered by early sync, UL grant (candidate cell) and T-CRNTI in RAR are not needed. FFS on whether to introduce new RAR format.

Proposal 6	For RACH triggered for early synchronization, if RACH failure is declared, the MAC entity considers the RACH procedure is unsuccessfully completed without indicating upper layer.

Proposal 7	For PDCCH-ordered RACH, if reception of RAR is not configured, UE considers the RACH is completed upon transmission of preamble.

Proposal 8	If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell.

Proposal 9	If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH trigger.

Proposal 10	If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new RACH procedure.

Reference

- RP-222332 Further NR Mobility Enhancements    MediaTek Inc