3GPP TSG-RAN WG2 Meeting #122                                    R2-2305297

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.4.3

Source: 	OPPO

Title:  	Open issues for selective activation of SCGs for NR-DC

Document for:	Discussion and Decision

Introduction

In last meeting, the following agreements have been reached for selective activation of the cell groups regarding to candidate cell configuration and execution condition:

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format.

For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

- FFS on the following options for subsequent CPC:

- Option 1: Source MN generates the execution conditions for all subsequent CPC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPC.

For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. 
FFS if Candidate SN may generate/modify execution conditions for subsequent CPC

Assume for now that there is only one reference configuration.

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

- FFS whether MCG configuration is included.

- FFS RRC model for the reference configuration.

- Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included.

- 3. The execution conditions associated with each candidate target PSCell.

- a.	For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported.

- b.	For SN initiated procedure, execution conditions based on events A3/A5 are supported.

UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

In this contribution, we would like to provide our views on open issues for selective activation of SCGs, mainly including candidate condition evaluation, candidate cell configurations.

Discussion

NR-DC with selective activation cell of groups is one of key objectives for R18 mobility. In previous meeting, RAN2 has briefly discussed the term for NR-DC with selective activation of cell groups, but no consensus has been reached. In our understanding, we can simply use the term SACG which is short for selective activation of cell groups.

Use SACG as the term for NR-DC with selective activation of cell groups.

Candidate execution condition

For selective activation of SCG, we agreed to support both MN-initiated case and SN-initiated case. In legacy, for MN-initiated CPA/CPC, the execution condition of candidates referring to MCG measurement configuration and execution conditions based on A4 event are supported. For SN-initiated CPA/CPC, the execution condition of candidates referring to SCG measurement configuration and execution conditions based on A3/A5 event are supported.

Assume to support the following scenarios of SCG selective activation:

    - SN initiated intra-SN SCG selective activation

    - MN initiated inter-SN SCG selective activation

    - SN initiated inter-SN SCG selective activation

For MN initiated inter-SN SCG selective activation, it is agreed that source MN generates the execution conditions for the initial CPAC. Regarding to the subsequent CPC, whether source MN or candidate SN to generate the execution conditions is FFS.

For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

- FFS on the following options for subsequent CPC:

- Option 1: Source MN generates the execution conditions for all subsequent CPC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPC.

In our understanding, if we allow candidate SN to generate/modify the execution condition for subsequent CPC procedure, the following issues may be introduced:

- Issue 1：Additional Xn signalling interactions for execution condition preparation

To prepare the execution condition, each candidate is required to aware of the full candidate cell list, which may cause additional signalling interactions. And the execution condition associated measurement configuration which provided by MN may also be required as a reference for candidate SCG to prepare the execution condition for subsequent CPC.

- Issue 2：Reconfiguration for all candidate cells is required if NW modify the candidate list

If candidate SN prepares the execution condition for all of other candidates during SCG selective activation preparation, once NW modify the candidate cell list, i.e. add/release candidate cell, reconfiguration of execution condition for the new added or released candidate cell is required. As each candidate has prepared the execution conditions for all other candidates, reconfigurations for all candidate cells are required accordingly. If candidate cell prepares the execution conditions for all of other candidates after UE has accessed to the candidate cell, UE needs to postpone the condition evaluation for other candidate cells until reception of the execution condition reconfiguration message.

For MN-initiated SCG selective activation, Source MN generates the execution conditions for all subsequent CPC

For MN-initiated SCG selective activation, A3/A5 is not supported for initial and subsequent CPC.

For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. Whether candidate SN can generate/modify execution conditions for subsequent CPC is also FFS.

For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. 
FFS if Candidate SN may generate/modify execution conditions for subsequent CPC

Other than MN initiated case, the measurement configuration of candidates may be differed from source SN. For subsequent CPC, execution condition of candidates should be updated since UE does not apply source SN measurement configuration anymore. To avoid frequently reconfiguration of the execution condition, a common measurement configuration can be introduced for execution condition configuration. With common measurement, NW is not required to reconfigure the execution of all candidates upon each CPC.

Considering the reasons mentioned above, we have the following proposals:

For SN-initiated SCG selective activation, candidate SN may generate/modify execution conditions for subsequent CPC.

For SN-initiated CPC, introduce common measurement configuration for execution conditions association for subsequent CPC.

And it has been agreed that current serving SCG can be configured as a candidate in SCG selective activation. If NW configures the serving cell as a candidate, it is questionable whether and how UE skip the condition evaluation for the serving cell. For simplicity, we think it can leave to UE implementation on skipping the condition evaluation for a candidate which is a current serving cell.

It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell.

It is up to UE implementation on how to skip the condition evaluation for a candidate which is a current serving cell.

In previous meeting, baseline procedure for selective activation of cell groups has been agreed as general guidance:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

- a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

- b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

- c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

For subsequent CPC defined in step 2, it is unclear how and when UE performs candidate cell evaluation of other candidate cells. In legacy CPA/CPC, UE starts to evaluate the execution condition(s) upon receiving the configuration for CPC/CPA, and stops evaluating the candidates once conditional PSCell change/addition is triggered. For the first CPC/CPA, the existing evaluation mechanism can be reused, while it is not feasible to cover subsequent CPC/CPA since no reconfiguration message is expected. Therefore, further study is needed to define when to start the evaluation on candidate cells for subsequent CPC/CPA.

UE starts evaluating the execution condition(s) upon receiving the CPC/CPA configuration, and stops evaluating once PSCell change is triggered. For consecutive CPC/CPA, UE will not receive RRC message for CPC/CPA configuration since the configured CPC/CPA has not been released.

To evaluate other candidate cells, the most straightforward way is to start evaluating other candidate cells directly after last PScell change/addition procedure. In that case, addition measurement overload will be leaded since the cell change event may not be triggered immediately after completion of last CPC/CPA. On the other hand, the condition configuration may be invalid due to the change of measurement configuration, i.e. for SN-initiated SCG selective activation. Reconfiguration for measurements/conditions is required if reference measurement configuration changes. Another alternative is to define a new trigger to start evaluating candidate cells, which requires further RAN2 work on designing the new trigger.

As a baseline, UE starts the condition evaluation for subsequent CPC/CPA procedure upon CPA/CPC completion. FFS on other conditions to trigger evaluation for subsequent CPC.

Reference configuration

Reference configuration is employed for SCG selective activation to support delta candidate cell configuration, which is aiming to reduce the signalling overheads. The granularity of reference configuration has been discussed, and RAN2 assume single reference configuration is supported.

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

Assume for now that there is only one reference configuration.

However, SCG selective activation aiming to support consecutive CPA/CPC procedure without release the candidate configurations, and we agreed to support both MN-initiated case and SN-initiated case. There may be one case that UE receives reference configurations from MN and SN accordingly. For example, UE receives reference configuration, configuration for candidate cell#1 and candidate cell#2 from MN first. After then, SN initiates intra-SN SCG selective activation without MN involvement, a new reference configuration and configuration for candidate cell#1 are delivered to UE via SRB3. According to legacy conditional reconfiguration procedure, the new candidate #1 configuration will replace the old one, and if we support only one set of reference configuration, the new reference will replace the old one as well. In that case, whether the new reference configuration is still valid to generate a complete configuration for candidate cell#2 is questionable. In our understanding, coordination between MN and SN may be required to guarantee a valid reference configuration for all candidate cells.

NW should guarantee that the latest reference configuration is valid for all candidate cells.

Candidate configurations

Similar as legacy CPAC procedure, NW may provide multiple pre-configured candidate cells to UE for supporting successive CPAC procedure. It is questionable how to provide the candidate cell configurations for selective activation of SCG.

- Nested configuration: the candidate cell configuration is provided in a nested manner, i.e. CPC/CPA configuration in CPC/CPA configuration.

  - In this way, NW needs to know accurate UE trajectory for candidate cell preparation. Otherwise, the prepared candidate cell configuration may be wasted;

  - Furthermore, UE may not be able to move back and forth to candidate cells in first layer configuration with nested configuration, i.e., the candidate cells stored in UE variable might be replaced by the second layer configuration;

  - And the candidate cell preparation takes longer time since the candidate cell #1* configuration which nested in candidate cell #1 is prepared by candidate cell#1, and candidate cell #1 preparation is trigged by source SCG/MCG.

- Parallel configuration: the candidate cell configuration is provided within one set, which is similar as CPAC candidate cell configuration manner.

Considering the potential issues that would be brought by nested structure, it is proposed to use parallel way to provide candidate configurations for selective activation of SCGs.

Nested configuration is not supported for SACG candidate cell configuration.

In R16/17 CPAC, conditional reconfiguration is applied for candidate cell configuration. SACG may reuse legacy conditional reconfiguration way to provide the candidate cells configuration. one potential issue is that how to differentiate the candidate cell for subsequent CPAC and legacy conditional PScell change/addition. Additional information to indicate the candidate cell type may be required in this manner.

Explicit indication is introduced to differentiate whether the candidate cell is a SACG candidate or R17 CPAC candidate.

With pre-configured candidate cells, how to maintain the candidate configurations as well as guarantee the validity of the configurations for subsequent CPC/CPA should be studied. In general, we understand that it is NW responsibility to guarantee the validity of candidate cell configuration, for example, NW can modify/release the candidate cell configuration if it is invalid. And RAN2 also reached consensus on releasing CPC/CPA configurations by explicitly NW indication for inter-MN Pcell change scenario.

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

Further study is required for other cases to release the configuration for selective activation of SCG, for example:

- After SCG is released

- After UE goes to RRC_IDLE/ RRC_INACTIVE

- After UE triggers RRC re-establishment

RAN2 study whether to release the selective activation of SCG configurations in the following scenarios: 1) After SCG is released; 2) After UE goes to RRC_IDLE/ RRC_INACTIVE; 3) After RRC reestablishment.

One more issue is about the configuration type differentiation. For efficient use of candidate configurations, RAN2 assumes that a CPA conditional configuration can be used for CPC considering that CPA configuration always provides a complete RRC reconfiguration message to UE.

R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

And for SCG selective activation, as we already reach consensus on using delta configuration on top of reference configuration to form a complete candidate cell configuration, we see no difference for CPA candidate configuration and CPC candidate configuration since all candidate configuration will be formed as a complete configuration.

CPC conditional configuration can be used for CPA.

If CPC configuration cannot be used for CPA, an indication is introduced to differentiate whether the configuration is used for CPC or CPA.

Conclusion

Based on the discussion in section 2 we have following observations:

Observation 1	UE starts evaluating the execution condition(s) upon receiving the CPC/CPA configuration, and stops evaluating once PSCell change is triggered. For consecutive CPC/CPA, UE will not receive RRC message for CPC/CPA configuration since the configured CPC/CPA has not been released.

Based on the discussion above, we have the following proposals:

Proposal 1	Use SACG as the term for NR-DC with selective activation of cell groups.

Proposal 2	For MN-initiated SCG selective activation, Source MN generates the execution conditions for all subsequent CPC

Proposal 3	For MN-initiated SCG selective activation, A3/A5 is not supported for initial and subsequent CPC.

Proposal 4	For SN-initiated SCG selective activation, candidate SN may generate/modify execution conditions for subsequent CPC.

Proposal 5	For SN-initiated CPC, introduce common measurement configuration for execution conditions association for subsequent CPC.

Proposal 6	It is up to UE implementation on how to skip the condition evaluation for a candidate which is a current serving cell.

Proposal 7	As a baseline, UE starts the condition evaluation for subsequent CPC/CPA procedure upon CPA/CPC completion. FFS on other conditions to trigger evaluation for subsequent CPC.

Proposal 8	NW should guarantee that the latest reference configuration is valid for all candidate cells.

Proposal 9	Nested configuration is not supported for SACG candidate cell configuration.

Proposal 10	Explicit indication is introduced to differentiate whether the candidate cell is a SACG candidate or R17 CPAC candidate.

Proposal 11	RAN2 study whether to release the selective activation of SCG configurations in the following scenarios: 1) After SCG is released; 2) After UE goes to RRC_IDLE/ RRC_INACTIVE; 3) After RRC reestablishment.

Proposal 12	CPC conditional configuration can be used for CPA.

Proposal 13	If CPC configuration cannot be used for CPA, an indication is introduced to differentiate whether the configuration is used for CPC or CPA.

Reference

- RP-222332 Further NR Mobility Enhancements    MediaTek Inc