3GPP TSG-RAN WG2 Meeting #122                     			  R2-2305299

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.18.2

Source: 	OPPO

Title:  	Discussion on control plane issues for MT-SDT

Document for:	Discussion and Decision

Introduction

In last RAN2#121 meeting, the following agreements were made for MT-SDT.

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In this contribution, we would like to further discuss the control plane open issues for MT-SDT left from last RAN2.

Discussion

Resume cause for RRC resume procedure

Based on the agreement, UE initiates legacy RRC Resume procedure if the conditions to initiate paging-triggered MT-SDT are not met. It is FFS on the resume cause to be included in the RRCResumeRequest for the RRC Resume procedure triggered in this case.

3. In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

In our understanding, legacy RRC resume procedure and MT/MO-SDT procedure should be defined independently from each other. Therefore, when UE determines to initiate legacy RRC Resume procedure when the conditions to trigger SDT are not stratified, legacy UE behaviour upon the reception of paging message is followed to set the resume cause as the text cited below.

2> if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI: 3> if the UE is configured by upper layers with Access Identity 1: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess; 3> else if the UE is configured by upper layers with Access Identity 2: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess; 3> else if the UE is configured by upper layers with one or more Access Identities equal to 11-15: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess; 3> else: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

If the conditions to trigger MT-SDT are not fulfilled, legacy UE behaviour upon the reception of paging message is followed to set the resume cause, i.e., the resume cause is set based on Access Identity.

Signalling details of MT-SDT indication in paging

For MT-SDT, the procedure is triggered by network when there is DL data arrival. As agreed in last meeting,

a one-bit indication per UE will be include in paging to trigger MT-SDT.

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

In LTE MT-EDT, the indication to trigger MT-EDT is included in pagingRecord field as shown below.

PagingRecord-v1610 ::=				SEQUENCE {

accessType-r16						ENUMERATED {non3GPP}			OPTIONAL,		-- Need ON

mt-EDT-r16							ENUMERATED {true}				OPTIONAL		-- Need ON

}

The similar design can be reused for R18 MT-SDT. For the name of the indication, MT-SDT-r18 can be considered and the PagingRecord filed is extended to include this indication. The signalling structure is provided in Figured 1 as follows.

Figure 1 Signalling structure of paging with MT-SDT indication

MT-SDT indication is included in PagingRecord field.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	If the conditions to trigger MT-SDT are not fulfilled, legacy UE behaviour upon the reception of paging message is followed to set the resume cause, i.e., the resume cause is set based on Access Identity.

Proposal 2	MT-SDT indication is included in PagingRecord field.