3GPP TSG RAN WG2 #122												  R2-2305304

Incheon, Korea, May 22th –26th, 2023

Agenda Item:	7.8.3

Source: 	NEC

Title:	Discussion on Flight Path Reporting

Document for:	Discussion

- Introduction

In RAN2 121 the below agreements were made [1]:

- Flightpath update indication in UAI is configurable by the network - Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE - Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307] - As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s) - As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication. FFS if further differentiation is needed if we decide to have delta signaling

This contribution provides our considerations on flight path information and reporting.

- Discussion

2.4 Fight path plan reporting in RRC_INACTIVE

Fight path reporting mechanism introduced in LTE Rel-15 contains two steps:

- Step 1: Inform the network “flight path info is available”

- Step 2: Include flightPathInfoReport and set it to include the list of waypoints along the flight path upon reception of UEInformationRequest message (with flightPathInfoReq field present);

In previous meeting, RAN2 agreed to allow the flight path to be updated. Our understanding is that the flight path update mechanism itself is out of RAN2 scope, therefore RAN2 discussion should focus on how to report the flight path update to the network.

In previous meeting, RAN2 also agreed that UAV UEs need to inform the network “flight path info is available” firstly. The availability indication can be included in RRCConnectionSetupComplete message, RRCConnectionResumeComplete message, RRCConnectionReconfigurationComplete message or RRCConnectionReestablishmentComplete message，upon reception of RRCConnectionSetup,  RRCConnectionResume or RRCConnectionReconfiguration including mobilityControlInfo or RRCConnectionReestablishment by the UE. In other words, only RRC_CONNECTED UAV UEs can report flight path plan.

Only RRC_CONNECTED UAV UEs can report flight path.

For the NR, when flight path is updated, the above legacy LTE procedure can be reused by informing the network “flight path info is updated and available”. However, when the UAV UEs is in RRC_INACTIVE and merely have flight path update to report, they still have to enter RRC_CONNECTED state firstly and send the report according to specified procedures. In such a case, RRC connection setup and subsequently release to RRC_IDLE/INACTIVE state happens every time reporting flight path update.

In previous meeting, RAN2 agreed to adopt location list of waypoints (3D location information) and timestamp as the basic content of flight path report. As summarized in TS22.125, typical message size of waypoint report ranges from 84-140 bytes.

When small size flight path plan is frequently reported, LTE-like mechanism may cause unnecessary power consumption and signaling overhead. In general, we observe that enabling flight path plan report in RRC_INACTIVE with Rel-17 SDT procedure, which is a procedure allowing data and/or signaling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state), is beneficial from network performance/efficiency and UE battery performance perspectives.

RAN2 to consider enabling flight path update report in RRC_INACTIVE.

- Conclusion

Observation 1:	Only RRC_CONNECTED UAV UEs can report flight path.

Proposal 1:	RAN2 to consider enabling flight path update report in RRC_INACTIVE.

- References

[1] R2-23xxxxx, Report of 3GPP TSG RAN WG2 meeting #121bis-e, ETSI MCC.