3GPP TSG-RAN WG2 Meeting #122                                                                                 R2-2305319

Incheon, KR, May 22 – 26, 2023

Source:	CATT

Title:	Discussion on Unified TCI Framework Extension for Multi-TRP

Agenda Item:	7.20.3

Document for:	Discussion and Decision

- 1. Introduction

In last meeting, RAN2 discussed unified TCI framework extension for multi-TRP, and achieved the progress as below [1].

Working assumption: Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

In this contribution, we will continue to discuss unified TCI framework extension for multi-TRP and give our considerations. Our proposals are summarized in section 3.

- 2. Discussions

2.1 Unified TCI framework extension for M-DCI based MTRP

In last meeting, unified TCI framework extension for M-DCI based MTRP was discussed and achieved the working assumption that “Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation”.

The work assumption algins with the agreement achieved by RAN1:

Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value “RAN2 impact: MAC-CE for TCI state activation” “RAN4 impact: none”

Since this requirement is already very clear, it is nature for RAN2 to follow RAN1’s conclusion and enhance the R17 Unified TCI States Activation/Deactivation MAC CE to fulfill the above requirement from RAN1. Considering that there are some reserved bits in R17 Unified TCI States Activation/Deactivation MAC CE, the simplest way is using one of the reserved bit to indicate the CORSET Pool ID (coresetPoolIndex=0 or coresetPoolIndex=1), Figure 1 gives an example as below.

Therefore, we propose,

Proposal 1: RAN2 to confirm the work assumption that “Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation”.

Figure 1 Example of enhanced unified TCI States Activation/Deactivation MAC CE for M-DCI based MTRP

2.2 Unified TCI framework extension for S-DCI based MTRP

In RAN1#112bis-e meeting, RAN1 discussed unified TCI framework extension for S-DCI based MTRP and the following agreements were achieved [2].

Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

Based on the agreements above, it can be seen that for a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE), which means the TCI state activation MAC CE should support: the TCI state(s) of one TRP or both two TRPs (i.e. up to 2 TCI state) can be mapped to a TCI codepoint in DCI.

Similarly, for a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE), which means the TCI state activation MAC CE should support: the TCI state(s) of one TRP or both two TRPs (i.e. up to 4 TCI state) can be mapped to a TCI codepoint in DCI.

In addition, according to the agreement above (the red font), the TCI state activation command (MAC-CE) should indicate that each joint DL/UL TCI state mapped to a TCI codepoint is the first or second joint DL/UL TCI state. It means the besides to indicate the TCI state is mapping to with one or both of the TRP, the TCI state activation MAC CE should support to indicate which TRP’s TCI state is activated.

Proposal 2: For unified TCI framework extension for S-DCI based MTRP, the TCI state activation MAC CE should support that each TCI codepoint corresponds to one of the following cases:

- first TRP’s TCI state

- second TRP’s TCI state

- first TRP’s TCI state and second TRP’s TCI state

Furthermore, the TCI state mode should be joint DL/UL TCI mode or separate DL/UL TCI mode, if the joint DL/UL TCI model is configured, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint, i.e. for joint DL/UL TCI mode, the following cases should be supported to be indicated by the TCI state activation MAC CE for each corresponding TRP: one joint DL/UL TCI state.

If the separate DL/UL TCI mode is configured, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint, i.e. for separate DL/UL TCI mode, the following cases should be supported to be indicated by the TCI state activation MAC CE for each corresponding TRP: a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states.

Proposal 3a: For unified TCI framework extension for S-DCI based MTRP, if joint DL/UL TCI model is configured, the TCI state activation MAC CE should support that each corresponding TRP maps to:

- one joint DL/UL TCI state

Proposal 3b: For unified TCI framework extension for S-DCI based MTRP, if separate DL/UL TCI model is configured, the TCI state activation MAC CE should support that each corresponding TRP maps to:

- a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states

According to the requirement above, neither the Unified TCI States Activation/Deactivation MAC CE nor the Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE can fulfill the requirement. Therefore, a new MAC CE needs to be introduced for TCI state activation for unified TCI framework extension for S-DCI based MTRP.

Proposal 4: Introduce a new TCI state activation MAC CE for unified TCI framework extension for S-DCI based MTRP.

- 3. Conclusion

This paper gives some initial considerations on the Unified TCI Framework Extension for Multi-TRP. The contribution of this paper is summarized as follows,

Unified TCI framework extension for M-DCI based MTRP

Proposal 1: RAN2 to confirm the work assumption that “Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation”.

Unified TCI framework extension for S-DCI based MTRP

Proposal 2: For unified TCI framework extension for S-DCI based MTRP, the TCI state activation MAC CE should support that each TCI codepoint corresponds to one of the following cases:

- first TRP’s TCI state

- second TRP’s TCI state

- first TRP’s TCI state and second TRP’s TCI state

Proposal 3a: For unified TCI framework extension for S-DCI based MTRP, if joint DL/UL TCI model is configured, the TCI state activation MAC CE should support that each corresponding TRP maps to:

- one joint DL/UL TCI state

Proposal 3b: For unified TCI framework extension for S-DCI based MTRP, if separate DL/UL TCI model is configured, the TCI state activation MAC CE should support that each corresponding TRP maps to:

- a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states

Proposal 4: Introduce a new TCI state activation MAC CE for unified TCI framework extension for S-DCI based MTRP.

- 4. Reference

[1] Draft_R2_121bis_meeting_report_v2

[2] Draft_Minutes_report_RAN1#112b-e_v010