3GPP TSG-RAN WG2 Meeting #122	R2-2305324

Incheon, Korea, 22nd-26th May, 2023

Source: 	vivo

Title:  	Remaining issues on SON for SPR

Agenda Item:	7.13.4

Document for:	Discussion and Decision

Introduction

The following agreements regarding SPR were achieved in the previous meeting [1][2][3]:

RAN2#119bis-e:

Agreements

1	RAN2 confirms the scenarios for SPR for NR-DC, including:

•	SN- and MN-initiated classic PSCell change / CPC

•	Intra-SN classic PSCell change / CPC

•	Classic Addition / CPA

1a	RAN2 will discuss HO with SN change later, after the basic solution for SPR is known

2	Given that PSCell addition is proposed by all companies, SPR is used as the abbreviations to use for the feature.

3	RAN2 confirm to prioritize NR-DC scenario for SPR.

4	SHR solution is taken as baseline for the SPR in terms of configuration and reporting at high level. Details of the configuration and report need to be tailored/customized/new message per use case.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

RAN2#120:

Agreements:

1	For Q5 in R2-2211160, RAN2 confirms the support for the parameters for inter-RAT SHR from NR to LTE when T310 and T312 are configured as triggering condition.

2	T304 trigger for inter-RAT SHR from NR to LTE is not supported.

3	Only MN can retrieve the SPR from the UE.

4	For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.:

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

T304 trigger needs to be configured by the target SN node.

Agreements:

1	UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

2	UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

3	Only the latest successful PSCell change/addition is reported by the UE.

4	Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

5	UE records/reports PCell SHR and PSCell SPR separately

RAN2#121:

Agreement

1: UE includes available location information in SPR .

2: UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched.

3:  At least the following options are needed for releasing  SPR report:

a. New SPR is initiated

b. Upon retrieval of SPR

c. Detach is initiated.

4:  In SPR, reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

Although much progress has been made regarding SPR, there are some remaining issues unresolved mainly including the following three aspects. In this contribution, we will work on solving these issues to draw a full picture of SPR.

- Issue 1: Remaining issues of SPR configuration;

- Issue 2: What other information can be included in SPR;

- Issue 3: Whether additional work is needed for HO with SN change.

Discussion

- Issue 1: Remaining issues of SPR configuration

Regarding SPR configuration for the UE, there are three FFS after RAN2#119bis-e [1], one of them (i.e.,5c) was resolved at RAN2#120 and two remaining issues highlighted below (i.e., 5a and 5b) that need to be discussed.

5 Network configures SPR configuration IE for the UE, with at least the following triggering conditions: • T310 triggering condition • T312 triggering condition • T304 triggering condition 5a: Other triggering conditions are FFS 5b: Values of the triggering conditions are FFS 5c: Which node configures the triggering condition is FFS.

Referring to Rel-17 SHR, since the problems that occur during PSCell change/addition procedure and handover are similar, we think the current triggering conditions (i.e., T310 triggering condition, T312 triggering condition and T304 triggering condition) are sufficient, no other trigger condition is needed.

Proposal 1: The current agreed triggering conditions of SPR are sufficient, i.e., no other triggering condition is needed.

The next issue is how to configure the value of triggering conditions, we think the same configure mechanism can be adopted as SHR, i.e., define separate thresholds for T310/T312/T304 and the percentage values are used for the trigger conditions. RAN3 also assumes the triggers SPR should be represented in terms of percentage values in RAN3#119bis-e [4], the WA is shown as follows. Furthermore, we think the values of percentage can also be reused i.e., 40%, 60%, 80%, and the percentage value also includes 20% for the threshold for T312.

WA: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

Proposal 2: Regarding values of the triggering conditions of SPR, RAN2 to agree the following:

- Define separate thresholds for T310/T312/T304;

- The percentage value is to indicate the ratio of the threshold value (unit: ms) over the signalled T310/T312/T304 value (unit: ms);

- The percentage values are 40%, 60%, 80%, and the percentage value also includes 20% for the threshold for T312.

- Issue 2: What other information can be included in SPR

In order to enable the network to optimize the configuration of PSCell addition/change, the node that retrieves the SPR should forward the corresponding information to the network that optimizes the configuration. Regarding forwarding of SPR, in RAN3#119bis-e [4], RAN3 agreed that the “new node” should send the SPR to the “new node” and the “old node” is responsible for forwarding to the respective node(s) which should perform the SPR optimization. And CGI of the PCell which sent the SPR configuration is to assist in the forwarding of SPR from RAN3 perspective. The agreements from RAN3 are shown as follows. To align with RAN3 agreements, RAN2 to agree that CGI of the PCell which sent the SPR configuration should be included in SPR.

Furthermore, upon “old node” receiving the SPR, the “old node” should forward the information in SPR to the respective node(s) which should perform the SPR optimization. If the old node receives the SPR long after it sends the PSCell addition/change configuration was sent, this configuration may already be released when it receives the SPR, so the “old node” does not know whether or which network node to forward the SPR. For PSCell addition and MN-initiated PSCell change, MN is the network node that needs SPR and need not forward the SPR to other nodes; for SN-initiated PSCell change, MN should forward the received SPR to SN. To assist old MN node to decide whether or which network node to forward the SPR, an indication is needed to indicate the type of PSCell addition/change (i.e., PSCell addition, MN-initiated PSCell change or SN-initiated PSCell change). This also aligns with the work assumption in RAN3 [4].

Agreements: In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization. ERICSSON: old node should be old MN node Agreements: To assist in the forwarding of SPR, UE may include the following in SPR - CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed) - WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Proposal 3: The following information should be included in the SPR:

- CGI of the PCell which sent the SPR configuration.

- An indication to indicate the type of PSCell addition/change, i.e., PSCell addition, MN-initiated PSCell change or SN-initiated PSCell change.

- Issue 3: Whether additional work is needed for HO with SN change

Regarding the scenario, RAN2 has agreed that HO with SN change will be discussed after the basic solution for SPR is known.

Agreements

1	RAN2 confirms the scenarios for SPR for NR-DC, including:

•	SN- and MN-initiated classic PSCell change / CPC

•	Intra-SN classic PSCell change / CPC

•	Classic Addition / CPA

1a	RAN2 will discuss HO with SN change later, after the basic solution for SPR is known

Next, we will analyze whether additional work is needed specifically for this scenario. If HO fails regardless of SN change succeeds or fails, SHR and SPR will not be generated; in other words, both SHR and SPR will be generated in the HO with SN procedure only if the HO succeeds and the SN change succeeds. In our understanding, this is no different from the UE successfully executing the HO procedure first and then executing the SN addition or change procedure, since SHR and SPR are generated and the network can request these two reports and forward the corresponding network node. Thus, in order to support HO with SN change, the existing SHR and SPR mechanisms are sufficient and no additional work is needed.

Proposal 4: For HO with SN change, the existing SHR and SPR mechanisms are sufficient and no additional work is needed.

Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: The current agreed triggering conditions of SPR are sufficient, i.e., no other triggering condition is needed.

Proposal 2: Regarding values of the triggering conditions of SPR, RAN2 to agree the following:

- Define separate thresholds for T310/T312/T304;

- The percentage value is to indicate the ratio of the threshold value (unit: ms) over the signalled T310/T312/T304 value (unit: ms);

- The percentage values are 40%, 60%, 80%, and the percentage value also includes 20% for the threshold for T312.

Proposal 3: The following information should be included in the SPR:

- CGI of the PCell which sent the SPR configuration.

- An indication to indicate the type of PSCell addition/change, i.e., PSCell addition, MN-initiated PSCell change or SN-initiated PSCell change.

Proposal 4: For HO with SN change, the existing SHR and SPR mechanisms are sufficient and no additional work is needed.

Reference

- Chairman_notes of 3GPP TSG RAN WG2 meeting #119bis-e, online

- Chairman_notes of 3GPP TSG RAN WG2 meeting #120

- Chairman_notes of 3GPP TSG RAN WG2 meeting #121

- Chairman_notes of 3GPP TSG RAN WG3 meeting #119bis-e, Online