3GPP TSG-RAN WG2 Meeting #122		R2-2305334

Incheon, Korea, May 22-26, 2023

Source:	vivo

Title:          on-demand PRS for PRS bandwidth aggregation

Agenda Item:	7.2.5

Document for:	Discussion and Decision

- Introduction

RAN1 discussed the on-demand PRS for PRS bandwidth aggregation at the previous meetings and made the following agreements:

Agreement (RAN1#112) - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details Agreement (RAN1#112) To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs Conclusion (RAN1#112bis-e) The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

This contribution provides an initial discussion on the on-demand PRS for PRS bandwidth aggregation from RAN2’s perspective.

- Discussion

- 2.1	Background

PRS bandwidth aggregation is to be introduced to achieve higher accuracy for positioning. Specifically,

- The LMF may inform UE that the PRS in two or three different PFLs are aggregated in assistance data.

- The LMF may indicate UE whether to perform joint measurement across aggregated PFLs in request location information.

- The target UE may indicate whether/which measurement is aggregated in provide location information.

At the RAN1#112 bis-e meeting, RAN1 agreed that the PRS aggregation is per TRP basis, and will decide whether it is per PRS resource set or PRS resource in RAN1#113 meeting.

Agreement For PRS bandwidth aggregation across PFLs, select one of the following options in RAN1#113 - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. - Option 3: Per TRP basis and per PRS resource basis. - For each TRP, support new signaling to indicate which PRS resource(s) across PFLs are linked. - For the non-linked PRS resources, no aggregation is assumed even if the conditions are satisfied.

The signaling structure of PRS assistance data is illustrated in Figure 2.1-1. If the PRS aggregation is per PRS resource set, one example is that the PRS resource set #0 and set #6 of TRP 0 can be aggregated. Otherwise, if the PRS aggregation is per PRS resource, one example is that PRS resource #0 of resource set #0 and resource set #6 of TRP 63 can be aggregated.

Figure 2.1-1 Signaling structure of PRS assistance data

Observation 1: The PRS aggregation is per TRP basis, and RAN1 will further decide whether it is per PRS resource set or PRS resource in the RAN1#113 meeting.

In Rel-17, for the parameters of UE-initiated requests of on-demand PRS,

- The number of DL PRS frequency layers is per FR

- The start/end time of DL PRS transmission is per UE

- Other parameters are per positioning frequency layer per FR

Agreement - From RAN1 perspective, for UE-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signalled - per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - per FR - Number of DL PRS frequency layers - per UE - Start/end time of DL PRS transmission - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - UE recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - UE requests to provide the QCL information in the assistance data

Observation 2: The parameters of UE-initiated request of on-demand PRS are per UE, per FR, or per positioning frequency layer.

Based on the above observations, the parameters of UE-initiated request of on-demand PRS are in larger granularity than the PRS aggregation. That is, the discussions in RAN2 can be slightly independent of RAN1 left issues.

Therefore, we propose that RAN2 can have an initial discussion on how to support UE-initiated on-demand PRS request for PRS bandwidth aggregation based on RAN1’s current progress.

Proposal 1: Based on RAN1’s current progress, RAN2 to have an initial discussion on how to support UE-initiated on-demand PRS request for PRS bandwidth aggregation.

- 2.2	Potential solutions

For UE-initiated On-Demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message. The configurations include a list of available On-Demand-DL-PRS-Configuration and each is associated with an ID, the On-Demand-DL-PRS-Configuration is per frequency layer.

-- ASN1START NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ... } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... } DL-PRS-Configuration-ID-r17 ::= SEQUENCE { nr-dl-prs-configuration-id-r17 INTEGER (1..maxOD-DL-PRS-Configs-r17), ... } -- ASN1STOP

When UE would initiate On-Demand PRS, the UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request can be a request for a pre-defined PRS configuration indicated with pre-defined PRS configuration ID(s) or explicit parameter for PRS configuration and may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements.

-- ASN1START NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE { dl-prs-StartTime-and-Duration-r17 DL-PRS-StartTime-and-Duration-r17 OPTIONAL, nr-on-demand-DL-PRS-Information-r17 NR-On-Demand-DL-PRS-Information-r17 OPTIONAL, dl-prs-configuration-id-PrefList-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF DL-PRS-Configuration-ID-r17 OPTIONAL, ... } DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE { dl-prs-start-time-r17 INTEGER (1..1024) OPTIONAL, dl-prs-duration-r17 SEQUENCE { seconds-r17 INTEGER (0..59) OPTIONAL, minutes-r17 INTEGER (0..59) OPTIONAL, hours-r17 INTEGER (0..23) OPTIONAL, ... } OPTIONAL, ... } -- ASN1STOP

To support the on-demand PRS request for PRS bandwidth aggregation, the following solutions can be considered:

Solution 1：Introducing multiple PFLs in the On-Demand-DL-PRS-Configuration to indicate that multiple PFLs associated with the dl-prs-configuration-id can be aggregated.

For this solution, the UE can request PRS aggregation with the dl-prs-configuration-id explicitly.

As some PRS resource parameters (e.g., Periodicity, Bandwidth) across the aggregated PFLs may be different, multiple values of these parameters for different PFLs should be included in one single On-Demand-DL-PRS-Configuration.

Solution 2: Indicating in the On-Demand-DL-PRS-Configuration that the PFL associated with this dl-prs-configuration-id can be aggregated with other PFLs.

For this solution, the UE may send the PRS aggregation request implicitly with a dl-prs-configuration-id and a bandwidth value greater than the associated PFL bandwidth. Or UE may request lists of PFL combinations in the NR-On-Demand-DL-PRS-Request to indicate the requested PFLs to be aggregated explicitly.

Note that the configuration and request of on-demand PRS aggregation may have more combinations. Therefore, we propose:

Proposal 2: To support the on-demand PRS configuration for PRS bandwidth aggregation, the following solutions can be considered:

- Solution 1: Introducing multiple PFLs in the On-Demand-DL-PRS-Configuration to indicate that multiple PFLs associated with the dl-prs-configuration-id can be aggregated.

- Solution 2: Introducing an indication in the On-Demand-DL-PRS-Configuration to indicate that the PFL associated with the dl-prs-configuration-id can be aggregated with other PFLs.

Proposal 3: To support the on-demand PRS request for PRS bandwidth aggregation, the following alternatives can be considered:

- Alt 1: UE may request PRS aggregation using explicit dl-prs-configuration-id.

- Alt 2: Introducing one or multiple lists of PFL combinations in the NR-On-Demand-DL-PRS-Request to indicate the requested PFLs to be aggregated.

- Alt 3: UE may request a bandwidth value that exceeds the PFL bandwidth to request PRS aggregation implicitly.

- Conclusion

Observation 1: The PRS aggregation is per TRP basis, and RAN1 will further decide whether it is per PRS resource set or PRS resource in the RAN1#113 meeting.

Observation 2: The parameters of UE-initiated request of on-demand PRS are per UE, per FR, or per positioning frequency layer.

Proposal 1: Based on RAN1’s current progress, RAN2 to have an initial discussion on how to support UE-initiated on-demand PRS request for PRS bandwidth aggregation.

Proposal 2: To support the on-demand PRS configuration for PRS bandwidth aggregation, the following solutions can be considered:

- Solution 1: Introducing multiple PFLs in the On-Demand-DL-PRS-Configuration to indicate that multiple PFLs associated with the dl-prs-configuration-id can be aggregated.

- Solution 2: Introducing an indication in the On-Demand-DL-PRS-Configuration to indicate that the PFL associated with the dl-prs-configuration-id can be aggregated with other PFLs.

Proposal 3: To support the on-demand PRS request for PRS bandwidth aggregation, the following alternatives can be considered:

- Alt 1: UE may request PRS aggregation using explicit dl-prs-configuration-id.

- Alt 2: Introducing one or multiple lists of PFL combinations in the NR-On-Demand-DL-PRS-Request to indicate the requested PFLs to be aggregated.

- Alt 3: UE may request a bandwidth value that exceeds the PFL bandwidth to request PRS aggregation implicitly.

- Reference