3GPP TSG-RAN WG2 Meeting #122	R2-2305335

Incheon, Korea, 22th May – 26th May, 2023

Source:	vivo

Title:	Discussion on cell DTX-DRX mechanism

Agenda Item:	7.3.2

Document for:	Discussion and Decision

Introduction

According to R2#121bis meeting, the following agreements were made [1]:

Agreements 1. A periodic cell DTX/DRX configuration is explicitly signalled to the UEs. 2. A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling. 3. The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration. 4. As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released. 5. From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details. Ask about feasibility and reliability of using L1 signaling. Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration. Extract a few key benefits of dynamic signaling from email discussion and online discussions 6. As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period 7. As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods 8. As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic 9. (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH. For the FFS case there may be some exceptions. 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions

In this contribution, we will further discuss the alignment between UE DRX (C-DRX) and cell DTX, and the leftover issues for behaviors during cell DTX/DRX non-active period.

Discussion

Alignment between C-DRX and cell DTX

Definition of ‘alignment’

According to TR 38.864 [2], RAN2 agreed during the SI phase that ‘It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs.’. At this stage with some progress of cell DTX/DRX, we would like to revisit how it is beneficial to align C-DRX with Cell DTX and DRX alignment among multiple UEs for better understanding of future discussion.

C-DRX Alignment among multiple UEs

In legacy C-DRX, the gNB can configure the C-DRX pattern among UEs distributedly for scheduling balance from time domain. Upon the case for cell DTX activation, the cell load is relatively low and thus C-DRX pattern among UEs can be aligned with best effort to achieve a common duration on which some channels/signals can be dropped for NES gain.

- C-DRX alignment among multiple UEs is beneficial in the sense to achieve as long a common duration as possible for the cell to enable cell DTX and potentially achieve NES gain.

C-DRX Alignment with cell DTX

Since C-DRX is a feature for UE power saving, it was specified from the UE’s perspective that the UE can drop some PDCCH monitoring occasionally except for PDCCH for SR/RACH/retransmission. In other words, according to the legacy procedure, the gNB always has the freedom to choose whether to schedule new transmissions or not. With this agreement, when we revisit the conclusion ‘It is beneficial to align UE DRX with Cell DTX’, the benefit is no long about NES gain. Rather, alignment between C- DRX with Cell DTX seems to be meant for trying to minimize the cell DTX impact to scheduling UE’s transmission. That is, if C-DRX pattern is not aligned with cell DTX pattern in some sense, the UE may be hardly scheduled by the serving cell.

- ‘Alignment between UE DRX and cell DTX’ is beneficial not in the sense for NES gain. Rather, it is beneficial for minimizing the cell DTX impact to scheduling UE’s transmission.

To help further progress with the alignment issue, RAN2 needs to first reach a consensus about how to understand ‘alignment’. According to the R2#121bis-e agreement, the gNB does not schedule UE-specific dynamic grants/assignments for new transmissions during Cell DTX non-active period [1]:

Agreements 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions

Therefore, there is no need to limit the C-DRX on-duration to be located within cell DTX on-duration, since UE DRX onDurationTimer is only part of C-DRX active time. We propose that it is enough for RAN2 to move forward with the understanding that alignment between C-DRX and cell DTX means the on-duration of both mechanism is at least partially overlapped with each other. Further restriction on the alignment concept is not necessary.

- Alignment between C-DRX and cell DTX means that the on-duration of C-DRX is at least partially overlapped with the on-duration of cell DTX.

How to align C-DRX with cell DTX

Based on previous discussion among companies, we observe two options about whether to align C-DRX with cell DTX:

- Opt. 1: It is up to the gNB implementation on whether aligning C-DRX with cell DTX or not, i.e. the gNB can implicitly align C-DRX with cell DTX will legacy RRCReconfiguration signalling;

Proponents of Opt.1 think the gNB/UE behavior will be eventually specified during cell DTX non-active period, and hence it is not necessary to align C-DRX pattern with cell DTX. UE can determine its PDCCH monitoring behavior in the intersection duration of C-DRX and cell DTX.

Opt.1 has no spec impact to legacy C-DRX configuration. The potantial problems with Opt. 1 may include: If the cycles of C-DRX and cell DTX are not multiple to each other, the intersection duration can be aperiodic for each original cycle; If the cycles of C-DRX and cell DTX are multiple to each other, and if the gNB does not reconfigure C-DRX to be aligned with cell DTX, the intersection duration of C-DRX and cell DTX may be empty. We suggest companies to discuss whether the potantial problems are concerning.

- Opt. 2: The gNB explicitly configures/signals parameters for the C-DRX pattern that is aligned with cell DTX to the UE, e.g. the gNB configures C-DRX cycle/onDurationTimer/offset or a whole set of a second C-DRX configuration parameters along with cell DTX configuration to align C-DRX with cell DTX, and activate/deactivate the use of such parameters when cell DTX is activated/deactivated;

Proponents of Opt.2 think it is necessary to align C-DRX with cell DTX, and there are some spec impacts. From our perspective, we observe pros and cons for Opt. 2:

Pros

(1) If L1 common signalling is supported, the gNB can align C-DRX among UEs with cell DTX all at once from the second time and on without one-to-one RRC reconfiguration for C-DRX (i.e. for the first time, aligned C-DRX is activated along with cell DTX, since RAN2 has agreed cell DTX is implicitly activated immediately when cell DTX is configured);

(2) Aligning C-DRX with cell DTX from on-duration and/or cycle perspective helps to realize periodical and predictable PDCCH scheduling/monitoring behaviors;

Cons

(1) The legacy C-DRX configuration is per DRX group for serving cells. However, it was agreed during SI phase that cell DTX operation is per serving cell. An awkward situation may occur if the gNB wants to configure unaligned cell DTX pattern for different serving cells. In such situation, how C-DRX can be aligned with cell DTX. Possible way-out is to either change legacy per DRX group C-DRX behavior (which will involve intensive discussion and delay the WI progress) or we confine how the gNB realizes per serving cell’s cell DTX.

(2) RAN2 has not discussed or agreed yet whether to support L1/L2 signalling. If no L1/L2 signalling is supported, it makes no sense to support explicit signalling for C-DRX alignment. The gNB can simply reconfigure C-DRX along with cell DTX configuration, i.e. the C-DRX is reconfigured to be aligned with cell DTX, and cell DTX is activated (per RAN2 agreement that cell DTX is activated immediately once configured by RRC) at the same time.

As a complicated issue analyzed above, we propose RAN2 to further discuss the two options about whether to align C-DRX with cell DTX:

- RAN2 to discuss the following two options about whether to align C-DRX with cell DTX:

- Option 1: It is up to the gNB implementation on whether aligning C-DRX with cell DTX or not, i.e. the gNB can implicitly align C-DRX with cell DTX will legacy RRCReconfiguration signalling;

- Option 2: The gNB explicitly configures/signals parameters for the C-DRX pattern that is aligned with cell DTX to the UE, e.g. the gNB configures C-DRX cycle/onDurationTimer/offset or a whole set of a second C-DRX configuration parameters along with cell DTX configuration to align C-DRX with cell DTX, and activate/deactivate the use of such parameters when cell DTX is activated/deactivated.

Behaviors during cell DTX/DRX non-active period

SR

During the last meeting, the following agreements are made [1]:

Agreements 6. As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period 7. As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods 8. As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

SR can be used for different purpose, including requesting resources for high priority traffic. In NR, with type-1/type-2 CG, the gNB can also well serves scenraios with low latency traffic such as URLLC. Since RAN2 has agreed that UE does not transmit on CG occasions during Cell DRX non-active periods, there is no point to support SR transmission during cell DRX non-active periods for high priority traffic as they are similar from this perspective. Companies supporting configurable SR during cell DRX non-active period thinks it is necessary to keep SR for critical service, e.g. emergency calls. However, opponents also pointed out it would require the gNB to align SR for all UEs and RACH can be used instead anyway. Taking all the comments into consideration, we think it is simpler to go with no SR transmission during cell DRX non-active period.

- RAN2 to confirm: As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, and SR is not configurable to be transmitted during cell DRX non-active periods.

PDCCH for new transmission

This section treats the FFS question in the agreement below during last meeting [1]:

Agreements 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions

RACH

In R2#121, the following agreement is made[3]:

Agreements 1. There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs 2. Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX). No further enhancements for CBRA and CFRA will be pursued.

CFRA

According to TS 38.321 [4], the following procedure is about UE monitoring PDCCH when using CFRA resource:

1> if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity: 2> if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network: 3> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6]. 2> else: 3> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission. 2> monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running. 1> else: 2> if the Random Access Preamble was transmitted on a non-terrestrial network: 3> start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6]. 2> else: 3> start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission. 2> monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

In other words, the UE monitors PDCCH identified by the C-RNTI when using CFRA resource for BFR, and monitors PDCCH identified by the RA-RNTI when using CFRA resource for PDCCH order and HO. Therefore, we mainly focus on the case where CFRA is triggered for BFR. Since the UE needs to timely know whether it recovers from beam failure and switches to the right beam, it is necessary for the UE to monitor PDCCH identified by the C-RNTI for BFR, regardless whether this PDCCH is for new transmission or retransmission.

- For CFRA triggered by BFR, UE needs to monitor PDCCH identified by the C-RNTI to timely know whether it recovers from beam failure and switches to the right beam regardless whether this PDCCH is for new transmission or retransmission.

CBRA

According to TS 38.321 [4], the following procedure is about how UE determines the contention resolution is successful:

1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in Msg3: 3> if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Contention Resolution successful; 4> stop ra-ContentionResolutionTimer; 4> discard the TEMPORARY_C-RNTI; 4> consider this Random Access procedure successfully completed.

From this procedure, we can observe that it is necessary for the UE to receives a UL grant for a new transmission to consider the Contention Resolution successful.

- For CBRA, UE needs to receives a UL grant for a new transmission to consider the Contention Resolution successful.

Since we have agreed that RACH can be transmitted for CONNECTED UE during cell DTX/DRX non-active period, the gNB behavior for scheduling new transmission should not impact RACH. Furthermore, R18 IDLE UE and legacy UE will perform RACH anyway, it makes no sense that the gNB can schedule PDCCH for R18 IDLE UE and legacy UE, but not R18 CONNECTED UE.

In sum, for both CBRA and CFRA, we propose that the gNB can schedule UE-specific dynamic grants/assignments for new transmissions for the RACH procedure  during the cell DTX non-active period.

- The gNB can schedule UE-specific dynamic grants/assignments for new transmissions for the RACH procedure during the cell DTX non-active period; and the UE monitors PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period.

retransmission

During the last meeting, companies discussed whether PDCCH for new transmission can be allowed for retransmission occaion, e.g. when C-DRX retransmissionTimer is running, during cell DTX non-active period. Since the UE cannot distinguish whether a PDCCH is for new transmission or retransmission until it decodes the NDI field, it is meaningless to further restriction PDCCH scheduling behavior for this case. It can be up to the gNB implementation whether to schedule PDCCH for new transmission on the retransmission occasions.

- The gNB can schedule UE-specific dynamic grants/assignments for new transmissions on the retransmission occasions during the cell DTX non-active period, e.g. when C-DRX retransmissionTimer is running; and the UE monitors PDCCH for dynamic grants/assignments for new transmissions on the retransmission occasions during Cell DTX non-active period.

SR

Whether SR can be configured to be allowed to transmit during cell DRX non-active period is still under discussion. Regardless whether it is allowed or not, there is anyway a case where the UE transmits an SR at the end of a cell DTX on-duration. In legacy C-DRX procedure, the UE keeps C-DRX active time as long as the SR is still pending. We are not certain whether this behavior should be aligned from the gNB side. If so, the NES gain can be potentially impacted.

- In C-DRX, the UE keeps C-DRX active time as long as an SR is pending.

- RAN2 to discuss whether an SR extends cell DTX active time as long as the SR is pending at least for PDCCH scheduling new transmission.

PDCCH for retransmission

Allowing retranmission for dynamic scheduling during cell DTX non-active time does not cause severe NES gain loss as it is corner case that the gNB schedules PDCCH for new transmission at the end of cell DTX on-duration and the UE happens to errorly decode the PDCCH. Also, the gNB can rely on implementation to avoid to schedule dynamic grants/assignments with PDCCH for new transmission at the end of cell DTX on-duration to avoid potential retransmissions during cell DTX non-active period.

Companies also mentioned about retranmission for SPS. Since RAN2 has agreed UE doesn’t monitor SPS occasions during Cell DTX non-active period, it will be the similar case as retranmission for dynamic scheduling, i.e. it is corner case that UE needs to monitor retranmission for SPS, which only happens when UE errorly decodes SPS for new transmission at the end of a cell DTX on-duration.

In sum, we propose that the gNB can schedule dynamic grants/assignments with PDCCH for retransmission during cell DTX non-active period.

- The gNB can schedule dynamic grants/assignments with PDCCH for retransmission during cell DTX non-active period.

Conclusion

Based on the discussion, we have the following observation and proposals:

Alignment between C-DRX and cell DTX

- C-DRX alignment among multiple UEs is beneficial in the sense to achieve as long a common duration as possible for the cell to enable cell DTX and potentially achieve NES gain.

- ‘Alignment between UE DRX and cell DTX’ is beneficial not in the sense for NES gain. Rather, it is beneficial for minimizing the cell DTX impact to scheduling UE’s transmission.

- Alignment between C-DRX and cell DTX means that the on-duration of C-DRX is at least partially overlapped with the on-duration of cell DTX.

- RAN2 to discuss the following two options about whether to align C-DRX with cell DTX:

- Opt. 1: It is up to the gNB implementation on whether aligning C-DRX with cell DTX or not, i.e. the gNB can implicitly align C-DRX with cell DTX will legacy RRCReconfiguration signalling;

- Opt. 2: The gNB explicitly configures/signals parameters for the C-DRX pattern that is aligned with cell DTX to the UE, e.g. the gNB configures C-DRX cycle/onDurationTimer/offset or a whole set of a second C-DRX configuration parameters along with cell DTX configuration to align C-DRX with cell DTX, and activate/deactivate the use of such parameters when cell DTX is activated/deactivated;

Behaviors during cell DTX/DRX non-active period

Configurable SR

- RAN2 to confirm: As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, and SR is not configurable to be transmitted during cell DRX non-active periods.

PDCCH for new transmission

- For CFRA triggered by BFR, UE needs to monitor PDCCH identified by the C-RNTI to timely know whether it recovers from beam failure and switches to the right beam regardless whether this PDCCH is for new transmission or retransmission.

- For CBRA, UE needs to receives a UL grant for a new transmission to consider the Contention Resolution successful.

- The gNB can schedule UE-specific dynamic grants/assignments for new transmissions for the RACH procedure during the cell DTX non-active period; and the UE monitors PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period.

- The gNB can schedule UE-specific dynamic grants/assignments for new transmissions on the retransmission occasions during the cell DTX non-active period, e.g. when C-DRX retransmissionTimer is running; and the UE monitors PDCCH for dynamic grants/assignments for new transmissions on the retransmission occasions during Cell DTX non-active period.

- In C-DRX, the UE keeps C-DRX active time as long as an SR is pending.

- RAN2 to discuss whether an SR extends cell DTX active time as long as the SR is pending at least for PDCCH scheduling new transmission.

PDCCH for retransmission

- The gNB can schedule dynamic grants/assignments with PDCCH for retransmission during cell DTX non-active period.

References

- RAN2 #121bis-e meeting minutes

- TR 38.864, V18.1.0

- RAN2 #121 meeting minutes

- TS 38.321, V17.4.0