3GPP TSG-RAN WG2 Meeting #122                 	R2-2305338

Incheon, 22th – 26th May, 2023

Agenda Item:	7.3.5

Source: 	vivo

Title:         	Conditional handover enhancement for network energy saving

Document for: 	Discussion and Decision

- Introduction

One objective of the Rel-18 network energy saving WID [1] is provided below:

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

For this objective, RAN2 has made the below agreements so far.

Agreements: - Study whether CHO enhancements are needed for the purpose of turning off the cell - Continue discussing CHO in the context of different NES techniques. - RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”. FFS further details - For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode” - As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration. FFS what trigger is used for execution of CHO

This paper will continue discussing the potential CHO enhancements in case source/target cell is in NES mode, based on the above agreements.

- Discussion

2.1 Scenario 1: Source cell is in NES mode

RAN2 has agreed the baseline procedure for CHO enhancement in case of source cell in NES mode. As shown in the below Figure 1, when the source cell starts NES mode in Time B, the network sends the CHO configuration in Time A which is earlier than Time B. As legacy, the UE starts CHO evaluation upon receiving the CHO configuration. Some details are still unclear.

Figure 1: Baseline procedure

What trigger is used for the execution of CHO before Time B?

T1 Event

It is difficult for the network to configure a proper T1 event as the network may decide to turn off the source cell at any time based on the traffic fluctuation. In other words, Time B is unpredictable. So, T1 event is not suitable for triggering CHO execution for network energy saving case.

Observation 1: It is difficult for the network to configure a proper T1 event as a trigger of CHO execution since the source cell may decide to enter NES mode at any time.

A3/5 Events

From our understanding, A3/5 event is used for UE mobility. So, basically the source cell may not configure A3/5 event for the UEs in the cell centre. Even if A3/5 event is configured, the events may not be satisfied if the quality of source cell is still good enough due to no mobility at the UE before the source cell enters the NES mode. So, A3/5 events cannot ensure to trigger CHO execution before the source cell enters NES mode.

Figure 2: A3/5 event based CHO

Observation 2: A3/5 events cannot ensure to trigger CHO execution before the source cell enters NES mode.

A4 Event

The source cell that is going to enter NES mode can be fully overlaid by one candidate cell or partially overlaid by multiple candidate cells.

- If the source cell is fully overlaid by one candidate cell, the UE can handover to this candidate cell without quality evaluation. Therefore, the network may not configure A4 event for triggering of CHO execution. The question is whether to trigger the CHO execution upon CHO configuration in this case and its difference with the traditional HO.

- If the source cell is partially overlaid by multiple candidate cells, the UE needs to evaluate the quality of the candidate cells. The UE can select the cell satisfied A4 event, or the strongest cell. As the strongest cell may have low signal quality, the UE may suffer HOF if it selects this cell. So, it is more reasonable to select the cell satisfying A4 event. Since A4 event maybe satisfied shortly upon CHO configuration and then the UE will execute CHO, the question is whether to start A4 event based CHO evaluation upon the configuration of CHO.

Basically, the source cell that is going to enter NES mode is a capacity booster cell. It is beneficial for the UE to maintain the high throughput in the source cell until the source cell enters NES mode.

Observation 3: Using A4 event as a trigger of CHO execution will sacrifice UE’s throughput in capacity boost source cell.

An explicit signalling w/o A4 event

Given above, it is preferred to introduce an explicit signaling to trigger CHO execution or A4 event based CHO evaluation. Specifically, if A4 event is not configured, the explicit signaling is used to trigger CHO execution. If A4 event is configured, the explicit signaling is used to trigger CHO evaluation based on A4 event.

Here are three examples for the proposed explicit trigger.

Figure 3a: An explicit signaling to trigger CHO execution

Figure 3b: An explicit signaling to trigger A4 based CHO evaluation

Figure 3c: An explicit signaling to trigger CHO evaluation from A3/5 event to A4 event

Thus, the below proposals are given:

Proposal 1: An explicit signaling is introduced to trigger:

- CHO execution if A4 event is not configured;

- A4 event based CHO evaluation if A4 event is configured.

Proposal 2:  If A4 event based CHO evaluation is triggered, the UE stops A3/A5 event based CHO evaluation if A3/A5 event is used previously.

Proposal 3: The UE does not start A4 event based CHO evaluation upon when receiving the CHO configuration includes A4 event.

The signalling of the explicit trigger for CHO:

Considering RAN1 is not involved in this objective, so L1 signalling is precluded. In our view, the following options for sending the explicit trigger can be considered:

- Option 1: the explicit trigger is sent via dedicated MAC CE.

- Option 2: the explicit trigger is included in short message which is sent on common POs.

- Option 3: the explicit trigger is the NES bar introduced in SIB1. Before turning off the source cell, the network may bar RRC IDLE/INACTIVE to camp on this cell, so it is possible to reuse this bar info to trigger the RRC CONNECTED UEs to perform CHO.

From our understanding, all options can work. The benefit of option 1 is low latency. Option 2 and 3 can reduce some signaling overhead, where option 2 does not require the UE to receive SIB for receiving the trigger of CHO, and option 3 does not need to introduce any other indication in SIB. Thus, we have the following proposal:

Proposal 4: For the explicit signalling for CHO, RAN2 to consider the below options:

- Dedicated MAC CE;

- Short message sent on common PO;

- Reuse the cell bar introduced for NES purpose.

How to handle the case when no suitable candidate is found, and the source cell is about to sleep?

In mobility scenario, candidate cells are configured based on UE mobility and network local strategy. In the current specification, CHO configuration can be modified, which means the network can reconfigure the candidate cells to the UE due to either network local policy or the UE mobility based on the measurement reporting. However, in the network energy saving scenario, if the network decides to turn off  the source cell, the source cell may have already activated cell DTX/DRX, so the network may not be able to receive the MR or reconfigure the candidate cells timely. Then, it is possible that the UE cannot find a fulfilled candidate cell after receiving the trigger of A4 event based CHO evaluation.

Figure 4: When no suitable candidate cell is found, and the source cell is about to sleep

In this case, the UE shall notify the source cell within a certain time, instead of fallback to RRC re-establishment, since the source cell is still there and the quality is good enough. After receiving this notification (e.g., RACH), the source cell can postpone to activate NES mode (e.g. turn off the cell), or obtain the MR from the UE and send traditional HO command to the UE. If the source cell does not receive any notification within a certain time, the network can decide to turn off the source cell without waiting for the notification from the target gNB(s). Thus, we have the following proposal:

Proposal 5: After receiving the explicit signaling to trigger CHO, if the UE cannot find a candidate cell satisfies A4 event, the UE should notify the source cell within a certain time.

2.2 Scenario 2: Target cell is in NES mode

In this scenario, when CHO is configured at the UE, and if multiple candidate target cells satisfy the execution condition, currently it is up to UE implementation to select one of them to perform handover. If network energy saving technique(s) is implemented, it is possible that the satisfied candidate target cell may plan to enter NES mode. In this case, if the UE selects the target cells via UE implementation, the UE may handover to the NES cell. From the UE’s perspective, its performance maybe impacted if it is switched to this NES cell. From the network’s perspective, the network may expect the UEs to select the NES cell only when there is no available normal cell.

Figure 4: RRC re-establishment to the NES cells

In the last RAN2 meeting, some companies think the network should not configure NES cells as candidate cells in CHO. However, we think this means network energy saving and UE fast mobility cannot be achieved at the same time. The Figure 4 shows an example. If the cell 1 activates the cell DTX/DRX, it is possible that cell 1 is  the best target cell of one UE. However, if the network does not configure cell 1 to this UE because this cell enters NES mode, then the UE will suffer from RLF and perform RRC re-establishment. Since the UE can still access to the cell 1 which is in cell DTX / DRX operation, the handover results are the same but longer delay occurs at the UE if the cell 1 is not configured as an candidate cell in the CHO configuration.

So, the NES cells can be candidate cells of the UE and the UE should be able to know the NES information of the candidate cells for better selection. There are two candidate options for UE to select target cells:

- Option 1: the NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select;

- Option 2: The NES mode/priority/NES capability of candidate cells are configured by the network, and the UE selects the target cell based on some specified rules.

From our understanding, option 1 can provide more flexibility for the UE and has minimum spec impact, and hence preferred. Thus, we have the following proposal:

Proposal 6: The NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select based on the NES mode and the signal quality of the candidate cells.

- Conclusion

In this paper, the following observation and proposals are given:

For scenario 1 of CHO enhancement:

Observation 1: It is difficult for the network to configure a proper T1 event as a trigger of CHO execution since the source cell may decide to enter NES mode at any time.

Observation 2: A3/5 events cannot ensure to trigger CHO execution before the source cell enters NES mode.

Observation 3: Using A4 event as a trigger of CHO execution will sacrifice UE’s throughput in capacity boost source cell.

Proposal 1: An explicit signaling is introduced to trigger:

- CHO execution if A4 event is not configured;

- A4 event based CHO evaluation if A4 event is configured.

Proposal 2:  If A4 event based CHO evaluation is triggered, the UE stops A3/A5 event based CHO evaluation if A3/A5 event is used previously.

Proposal 3: The UE does not start A4 event based CHO evaluation upon when receiving the CHO configuration includes A4 event.

Proposal 4: For the explicit signalling for CHO, RAN2 to consider the below options:

- Dedicated MAC CE;

- Short message sent on common PO;

- Reuse the cell bar introduced for NES purpose.

Proposal 5: After receiving the explicit signaling to trigger CHO, if the UE cannot find a candidate cell satisfies A4 event, the UE should notify the source cell within a certain time.

For scenario 2 of CHO enhancement:

Proposal 5: The NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select based on the NES mode and the signal quality of the candidate cells.

- References

[1] RP-223540, Network energy savings for NR, Huawei.