3GPP TSG-RAN WG2 Meeting #122	R2-2305339

Incheon, Korea, 22th May – 26th May, 2023

Source:	vivo

Title:	discussion on UL tx switching

Agenda Item:	7.25.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the leftover issues in RAN2 for UL tx switching.

Discussion

UE capability reporting

Switching period reporting for 1T-1T

During RAN4 #106 meeting, the following agreement was made according the LS[1]:

Issue 2: 1Tx-1Tx switching case In RAN4 #106, RAN4 discussed the scenario of 1Tx-1Tx switching, i.e., the UL carriers in both bands before and after switching are capable of one transmit antenna connector, and agreed to apply the same length of switching period for 1Tx-1Tx switching and 1Tx-2Tx switching.

It has been agreed to apply the same length of switching period for 1Tx-1Tx switching and 1Tx-2Tx switching in RAN4’s LS. However, how to report the 1Tx-1Tx switching period needs clarification. One understanding is that a single parameter is reported (e.g.: uplinkTxSwitchingPeriod-r16 [2]) applying for both 1Tx-1Tx switching and 1Tx-2Tx switching. Another understanding is that two parameters (e.g.: uplinkTxSwitchingPeriod-r16, uplinkTxSwitching1T1T-r18) are reported separately applying for 1Tx-2Tx switching and 1Tx-1Tx switching respectively, but the two parameters should have the same value.

ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700 Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 ... - uplinkTxSwitchingPeriod-r16 indicates the length of UL Tx switching period of 1Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall not report the value n210us for EN-DC band combinations. n35us represents 35 us, n140us represents 140us, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod2T2T-r17... - uplinkTxSwitching-DL-Interruption-r16 ...

In our view, there is no additional benefit to introducing a new parameter to indicate 1Tx-1Tx switching period. For simplicity, the legacy parameter uplinkTxSwitchingPeriod-r16 is applied for both 1Tx-1Tx switching and 1Tx-2Tx switching.

Proposal 1: The legacy parameter uplinkTxSwitchingPeriod-r16 is applied for both 1Tx-1Tx switching and 1Tx-2Tx switching.

Fallback of Rel-18 Tx switching to Rel-16/17 Tx switching

During RAN4 #106bis meeting, the following agreement was made according the LS [2]:

RAN4 has also discussed the fallback of Rel-18 Tx switching to Rel-16/17 Tx switching, and reached the following agreement and understanding: - The indication of supported switching band combinations of the UE capability can also imply the capabilities of the fallbacks in accordance with the current 38.331 fallback framework. - Rel-18 Tx switching capability will include all sufficient capabilities for two-band switching. In addition, UE may also declare two-band Tx switching capabilities with Rel-16 and/or Rel-17 capabilities. If UE is configured for two-band Tx switching only and UE declares those Rel-16/17 capabilities, then Rel-16/17 capabilities apply for this two band Tx switching.

For the first bullet “The indication of supported switching band combinations of the UE capability can also imply the capabilities of the fallbacks in accordance with the current 38.331 fallback framework.”, we think there are 2 alternatives to meet RAN4’s understanding:

- Alt. 1: The UE reports supported Rel-18 tx switching band combinations, and optionally reports supported Rel-16/17 tx switching band combinations for fallback capability reporting;

- Alt. 2: The UE should report both supported Rel-16/17 tx switching band combinations and supported Rel-18 tx switching band combinations;

Taking the second bullet of the RAN4 agreement into consideration, i.e. “If UE is configured for two-band Tx switching only and UE declares those Rel-16/17 capabilities, then Rel-16/17 capabilities apply for this two band Tx switching.”, we think Alt.1 is more aligned with RAN4 understanding of fallback capability reporting and propose to go with Alt.1.

Proposal 2: For supporting Rel-18 tx switching fallback capability reporting, the UE reports supported Rel-18 tx switching band combinations, and optionally reports supported Rel-16/17 tx switching band combinations for fallback capability reporting.

Conclusion

Based on the discussion, we have the following proposals:

Proposal 1: The legacy parameter uplinkTxSwitchingPeriod-r16 is applied for both 1Tx-1Tx switching and 1Tx-2Tx switching.

Proposal 2: For supporting Rel-18 tx switching fallback capability reporting, the UE reports supported Rel-18 tx switching band combinations, and optionally reports supported Rel-16/17 tx switching band combinations for fallback capability reporting.

References

- R4-2303507, LS on Rel-18 Multi-carrier enhancement for NR

- TS 38.306, V17.4.0

- R4-2306623, LS on Rel-18 Tx switching across 3/4 bands