3GPP TSG-RAN WG2 Meeting #122                                     R2-2305343

Incheon, Korea, May 22-26, 2023

Source: 			OPPO

Title: 	Further discussion on sidelink positioning

Agenda item:		7.2.2

Document for: 	Discussion and Decision

- Introduction

In the RAN2#121 meeting, discussion on sidelink positioning has been made, and following agreement have been made on the aspects such as positioning protocol, signaling procedure, etc.

Agreement:

- PC5-U is used for transport of SLPP

- With respect to the overall signaling procedure for PC5-only positioning (including at least IC and OOC; FFS if there are differences for PC), it is proposed to agree that the sidelink positioning procedure comprises the following series of steps as a baseline, between the LMF/positioning server UE/NG-RAN/candidate Anchor UE(s) and Target UE(s):

- Triggering event

- Sidelink positioning capability exchange

- Sidelink positioning assistance data transfer

- SL Positioning Request Location Information

- Measurement of SL-PRS

- Location calculation

- SL Positioning Provide Location Information

Some steps may have dependencies on SA2 and can be revisited in this light. The order is subject to further discussion. FFS if discovery and selection of anchor UEs and/or server UE are part of the positioning layer in RAN2 scope. LS to SA2 to ask for confirmation and guidance on the SA2 aspects.

- RAN2 do not intend to discuss assistant UE functionality in Rel-18.

To be indicated in the LS to SA2 in discussion [409].

Direct ranging between two UEs with the server providing assistance to both sides is not precluded.

- RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.

Subsequently, in the last RAN2 #121bis-e meeting, further agreements have been made on sidelink positioning:

- LS on group positioning is postponed for a reply from next meeting.

WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning or multiple UE pairs) (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.

WA: at least part of the group management for group positioning is performed at upper/application layer.

- Sidelink Positioning Architecture in Figure 1-4 on R2-2304301 is taken as baseline in TS 38.305 for further discussion.

RAN2 understanding is that there is no impact to LTE specs from this objective.

- Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2.  Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor.

- Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario.

- Anchor UE selection can be included in the sidelink positioning procedures at least for out of coverage scenario.

- RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles. FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

- The UE role information is indicated in the discovery SLPP metafield. FFS if this applies to both discovery modes and which messages.

In this paper, we would like to present our views on supporting the sidelink positioning in NR from the perspective of architecture, protocol stack, signaling procedure.

- Discussion

- 2.1 support the sidelink positioning in the in-coverage scenario

Generally, after the RAN2#121 meeting, there are two options on table regarding how to support sidelink positioning in the IC scenario:

- Option 1: making the LMF as one of the endpoints of the SLPP

- Option 2: SLPP msg to be carried in the LPP container

Conventionally, the UE needs to support the LPP protocol for the Uu based positioning, and naturally, to further support the Uu+PC5 positioning, SLPP msg could be simply carried in the LPP container. On the other hand, with such option chosen, for the PC5 only positioning, if the UE needs to directly exchange msgs with the LMF for performing the PC5 only positioning, it is necessary for UEs to support at least the LPP container function as the prerequisite condition, which may become a burden for the PC5-only enabled UEs.

From our point of view, a unified protocol solution for the Uu+PC5 positioning and the PC5-only positioning method should be pursued. Decoupling the SL positioning/ranging function from the Uu positioning so that UEs could only choose to support the SL positioning/ranging functionality but not the Uu positioning may not be the demand of majority of UE types. In addition, even if there exist some of UE types only demanding the SL ranging functionality, for example, in the V2X scenarios for collision avoidance, it is not hard for them to only support the basic LPP container functionality rather than the whole LPP protocol. As a result, we are keen to support Option 2

Proposal 1: RAN2 to agree to support transmitting the SLPP msg in the LPP container for the PC5-only and PC5+Uu positioning in the IC and PC scenarios.

- 2.2 Combination of Uu- and PC5-based positioning

As well known, the DL-PRS/SRSp propagation path between the UE and the gNB may be blocked or scattered to a large extent in the urban area, which causes NLOS complicated multipath and therefore degrades of the positioning accuracy. However, it should be noted that whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Observation 1: whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Such observation leads to two modes of adding PC5 based positioning for further calibration of the Uu based positioning result:

  - Option 1: UE/LMF autonomously triggers PC5 based positioning during the Uu positioning session, potentially based on pre-configured condition, e.g., detection of NLOS transmission of the DL-PRS/SRSp, low RSRPP of the received DL-PRS/SRSp, etc.

  - Option 2: LMF triggers PC5+Uu based positioning after the Uu positioning session is ended. The decision of triggering further PC5 positioning to calibrate the Uu based positioning could be a result of the bad Uu positioning result, i .e., low positioning accuracy or the integrity of the positioning result could not be guaranteed.

Obviously, Option 2 consumes more time than Option 1 to obtain a satisfied positioning result, due to the fact that new positioning session embracing PC5 based positioning signalling flow has to be triggered, and the Uu positioning capability and reference signal configuration needs to be transferred again between the UE and the gNB during such new positioning session.

Observation 2: Option 2 (LMF triggering PC5+Uu based positioning in a new positioning session) may consume more time than Option 1 (autonomously triggering PC5 based positioning based on some preconfigured condition) to obtain a satisfied positioning result, since another new positioning session has to be triggered for the option 2, and the Uu positioning capability and reference signal configuration may need to be transferred again between the UE and the gNB during such new positioning session.

- 2.3 Consideration on sidelink positioning signalling procedure

Conventionally, in the Uu positioning procedure, three types of location request have been introduced as follows:

- Mobile Originated Location Request (MO-LR)

- Mobile Terminated Location Request (MT-LR)

- Network Induced Location Request (NI-LR)

As indicated in the [1], the UE, the LCS client, the AF external to or internal to a serving PLMN, or the server AMF could initialize the location request and subsequently the related positioning signaling procedure. For the sidelink positioning, other than directly triggered by the target UE itself, LCS client or the server AMF, as indicated in the above section 2.2, we think it is also necessary to allow the LMF or the target UE to trigger the sidelink positioning signaling procedure upon some conditions such as the deteriorated Uu positioning result, NLOS DL-PRS/SRS positioning measurement result, etc., are detected in the on-going Uu based positioning session. In addition, to motivate the autonomous triggering of the sidelink positioning, sidelink positioning triggering events could be defined and pre-configured towards the target UE. For the Option 1, if it is LMF to trigger using PC5 based positioning method to calibrate the Uu based positioning result, then no RAN2 spec impact is foreseen. On the other hand, if it is UE to trigger using PC5 based positioning method, the criteria as mentioned in the section 2.2 should be defined by the network firstly and transmitted towards the UE as the pre-configured condition, as indicated in the following exampling figure.

Figure 1: an exampling msg flow for using the SL positioning to calibrate the Uu positioning result.

Proposal 2: RAN2 to agree that target UE could autonomously trigger the sidelink positioning to calibrate the Uu positioning result.

Proposal 3: RAN2 to agree that sidelink positioning triggering event could be defined and pre-configured towards the target UE.

Generally, the capability transfer and discovery procedure should be cooperated for the choice of the anchor UE and location server UE. Specifically, the discovery procedure should play the role of coarse filtration at first, e.g., according to the UE role indication, UEs not able to take the task of anchor UE could be excluded from the capability transfer procedure in the following positioning session. In this way, only limited set of UEs should be involved in the capability transfer procedure, which accelerates the positioning process. In addition, RAN2 should further study which set of information to be included in the discovery msg and which other set of information to be included in the capability exchange msg.

In addition, we think that discovery procedure should be also done by the target UE regardless of the coverage scenario, since SL-PRS measurement should be performed between the target UE and the anchor UEs, and we need to guarantee that candidate anchor UEs have good radio propagation path towards the target UE so that SL-PRS measurement result is sufficiently accurate and stable. Also, if the location server UE is proximity to the target UE, even if the distance between the location server UE and the anchor UE is long, still can we use the target UE to relay the signaling msg between the location server UE and the target UE. Finally, the list of the candidate UEs surviving from the discovery procedure should be transmitted from the target UE towards the LMF or location server UE for the IC or OOC scenario, respectively, for further including them in the subsequent positioning session.

Observation 3: Exhibiting good radio propagation path between candidate anchor UEs and the target UE is good to have sufficiently accurate and stable SL-PRS measurement result.

Observation 4: Letting the target UE to perform SL discovery procedure results in a SL location server UE that is proximity to the target UE, even if the distance between the location server UE and the anchor UE is long, still can we use the target UE to relay the signaling msg between the location server UE and the target UE.

Proposal 5: RAN2 to agree that discovery procedure should be done by the target UE regardless of the coverage scenario

Regarding the last RAN2 meeting left FFS regarding how much information is indicated about anchor UEs (e.g., knowledge of location) in the discovery msg, we think that due to the privacy/security/ concern, it is possible for the anchor UEs to only allow to expose the location information to the LMF or the LMF designated location server UE in coverage but not the location server UE out-of-coverage. In our opinion, such information should be included in the discovery msg also, so that the anchor UEs refusing to expose the location information to the location server UE OOC could be filtered out before initialization of the PC5-based positioning session.

Proposal 6: RAN2 to agree that, besides the anchor UE location information, a flag indicating of the possibility of exposing the location information to the location server UE OOC should be included in the discovery msg also.

Regarding assistance data transfer, in the context of SL positioning, we think it could serve for three purposes. Firstly, if the SL-PRS configuration could be autonomously decided by SL-PRS transmitter UE, it may need to send the SL-PRS configuration to the location server UE for further distribution. Subsequently, the location server could also employ the assistance data transfer msg to distribute the SL-PRS configuration to the SL-PRS monitor UEs for performing further measurement. Thirdly, since it has been agreed in the last RAN2 meeting that the location server UE can be armed with the SL-PRS configuration coordination functionality, the assistance data transfer msg could be employed to send the SL-PRS configuration towards the SL-PRS transmitter UE.

In addition, the location information of anchor UEs could be also included in the assistance data transfer msg to be transmitted towards the location server UE or other UEs for the position calculation.

Overall, SL positioning assistance data transfer msg should include required assistance data for UEs to perform the necessary SL-PRS measurements. As a result, we propose RAN2 that SL positioning assistance data transfer msg could be used in following scenarios, optionally with the location information of the anchor UEs included:

- the SL-PRS transmitter UE sends out its SL-PRS configuration

- the location server UE/LMF transmits the SL-PRS configuration of transmitter UEs to the SL-PRS monitor UEs

- the location server UE/LMF sends the SL-PRS configuration to the SL-PRS transmitter UEs for further transmission.

Observation 4: the SL positioning assistance data transfer msg should include required assistance data for UEs to perform the necessary SL-PRS measurements.

Proposal 7: RAN2 to agree that SL positioning assistance data transfer msg could be used in following scenarios, optionally with the location information of the anchor UEs to be included:

- the SL-PRS transmitter UE sends out its SL-PRS configuration

- the location server UE/LMF transmits the SL-PRS configuration of other transmitter UEs to the SL-PRS monitor UEs

- the location server UE/LMF sends the SL-PRS configuration to be used to the SL-PRS transmitter UEs.

Next, similar to the Uu based positioning, the SL Positioning Location Information Request should tell the SL-PRS monitor UE which kind of SL-PRS measurement result is needed, which is related to the positioning method chosen by the location server UE. Bearing in mind that SL-AOA, SL-TDOA and SL-RTT have been agreed in the WID as the candidate sidelink positioning method, we kindly propose RAN2 to at least include following three positioning method related IEs in the SLPP RequestLocationInformation msgs:

- SL-AOA-RequestLocationInformation

- SL-TDOA-RequestLocationInformation

- SL-RTT-RequestLocationInformation

Once the SL-PRS monitor UE receives the SLPP RequestLocationInformation msg, it can perform the SL-PRS measurement corresponding to the demanded positioning method and report the related positioning measurement result to the location server.

Proposal 8: RAN2 to agree that at least following three positioning method related IEs should be included in the SLPP RequestLocationInformation msgs:

- SL-AOA-RequestLocationInformation

- SL-TDOA-RequestLocationInformation

- SL-RTT-RequestLocationInformation

Subsequently, after performing the SL positioning measurement, the SL-PRS monitor UE or the UE taking the task of position calculation should send back the positioning measurement result or position estimates to the location server. As a result, we propose at least following IEs should be at least included in the SLPP ProvideLocationInformation msg:

- commonIEsProvideLocationInformation (including the position estimate result or the ranging result)

- SL-Multi-RTT-ProvideLocationInformation

- SL-TDOA-ProvideLocationInformation

- SL-AOA-ProvideLocationInformation

Proposal 9: RAN2 to agree at least following IE should be included in the SLPP ProvideLocationInformation msg at least:

- commonIEsProvideLocationInformation (including the position estimate result or the ranging result)

- SL-Multi-RTT-ProvideLocationInformation

- SL-TDOA-ProvideLocationInformation

- SL-AOA-ProvideLocationInformation

- 2.4 Group positioning of multiple UEs per SA2 Request

In the LS sent from SA2[2], use cases regarding multiple target UEs being involved in the same positioning session has been presented, and SA2 would like to ask RAN2 whether the SLPP would support multiple target UEs in the same positioning session, and whether there is possibility of signaling the positioning results of multiple Target UEs in the same message. In the last RAN2 meeting, extensive discussion on supporting the group positioning was made. Recall that one of the major controversial issues is whether or not absolute positioning should be considered.

In our opinion, the classical group positioning scenario is vehicle platoon in which vehicles moves together at a certain speed towards the same direction on/off road. In such cases, the most important task for the vehicles other than the leading one is to follow the front vehicle in the platoon. If the distance between the neighbor vehicle in the platoon is increasing, the autopilot system should instruct the vehicle in the rear to speed up to catch up with the one in the front. Similarly, if the front vehicle makes a turn in the road intersection, the rear one should follow autonomously to make a turn also. Therefore, rather than the absolute positioning result, the relative positioning/ranging result between pairs of vehicles in the platoon should be the focus to be obtained by the system. Of course, the task of the leading car is to lead other vehicles to the destination, hence the absolute positioning result of the leading car is obviously needed by the self-driving system and could be retrieved from possibly any available positioning method other than the PC5-based, such as GNSS, Uu-based positioning method, etc. As a result, we prefer that only relative positioning results of the accompany UEs are retrieved for the case of positioning of multiple target UEs.

Observation 5: the most important task for the vehicles other than the lead one is to follow the front vehicle in the platoon. If the distance between the neighbor vehicle in the platoon is increasing, the autopilot system should instruct the vehicle in the rear to speed up to catch up with the one in the front. Similarly, if the front vehicle makes a turn in the road intersection, the rear one should follow to make a tune also.

Proposal 10: RAN2 to agree that only relative positioning/ranging results between pairs of UEs are considered for the case of group positioning for multiple UEs

As for the absolute positioning of the leading car, we think it could be performed in another independent PC5-based positioning session involving dynamically changing anchor UEs, Uu-based positioning session or RAT-independent positioning session. Note that the advantage of decoupling the positioning demand of absolute positioning of the leading car from the positioning of multiple target UEs is that the positioning session for the multiple target UEs does not need to take into account the changing anchor UEs, which eliminates the need of updating the anchor UE information towards the accompany UEs in the same positioning session. Our proposed sidelink based group positioning msg flow is illustrated as follows

Figure 2: group positioning of multiple UEs for an LCS client

The procedure is presented as follows:

- LCS client sends the LCS Request msg towards a UE randomly chosen from the group or the leading UE in the group chosen by the Application.

- The UE receives the msg initialize the discovery procedure for determination of the location server UE in the UE group.

- SLPP group management and groupcast establishment is finished within the UEs belong to the group.

- Sidelink positioning capability is exchanged within the group of UEs to determine the sidelink positioning method.

- Assistance data including the configuration of the SL-PPRS to be transmitted by each UE in the group is exchanged between UEs

- Location server UE sends the SLPP RequestLocationInformation msg towards the other UE in the group indicating which measurement result metrics are required.

- The UEs in the group perform SL-PRS transmission and measurement.

- All UEs in the group transmit the SL-PRS measurement result in the SLPP ProvideLocationInformation msg towards the location server UE

- The location server UE derives the SLPP relative positioning/ranging results between all pair of UEs and send the results towards the LCS client.

In the figure, one thing needs to be highlighted. As per the SA2’s request that reporting the positioning results of multiple Target UEs in the same msg, a location server UE should be designated to take the job of the collecting the SL measurement result from various UEs in the group. For such purposes, we think that discovery procedure is needed, as currently SA2 has agreed that role indication shall be included in the discovery msg.

Proposal 11: RAN2 to capture the msg flow indicated in the figure 2 as the baseline for the positioning of group UEs in the stage 2 spec.

Proposal 12: RAN2 to agree that discovery procedure shall be involved in the group positioning of multiple UE for discovery and determination of location server UE.

Proposal 13: LS reply to SA2 to confirm that it is feasible for the SLPP to support multiple Target UEs relative positioning/ranging, and positioning result of multiple Target UEs could be sent in one single msg.

- 2.5 Consideration of the session-less positioning procedure

According to [3], the session-less positioning procedure targets at providing the positioning services in highly dynamic mobility environment, wherein the SL-PRS configurations and the location measurement results are exchanged with each other UEs without establishing the SL unicast communication link in prior. The UEs in proximity to the initiating UE are free to choose to join in the positioning or not. Also note that the SL-PRS configuration of each UE is independently chosen by each UE from the configuration pool provided by the initiating UE.

However, as well known, for obtaining the absolute positioning result of one particular UE, the location information of the reference points, in the SL positioning case, the anchor UEs should be known in advance by the positioning entity. This principle, however, cannot be held in the session-less positioning procedure if the location information of UEs participating in as the anchor UE are unknown. For example, if the Mult-RTT positioning method is used, the positioning entity could only know the distance between each pair of UEs participated in the session-less positioning procedure, which is definitely not enough for obtaining the absolute positioning result.

Observation 6: the principle that the location information of the reference points, say, the anchor UEs in the SL positioning case, should be known in advance by the positioning entity cannot be held in the session-less positioning procedure, if the location information of UEs participating in as the anchor UEs are unknown.

As a result, we propose that for one particular session-less positioning procedure, except the initiating UE, the participating UEs should be the ones with known location information, for instance the RSUs. In other words, the initiating UE should only perform SL-PRS measurement of the UEs with pre-configured deterministic SL-PRS configuration and known location information, for instance, the RSUs.

Proposal 14: for one particular session-less positioning procedure, the initiating UE, should only perform SL-PRS measurement of the UEs with known location information, for instance, the RSUs.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Proposal 1: RAN2 to agree to support transmitting the SLPP msg in the LPP container for the PC5-only and PC5+Uu positioning in the IC and PC scenarios.

Observation 1: whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Observation 2: Option 2 (LMF triggering PC5+Uu based positioning in a new positioning session) may consume more time than Option 1 (autonomously triggering PC5 based positioning based on some preconfigured condition) to obtain a satisfied positioning result, since another new positioning session has to be triggered for the option 2, and the Uu positioning capability and reference signal configuration may need to be transferred again between the UE and the gNB during such new positioning session.

Proposal 2: RAN2 to agree that target UE could autonomously trigger the sidelink positioning to calibrate the Uu positioning result.

Proposal 3: RAN2 to agree that sidelink positioning triggering event could be defined and pre-configured towards the target UE.

Observation 3: Exhibiting good radio propagation path between candidate anchor UEs and the target UE is good to have sufficiently accurate and stable SL-PRS measurement result.

Observation 4: letting the target UE to perform SL discovery procedure results in a SL location server UE that is proximity to the target UE. Even if the distance between the location server UE and the anchor UE is long, still can we use the target UE to relay the signaling msg between the location server UE and the target UE.

Proposal 5: RAN2 to agree that discovery procedure should be done by the target UE regardless of the coverage scenario

Proposal 6: RAN2 to agree that, besides the anchor UE location information, a flag indicating of the possibility of exposing the location information to the location server UE OOC should be included in the discovery msg also.

Observation 4: the SL positioning assistance data transfer msg should include required assistance data for UEs to perform the necessary SL-PRS measurements.

Proposal 7: RAN2 to agree that SL positioning assistance data transfer msg could be used in following scenarios, optionally with the location information of the anchor UEs included:

- the SL-PRS transmitter UE sends out its SL-PRS configuration

- the location server UE/LMF transmits the SL-PRS configuration of other transmitter UEs to the SL-PRS monitor UEs

- the location server UE/LMF sends the SL-PRS configuration to be used to the SL-PRS transmitter UEs.

Proposal 8: RAN2 to agree that at least following three positioning method related IEs should be included in the SLPP RequestLocationInformation msgs:

- SL-AOA-RequestLocationInformation

- SL-TDOA-RequestLocationInformation

- SL-RTT-RequestLocationInformation

Proposal 9: RAN2 to agree at least following IE should be included in the SLPP ProvideLocationInformation msg at least:

- commonIEsProvideLocationInformation (including the position estimate result or the ranging result)

- SL-Multi-RTT-ProvideLocationInformation

- SL-TDOA-ProvideLocationInformation

- SL-AOA-ProvideLocationInformation

Observation 5: the most important task for the vehicles other than the lead one is to follow the front vehicle in the platoon. If the distance between the neighbor vehicle in the platoon is increasing, the autopilot system should instruct the vehicle in the rear to speed up to catch up with the one in the front. Similarly, if the front vehicle makes a turn in the road intersection, the rear one should follow to make a tune also.

Proposal 10: RAN2 to agree that only relative positioning/ranging results between pairs of UEs are considered for the case of group positioning for multiple UEs

Proposal 11: RAN2 to capture the msg flow indicated in the figure 2 as the baseline for the positioning of group UEs in the stage 2 spec.

Proposal 12: RAN2 to agree that discovery procedure shall be involved in the group positioning of multiple UE for discovery and determination of location server UE.

Proposal 13: LS reply to SA2 to confirm that it is feasible for the SLPP to support multiple Target UEs relative positioning/ranging, and positioning result of multiple Target UEs could be sent in one single msg.

Observation 6: the principle that the location information of the reference points, say, the anchor UEs in the SL positioning case, should be known in advance by the positioning entity cannot be held in the session-less positioning procedure, if the location information of UEs participating in as the anchor UEs are unknown.

Proposal 14: for one particular session-less positioning procedure, the initiating UE should only perform SL-PRS measurement of the UEs with known location information, for instance, the RSUs.

Reference

[1] TS 23.273, 3GPP 5G system Location Services Stage 2, 3GPP

[2] S2-2303837, LS on support of multiple Target UEs, 3GPP SA2

[3] R2-2300529, Sidelink Positioning Protocol (SLPP) Signaling and Procedures, Qualcomm