3GPP TSG-RAN WG2 #122	Tdoc R2-2305355

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	7.21.2

Source:	Ericsson

Title:	Discussion on Multiple PRACH Transmission Configuration

Document for:	Discussion

# 1 Introduction

In the first RAN2-meeting for Coverage Enhancements Rel-18 (RAN2#121bis-e) the following agreements where made:

Agreements - RAN2 assumes that MSG1 repetition can be applicable to all 4-step CBRA procedures (FFS for SI request) - CFRA support is FFS - RAN2 assumes that MSG1 repetition can be applicable to NUL - RAN2 assumes that MSG1 repetition can be applicable to SUL - Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE) - RAN2 waits for further inputs from RAN1 for how to associate RA-RNTI to the PRACH occasion for multiple PRACH transmissions and also for ra-ResponseWindow start point - General assumption is that various feature combinations can be configured (which is up to network implementation), unless explicitly specified otherwise - RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS - BWP selection mechanism is not impacted by PRACH coverage enhancements. Legacy BWP selection mechanism is re-used - RA type selection mechanism is not impacted by PRACH coverage enhancements. Legacy RA type selection mechanism is re-used

In RAN1#112bis-e, the following agreements (among others) where made:

Agreements - There is no consensus to support multiple PRACH transmissions within one RACH attempt located at same time instance in Rel-18. - There is no consensus to support utilizing different preambles during the multiple PRACH transmissions with the same Tx beam in one attempt. - Multiple PRACH transmissions within one RACH attempt are only performed within one RO group. - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions. Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value. Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions. Note3: Valid RO(s) refers to what is defined in existing specification. - The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

Based on these agreements from RAN1 and RAN2, this discussion paper handles the further configuration aspects necessary.

# 2 Discussion

RACH Partitioning Framework and RO groups

The agreements made in RAN2#121bis-e agreed that RACH Partitioning Framework (RPF) should be reused, and that msg1 repetitions should be signalled as one feature in the IE FeatureCombination.

#### FeatureCombination The IE FeatureCombination indicates a feature or a combination of features to be associated with a set of Random Access resources (i.e. an instance of FeatureCombinationPreambles). FeatureCombination information element -- ASN1START -- TAG-FEATURECOMBINATION-START FeatureCombination-r17 ::= SEQUENCE { redCap-r17 ENUMERATED {true} OPTIONAL, -- Need R smallData-r17 ENUMERATED {true} OPTIONAL, -- Need R nsag-r17 NSAG-List-r17 OPTIONAL, -- Need R msg3-Repetitions-r17 ENUMERATED {true} OPTIONAL, -- Need R spare4 ENUMERATED {true} OPTIONAL, -- Need R spare3 ENUMERATED {true} OPTIONAL, -- Need R spare2 ENUMERATED {true} OPTIONAL, -- Need R spare1 ENUMERATED {true} OPTIONAL -- Need R } NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17 -- TAG-FEATURECOMBINATION-STOP -- ASN1STOP

There are four spare values available in the FeatureInformation IE.

Parameter msg1-Repetitions-r18 replaces one of spare1-4 in the FeatureCombination information element.

By the usage of a feature information parameter, the UE can combine msg1 repetitions together with other features in the RPF. However, the gNB must know how many and which particular PRACHs to combine for a UE across all RACH Occasions. That is why our suggestion is in line with that discussed during RAN2#121bis-e, to add configuration options to the FeatureCombinationPreambles to indicate the specific repetition factor used for a certain partition.

Add configuration options to FeatureCombinationPreambles IE to indicate the specific repetition factor for a preamble partition that supports msg1 repetitions.

The preamble partition will be placed on ROs according to the configuration of RACH resources in the cell. The existing configuration options for both release 16 and 17 are very flexible, but also creates a lot of different options to consider on how preamble partitions will be placed. Given other agreements in RAN1 [e.g. agreement 11] that excludes the use of ROs that occur at the same time instances for a repetitions, it seems wise to discuss some general principles from RAN2 on where a set of Multiple PRACH Transmission starts, and ends.

The configuration of RACH occasions enables many different scenarios.

RAN2 discusses general principles for when a set of repetitions should start and end.

The agreement 13 from RAN1 [see chapter 1 Introduction] states that an RO group has to fully contain all Multiple PRACH Transmission from one RACH attempt. This translates to that all preambles (2, 4 or 8) are sent within the RO group.

Based on the configuration of ROs in 3GPP TS 38.211 – Tables 6.3.3.2-2/3/4, it can be seen that the maximum number of RACH Occasions (not counting FDMed ROs) within a PRACH slot is 7.

One PRACH slot cannot support a repetition factor of 8.

A RACH attempt with Multiple PRACH Transmissions can span across one or more PRACH slots.

With Observation 2 and Observation 3 in mind it is clear that a decision on direction is needed for the determination of the RO group. One option could be that a list of ROs for the Multiple PRACH Transmission and the corresponding repetition factor (K) is derived from the RPF configuration together with the SSB to RO mapping, see Figure 1.

Figure 1; RO mapping example for a specific PRACH Config Index

This sequence of ROs could be indexed and then used for the UE to map the K ROs as an RO group for the Multiple PRACH Transmissions for this RACH attempt. However, the granularity of such a mapping should be discussed. Figure 1 only treats one system frame of 10 ms, however, the SSB to RO mapping might need longer granularity to be able to fit 8 PRACH transmissions for a selected SSB. Another possibility could be to use the System Frame Number (SFN) which repeats every 1024 frames, which seems a bit long. Other options could be the Association Period (AP) or the Association Pattern Period (APP), which is currently discussed in RAN1. One association pattern period is the time duration where a pattern between PRACH occasions and SS/PBCH block indexes repeats, and it ranges from 10 ms to 160 ms, depending on SSB periodicity (up to 160ms), TDD pattern, etc. Given the largest PRACH repetition factor 8 and the longest association pattern period, RO groups can repeat in a granularity of 8 x 160ms, namely 128 frames and 1/8 of all SFN values. .

RO groups repeat at most every 128 radio frames.

RAN2 discusses RO indexing and how 2, 4 and 8 transmissions are mapped over available ROs.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	There are four spare values available in the FeatureInformation IE.

Observation 2	The configuration of RACH occasions enables many different scenarios.

Observation 3	One PRACH slot cannot support a repetition factor of 8.

Observation 4	RO groups repeat at most every 128 radio frames.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Parameter msg1-Repetitions-r18 replaces one of spare1-4 in the FeatureCombination information element.

Proposal 2	Add configuration options to FeatureCombinationPreambles IE to indicate the specific repetition factor for a preamble partition that supports msg1 repetitions.

Proposal 3	RAN2 discusses general principles for when a set of repetitions should start and end.

Proposal 4	A RACH attempt with Multiple PRACH Transmissions can span across one or more PRACH slots.

Proposal 5	RAN2 discusses RO indexing and how 2, 4 and 8 transmissions are mapped over available ROs.

# References

3GPP WID, Further NR coverage enhancements, RP-221858, Link

Chairs notes with RAN1 agreements, RAN1#111 Toulouse

Chairs notes with RAN1 agreements, RAN1#112 Athens