3GPP TSG-RAN WG2 #122 	R2-2305361

Incheon, Korea 22 – 26 May 2023

Agenda Item:	7.5.2

Source:	NEC

Title:	XR awareness

Document for:	Discussion, Decision

Introduction

RAN2#121-bis meeting discussed XR awareness and agreed followings [1]:

- UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting. - FFS on whether EoDB signalling is needed.

This contribution further discusses remaining issues on XR awareness.

Discussion

According to TR 38.835 Clause 5.1, the following information may be provided by the CN to RAN (see TR 23.700-60 [9]):

-	Semi-static information per QoS flow:

-	Periodicity for UL and DL traffic of the QoS Flow provided via TSCAI/TSCAC;

-	DL Traffic jitter information (e.g. jitter range) associated with each periodicity of the QoS flow provided via TSCAI/TSCAC.

-	PDU Set QoS parameters of the QoS flow (i.e., applicable to all PDU sets of the QoS flow) provided by the SMF via NGAP (details are skipped)

Regarding whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR, RAN2 sent an LS [2] to SA2 and waiting for their reply. Then our observations are

RAN may have knowledge on the periodicity for UL traffic of the QoS Flow which TSCAI can be provided.

For DL, RAN may have knowledge on traffic jitter information (e.g., jitter range) associated with each periodicity of the QoS flow which TSCAI can be provided.

Since RAN2 is aiming to have UL jitter information like that as for DL, therefore, it is reasonable to also report UL traffic jitter information (e.g., jitter range) associated with each UL periodicity of the QoS flow which TSCAI can be provided. Moreover, to adjust the appropriate CG start offset, the latest UL traffic data arrival time is necessary. Without the latest UL traffic data arrival, the CG start offset may be configured too early or too late, which results in poor performance in terms of UL capacity and delay.

For reporting such a UL jitter information, the existing UE assistance information framework specified in TS38.331 clause 5.7.4 can be reused.

RAN2 to confirm that a UE capable of providing UL traffic jitter information may initiate the UE assistance information procedure if it is configured to provide UL traffic jitter information, and report both UL jitter information and the latest UL traffic data arrival time associated with configured each UL periodicity of the QoS flow which TSCAI can be provided.

Another remaining issue is whether UL end of data burst indication is needed. For DL, end of data burst indication in the header of the last PDU of the data burst is optionally provided by CN to RAN. In our view, zero BSR or padding BSR can be used to indicate there are no more data, additionally, as analyzed in [3], release preference information defined as a part of UAI which allows the UE to indicate its preferredRRC-State-r16 can also be used for indicating UL end of data burst instead. Therefore, we think there is no need to provide UL end of data burst indication from UE to RAN.

No need to provide UL end of data burst indication from UE to RAN.

Summary

This contribution further discussed remaining issues on XR awareness.

Observation 1 RAN may have knowledge on the periodicity for UL traffic of the QoS Flow which TSCAI can be provided.

Observation 2 For DL, RAN may have knowledge on traffic jitter information (e.g., jitter range) associated with each periodicity of the QoS flow which TSCAI can be provided.

Proposal 1 RAN2 to confirm that a UE capable of providing UL traffic jitter information may initiate the UE assistance information procedure if it is configured to provide UL traffic jitter information, and report both UL jitter information and the latest UL traffic data arrival time associated with configured each UL periodicity of the QoS flow which TSCAI can be provided.

Proposal 2 No need to provide UL end of data burst indication from UE to RAN.

- References

[1] R2-2304202 Report on LTE legacy, XR, QoE and MUSIM

[2] R2-2304400 LS on TSCAI for XR

[3] R2-2302909 XR awareness enhancements in RAN	Intel Corporation