3GPP TSG-RAN WG2 Meeting #122	                                      R2-2305364

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:		7.5.4.1

Source:			FGI

Title:		Generate new buffer status report table

Document for:	 	Discussion and decision

Introduction

According to the WID of XR enhancement for NR [1], RAN2 will work on capacity enhancements via BSR enhancements:

Specify the enhancements related to capacity: - BSR enhancements including at least new BS Table(s); (RAN2);

During the discussion in RAN2 #121bis, companies describe their respective views on new BSR table(s) in the email discussion [AT121bis-e][212]) BSR solutions (Qualcomm) [2]. The following agreement was achieved:

2a. Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor). 2b. Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

This paper discusses the suitable BSR table type (static or RRC configured) and provide suggestion to the way to generate new BSR table.

Discussion

XR traffic characteristic diversity

eXtended Reality (XR) and Cloud Gaming applications are one of the most expected use cases of Rel-18 5G system. They are based on various types of human-to-human, human-to-machine interactions. XR is an umbrella term that includes representative forms such as Augmented Reality (AR), Mixed Reality (MR) and Virtual Reality (VR) and the areas interpolated among them [3].

XR applications are still in the early growing stage. Various use cases in education, healthcare, retail, engineering, manufacturing have been widely discussed and promised. Even more use cases are under exploring. Existing XR applications have present a wide traffic characteristic in terms of data burst size, traffic periodicity, delay budget, etc. It is already known that a XR application may generate several different (multi-modal) traffic flows, each traffic flow has its specific traffic characteristic and QoS requirement.

Observation 1: Traffic characteristics between XR applications are of wide diversity.

XR traffic characteristic variation

Each XR application has its specific downlink and uplink traffic characteristic. For the design of new BSR table(s), uplink traffic characteristic is in the concerned. XR uplink traffic characteristic is affected by the user activities, sensor data encoder, device model, the number of devices equipped, etc. Furthermore, traffic characteristic would show an even different pattern in the case of tethering. Therefore, uplink traffic characteristic of a XR application may not be the same every time it is started, and may change from time to time during the application run time. Although, in general, uplink traffic of XR application will be in one of limited number of traffic patterns. However, as the emerging XR applications becomes popular in the future, it is foreseen there will be many different uplink traffic patterns used by UEs in a cell. In this scenario, static BSR table(s) is not able to effectively limit the quantization error in a small range among UEs. On the other hand, if necessary, the network can determine the BSR table suitable for specific XR application and signal the table to the UE via RRC signaling. The BSR table determination can be based on the XR application’s traffic pattern information obtained from the application server, or the statistic data collected from long-term, offline analysis.

Observation 2: Static BSR table(s) is not able to effectively limit the quantization error in a smaller range among UEs running various XR applications.

Observation 3: Dynamically configured BSR table(s) can better fit XR application’s traffic characteristic, and thus provides a smaller quantization error.

Proposal 1: RAN2 adopts RRC-configured BSR table.

The number of new BSR table needed

In the last meeting, a question await to answer is how many new BSR table(s) are needed to achieve the goal of reducing the quantization error to a [FFS] level. In our view, if two or more new BSR tables are introduced, UE has to indicate which table is used in the BSR. Therefore this requires new BSR format. However, so far there is no sufficient information indicates how many BSR tables are needed. We suggest to postpone the decision of the number of new BSR tables needed until the target quantization error is determined.

Proposal 2: Postpone the decision of the number of new BSR tables needed until the target quantization error is determined.

Conclusion

Based on the above discussion, for the XR BSR enhancement, we have the following observations.

Observation 1: Traffic characteristics between XR applications are of wide diversity.

Observation 2: Static BSR table(s) is not able to effectively limit the quantization error in a smaller range among UEs.

Observation 3: Dynamically configured BSR table(s) can better fit XR application’s traffic characteristic, and thus provides a smaller quantization error.

RAN2 is kindly asked to consider the following proposal.

Proposal 1: RAN2 adopts RRC-configured BSR table.

Proposal 2: Postpone the decision of the number of new BSR tables needed until the target quantization error is determined.

Reference

[1] RP-223502, New WID on XR Enhancements for NR

[2] R2-2304394	Report of [AT121bis-e][212][XR] BSR solutions (Qualcomm).

[3] TR 26.926, Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems.