3GPP TSG-RAN WG2 Meeting #122	                                  R2-2305365

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:		7.4.2.1

Source:			FGI

Title:		Discussion on LTM supervisor timer

Document for:	 	Discussion

- Introduction

In one of Rel-18 WIDs, L1/L2-triggered mobility (LTM) [1] was proposed to further enhance NR mobility by latency reduction. Generally, the time flow for LTM [2] includes LTM preparation, early synchronization, LTM execution and LTM completion. In RAN2#120 meeting [3], it was agreed that LTM cell switch is supervised by a timer.

The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch. LTM cell switch is supervised by a timer UE arrival in the target cell need to be indicated (somehow)

In RAN2#121bis meeting [4], it was agreed that the timer is a RRC layer timer, and the corresponding basic operation was agreed.

Following behaviors of LTM supervisor timer are agreed: - - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

In this contribution, we would share our views on how the UE operates the LTM supervisor timer.

- Discussion

LTM supervisor timer

In LTM, the UE operates an RRC layer timer, e.g., LTM supervisor timer, to monitor the success and failure of LTM. Upon the reception of the LTM cell switch MAC CE, the UE starts the LTM supervisor timer, implying the UE begins to monitor the success and failure of LTM. Upon the successful completion of LTM cell switch, the UE stops the LTM supervisor timer since the timer is no need to monitor the success and failure of the LTM. However, if the LTM supervisor timer keeps running and finally expires, the UE will consider LTM failure and initiate RRC re-establishment procedure.

The duration between when the timer starts and when the timer expires would impact the determination of LTM failure and the performance. Since the network supposed to know the deployment and track the UE movement, we propose that the value of the duration should be provided by the network to the UE. Furthermore, since LTM supervisor timer is an RRC layer timer, the initial value of the LTM supervisor timer, i.e., the value of the duration, is an L3 parameter. Thus, we further propose that the UE receives the initial value of the LTM supervisor timer, from the source serving cell in RRCReconfiguration message during RRC pre-configuration step.

Proposal 1: The initial value of the LTM supervisor timer is provided by the source serving cell to the UE in RRCReconfiguration message during RRC Pre-configuration step.

After the UE receives the initial value of the LTM supervisor timer, we need to further discuss when the UE set the LTM supervisor timer to the received initial value. There are two possible approaches: (1) Upon the UE receives the initial value of the LTM supervisor timer, the UE sets the value of the LTM supervisor timer to the received initial value; and (2) Before the UE starts the LTM supervisor timer, the UE sets the value of the LTM supervisor timer to the received initial value. Approach (1) is straightforward. However, if the subsequent cell switch is considered, only Approach (1) is insufficient because the LTM supervisor timer is stopped or expiry whenever a cell switch completes either successfully or unsuccessfully. On the other hand, Approach (2) is flexible whenever the subsequent cell switch is considered. Approach (2) can guarantee that the initial value of the timer is set before the UE starts the LTM supervisor timer. It is possible that the UE sets the initial value of the timer after the UE receives the LTM cell switch MAC CE but before the UE starts the LTM supervisor timer. In addition, the UE performs the subsequent cell switch upon receiving the LTM cell switch MAC CE. Therefore, we propose that the UE sets the initial value of the LTM supervisor timer when the UE receives the LTM cell switch MAC CE.

Observation 1: Whether the subsequent cell switch is applied can impact when the UE sets the initial value of the LTM supervisor timer.

Proposal 2: The UE sets the initial value of the LTM supervisor timer when the UE receives the LTM cell switch MAC CE.

Regarding the condition when the UE stops the LTM supervisor timer, it is noted that even when the UE is configured with LTM candidate cells, the UE can also execute L3 handover command [4]. Thus, in case when the L3 HO is triggered and executed, the UE should stop the LTM supervisor timer, if running.

Proposal 3: The UE stops the LTM supervisor timer, if running, when the UE executes L3 handover.

Conclusion

Based on the above, we have the following observations and proposals.

Proposal 1: The initial value of the LTM supervisor timer is provided by the source serving cell to the UE in RRCReconfiguration message during RRC Pre-configuration step.

Observation 1: Whether the subsequent cell switch is applied can impact when the UE sets the initial value of the LTM supervisor timer.

Proposal 2: The UE sets the initial value of the LTM supervisor timer when the UE receives the LTM cell switch MAC CE.

Proposal 3: The UE stops the LTM supervisor timer, if running, when the UE executes L3 handover.

Reference

[1] RP-223520 “Revised WID on Further NR mobility enhancements”.

[2] R2-2213332 “38.300 running CR for introduction of NR further mobility enhancements”.

[3] RAN2#120 Meeting Chairman’s note.

[4] RAN2#121-bis Meeting Chairman’s note.