3GPP TSG-RAN WG2 Meeting #122                     R2-2305372

Incheon, Korea, May. 22th – May. 26th, 2023

Agenda Item:	7.6.4

Source: 	Transsion Holdings

Title:  	Discussion on enhancement to discontinuous coverage for IoT NTN

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting, the enhancement to discontinuous coverage was discussed, and it has already agreed with the agreements mentioned in the table below: [4]

Agreements: 1、RAN2 will not introduce any enhancement to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap (e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement) 2、RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

In this contribution, we provide our views on enhancement to discontinuous coverage to proceed the work item.

- Discussion

- 2.1 Further enhancement to discontinuous coverage

- In R17 IoT-NTN, in order to enable the UE to save power during discontinuous coverage periods in RRC_IDLE, the network provides satellite assistance information (e.g. satellite ephemeris parameters, the start-time of upcoming satellite's coverage) to enable the UE to predict when coverage will be provided by the upcoming satellites.

Since NTN cells provide much bigger coverage then TN cells, the number of UEs served by a NTN cell is much larger than that of TN cell. For the (quasi-)earth fixed cells, as the network will broadcast the start-time of the upcoming satellite’s coverage, there will be a large number of UEs trying to access the network at the start-time of the upcoming satellite’s coverage which will result in a serious RACH congestion. In current specification, the network can provide a backoff indicator at RAR message to instruct the UEs to delay a random period to resend the preamble when RACH congestion happens. However this may cause another issue as IoT-NTN UEs need to acquire the SIB31(-NB) to get uplink synchronization before accessing the network and the uplink sync duration is limited by the ul-SyncValidityDuration. If it expires, the UE needs to reacquire SIB31(-NB). The specification has specified that IoT UEs are not required to acquire system information in connected state, which means reacquire SIB31(-NB) in connected will cause service interruption as the IoT-NTN UEs need to leave connected mode to acquire SIB31(-NB) and get uplink re-sync. If the UE spends too much time on RACH during RACH congestion, then there will be a limited time for data service as the ul-SyncValidityDuration may expire in connected state. So it needs to avoid RACH congestion when the next satellite starts serving the area rather than relieve congestion when RACH congestion happens. For example the current serving satellite can instruct the UE to delay a period to access the upcoming satellite, the delay period can be generated base on traffic load or just a random value.

- RAN2 needs to consider the RACH congestion issue as a large number of UEs may try to access the NTN cell at the next satellite’s service start time in discontinuous coverage scenario.

- This similar iusse has also been identified and discussed by SA2 and the solution of SA is as follows[4]:

- Solution #xx: Utilizing discontinuous coverage wait timer for satellite discontinuous coverage scenario:

This solution proposes to introduce a new timer, named discontinuous coverage wait (DCW) timer, to address both the signalling overload issue and the power saving issue when UEs are using access type and/or RAT type that offer discontinuous coverage. The DCW timer consists of 3 parts:

DCW timer = (T2-T1) + DCW value (i.e. random value in DCW range) + offset (optional)

- - The discontinuous period (T2-T1), used to ensure that the UEs do not initiate any 5G NAS signalling to 5GCN within this period

- - The DCW value, used by the UE to determine how long to wait before triggering the 5G NAS signalling after the discontinuous period.

- While the timer is running, the UE shall not initiate any 5G NAS signaling to 5G CN. Upon the expiration of the DCW timer, the UE shall initiate the 5G NAS signalling.

- The similar mechanism in AS by eNB can also be considered to randomized the UE’s access time to avoid RACH congestion, as it has already been agreed to provide the discontinuous period to UE in R17(the t-Service in SIB3/SIB3-NB and the t-ServiceStart in SIB32/SIB32-NB).

- The discontinuous coverage wait timer can be used in AS to avoid RACH congestion.

- When out of coverage, the UE is not required to perform AS functions. That means that the UEs will not monitor paging when in its predicting discontinuous coverage period. However the core network will initiate the paging procedure for EPS services when needed, there is an chance that the paging procedure will initiated during the UE’s predicting discontinuous coverage period which will result in paging failure. So the CN may need to avoid paging UE when in UE’s predicting discontinuous coverage period. That means the CN has to be aware of the UE’s discontinuous coverage period.

- The CN should be aware of the UE’s discontinuous coverage period to avoid paging issues.

- The R17 has already agreed to provide the satellite assistance information for the UE to predicting discontinuous coverage. This information can also be provide to the CN for predicting discontinuous coverage. For the quasi-earth fixed cells, the satellite assistance information can be the end service time of the current satellite and the start service time of the next satellite for certain area. For the earth moving cell, the the satellite assistance information can be the ephemeris of the satellites. More over the UE location can also be provided to the CN, the location can be the UE reported location、the cell id the UE last camped or the TAC the UE last registered. As the UE can predict the discontinuous coverage period based on the satellite assistance information, the predicting discontinuous coverage information can be provided to the CN for reference.

- The satellite assistance information or the discontinuous coverage information predicted by UE can be provided to CN for paging.

- In last meeting , it has been agreed that RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one):

- -	Explicit RRC Release using a new RRC Release cause

- -	UE Autonomous release (e.g. timer based or upon detection of coverage gap)

When discontinuous coverage happens, the RRC connection should be released both in UE and NW to avoid resouece consumption and unnecessary AS action.To make the UE and NW consistant understanding of discontinuous coverage, the explict RRC release using a new RRC Release cause(e.g. coverage discontinuity) can be used before the covearege gap. In  this case, when UE received RRC release measage with specific cause(e.g. coverage discontinuity), UE will release the RRC connection and stop unnecessary AS action until the coverage is restored.

- The explict RRC release using a new RRC Release cause(e.g. coverage discontinuity) can be used before the covearage gap.

- 3 Conclusions

In this contribution we discussed issues related to discontinuous coverage and made the following proposals:

- RAN2 needs to consider the RACH congestion issue as a large number of UEs may try to access the NTN cell at the next satellite’s service start time.

- The discontinuous coverage wait timer can be used in AS to avoid RACH congestion.

- The CN should be aware of the UE’s discontinuous coverage period to CN to avoid paging issues.

- The satellite assistance information or the discontinuous coverage information predicted by UE can be provided to CN for paging.

- The explict RRC release using a new RRC Release cause(e.g. coverage discontinuity) can be used before the covearage gap.

- 4 References

- “RP-221806 Revised WID on IoT NTN enhancements”

- “3GPP TS 38.300 V17.1.0”

- “TR 23.700-28   Study on Integration of satellite components in the 5G architecture”

- “R2-2304201 - Report from Break-out session on NR-NTN and IoT-NTN.docx”