3GPP TSG-RAN WG2 Meeting #122                           R2-2305379

Incheon, Korea, 22nd May – 26th May 2023

Agenda item:	7.11.2.1

Source:	TCL

Title:	Discussion for UEs receiving Multicast in RRC_INACTIVE state

Document for:	Discussion and Decision

- Introduction

At RANP#94-e meeting, a new RAN2 work item on ‘Enhancements of NR Multicast and Broadcast Services’ [1] was agreed. The goal of the work item is to further enhance the NR Multicast/Broadcast functions based on Rel -17 MBS. One of the objectives in the WID is mentioned as the following.

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

During R2#121b meeting, the following agreements have been reached.

Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

- No need to define a mechanism other than the frequency prioritization, i.e., per cell based prioritization in cell re-selection, to help UE choose the right cell to camp on.

- The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

- A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

- Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

- For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

- FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

- No additional enhancement (with regard to enhancements made for ‘deactivation/temp no data’) is needed specifically for enabling UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release.

- Legacy group paging (i.e., Rel-17 group paging) can be used to resume UE to RRC_CONNECTED state.

- Upon events like session activation/data transmission resumed, if PTM configuration is not available to UE, UE initiates RRC connection resumption.

- UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior).

- When both enhanced group paging and unicast paging are received by the UE (and targeted for this UE), the UE follows unicast Paging and goes to RRC CONNECTED.

In this contribution, we provides the discussion for UEs receiving multicast in RRC_INACTIVE.

- Discussion

As we can see above that fruitful agreements have been made during RAN2#121b meeting regarding CP issues for UEs receiving multicast in RRC_INACTIVE state, however the notification when network update the configuration for UEs in inactive has not been discussed which also mentioned by some companies. We can firstly review how R17 works when network update the PTM configuration. In R17, MCCH change notification is used for indication of update of PTM configuration for broadcast services.

In R17 MBS broadcast, MBS broadcast can be received by UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. A UE can receive the MBS configuration for broadcast session (e.g., parameters needed for MTCH reception) via MCCH in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The parameters needed for the reception of MCCH are provided via System Information.

MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session includes MBS session ID, associated G-RNTI scheduling information and information about neighbouring cells providing certain service on MTCH(s). MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; A notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification;

When the UE receives a MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.

Since the behaviour for UEs receiving multicast in inactive state is similar to broadcast, especially now we are keen to accept the draft agreement which is “UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection”, we can simply reuse the R17 mechanism for broadcast to minimize the spec impact and standards effort.

Proposal 1: Reuse R17 mechanism for broadcast which MCCH change notification is used when network update the PTM configuration.

However there is another issue coming with which is UE has to constantly monitor MCCH during RRC_INACTIVE for possible updated PTM configuration even if there is no any updated PTM configuration for a relatively long period. This will not be a rare case since PTM reconfiguration doesn’t happen so often, therefore we would have to consider how to decrease the UE power consumption.

Observation 1: Constantly monitor MCCH during RRC_INACTIVE for possible updated PTM configuration will increase the UE power consumption.

Some companies mentioned a solution which is “NW could decide to only configure the MCCH in SIB when a PTM change is pending, and release the MCCH configuration in SIB when the reconfiguration is completed.” However some companies also showed concern with this option. The main concern is in case when MCCH carry more information other than just updated PTM configuration, such as session notification (e.g., activation/deactivation) or PTM configuration of neighbour cell, the frequency of MCCH monitoring may not be seldom. Therefore in case RAN decides to specify enhancements to decrease UE power consumption due to monitor MCCH for updated PTM configuration it is needed to conclude firstly what information MCCH would provide rather than updated PTM configuration.

In our perspective, the power consumption issue might not be very severe. Firstly the balance between power consumption and MCCH modification period can be up to network implementation. Secondly UE will monitor Broadcast MCCH anyway, and Multicast MCCH modification period can be configured aligned with Broadcast MCCH. Therefore it might not be needed to do any enhancements regarding this issue.

Proposal 2: Recommend RAN2 not to discuss specifying enhancements to decrease UE power consumption due to monitor MCCH during RRC_INACTIVE.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposal:

Proposal 1: Reuse R17 mechanism for broadcast which MCCH change notification is used when network update the PTM configuration.

Observation 1: Constantly monitor MCCH during RRC_INACTIVE for possible updated PTM configuration will increase the UE power consumption.

Proposal 2: Recommend RAN2 not to discuss specifying enhancements to decrease UE power consumption due to monitor MCCH during RRC_INACTIVE.

Reference

- RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting #94-e. z