3GPP TSG-RAN WG2 #122	R2-2305383

Incheon, Korea, 22nd – 26th April 2023

Agenda Item:	7.14.4 Support of QoE measurements for NR-DC

Source:	Ericsson

Title:	QoE measurements in NR-DC

Document for:	Discussion

# 1	Introduction

As described in the WID on Enhancement on NR QoE management and optimizations for diverse services, NR-DC is one of the important commercial deployment scenarios for 5G networks and it is critical to support it in the NR QoE framework. Therefore, in the Rel-18 WID [1], the following objective is defined:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2].

- Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

Note 1: The QoE measurements are not performed separately for each leg.

- Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

- Specify the QoE measurement continuity in mobility scenarios in NR-DC.

- Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

Note 4: When the support for QMC in NR-DC is completed, how to reuse the NR-DC solution for supporting QMC in EN-DC should be considered, if time permits.

In RAN2#119bis, the following agreements related to NR-DC were taken:

Observation: Rel-18 QoE configuration may be created by MN or SN.

Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination.

1: In NR-DC scenario, both signalling-based and management-based QoE measurement collection shall be supported.

RAN2 assumes that there is a unique ID for QoE configurations across MN and SN. This can be accomplished by MN-SN coordination (e.g. similar as was done with measIds for NR-DC)

Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

In RAN2#121, the following agreements related to NR-DC were taken:

1: RRC configuration determines to which node UE sends the QoE report.  It is possible to change the reporting leg via RRC signalling after it has been configured.

3:	Split SRB for QoE reporting is not supported (unless serious problems are identified).

4:	Define new SRB (“SRB5”) for the QoE reporting to SN. SRB4 can only be configured for MCG (as in Rel-17). The priority of “SRB5” is lower than SRB1 or SRB3.

5:	If both MN and SN send the QoE configurations to the UE, MN and SN should not use the same set of identities.

RAN2 thinks it’s possible to have different m-based QoE configurations for UE in MN and SN if RAN3 allows it.

In RAN2#121bis, the following agreements related to NR-DC were taken:

1: Both SRB4 and SRB5 can be configured simultaneously.

7: SRB5 handling (setup, modification, release) is configured via SN RRC Reconfiguration message, and SRB5 should be released when the SCG is released.

8: According to the RAN2/RAN3 agreements, TS 37.340 can be updated based on the introduction of SRB5.

10: If SRB5 is configured, the SCG is not deactivated, UE can transmit the QoE reports related to SCG in MeasurementReportAppLayer message via SRB5.

11: RAN2 to agree the following RRC spec impacts with SRB5 introduced:

- 1)	SRB5 is for RRC messages which include application layer measurement report information (i.e. MeasurementReportAppLayer), all using DCCH logical channel.

- 2)	SRB5 has a lower priority than SRB3 and can only be configured by the network after AS security activation.

- 3)	Once AS security is activated, all RRC messages on SRB5 are integrity protected and ciphered by PDCP.

- 4)	Split SRB is not supported for SRB5.

- 5)	The integrity protection algorithm is common for SRB1, SRB2, SRB3 (if configured), SRB4 (if configured), SRB5 (if configured) and DRBs configured with integrity protection, with the same keyToUse value. The ciphering algorithm is common for SRB1, SRB2, SRB3 (if configured), SRB4 (if configured), SRB5 (if configured) and DRBs configured with the same keyToUse value.

- 6)	SRB5 release is supported, e.g. via srb5-ToRelease IE

13: UL segmentation can be used for message over SRB5.

14: As a baseline, Rel-17 pause/resume procedure is reused to pause/resume reporting of one or multiple QoE measurement configurations in a UE for NR-DC. Details are FFS, e.g. whether paused QoE reports can be reported to SN (if SN is not overload).

2: The network can optionally explicitly indicate the SRB for the QoE reporting if both SRB4 and SRB5 are configured. FFS on the granularity, e.g. per QoE config or otherwise.

3: MN- or SN-associated QoE reports can use either SRB4 or SRB5 if only one of SRB4 or SRB5 is configured for the UE. FFS whether network configuration is needed.

5: There is no feedback from AS to AL in case reporting SRB is changed. This means that an ongoing application layer measurement session in APP layer is not affected when the reporting SRB is changed. The reporting SRB can also be changed even if the application session (from AS layer point of view) is ongoing.

9: For NR-DC, if SRB5 is not configured (FFS on the SCG deactivation case), UE can transmit the SN-associated QoE reports via SRB4. FFS whether there are some ambiguities how MN knows where to forward this.

12: The UE resumes SRB5 (if configured) during RRC connection resume based on network indication (same as for SCG bearers in general).

18: FFS pending RAN3 decisions: Whether RVQoE reports and encapsulated QoE reports are reported together to the same node (MN or SN) in NR-DC.

4: For SRB switching in NR-DC scenario, FFS on the explicit indication and implicit indication, e.g. signaling impacts, details on UE/NW behaviours.

6: RAN2 can wait for more RAN3 progress on the alignment of MDT and QoE before discussing any issues.

In this contribution, QoE for NR-DC is discussed according to the objective defined in the WID [1].

# 2	Discussion

## 2.1	QoE configuration and reporting for NR-DC

In dual-connectivity, there are two procedures defined for communication between the UE and the SN. One procedure is with MN involvement and the other procedure is without MN involvement. The procedure with MN involvement is typically used when SRB3 is not configured, not available or not supported and the procedure without MN involvement can be used when SRB3 is configured and the reconfiguration does not impact the MN. These two procedures are described in TS 37.340. As an example, reconfiguration in the SN can be done by using one of the procedures for SN Modification:

SN initiated SN Modification with MN involvement

Figure 10.3.2-2: SN Modification procedure - SN initiated with MN involvement

[..]

SN initiated SN Modification without MN involvement

This procedure is not supported for NE-DC.

Figure 10.3.2-3: SN Modification – SN initiated without MN involvement

[..]

For UL messages initiated by the UE, two corresponding procedures are defined. The UE may either send the message directly using SRB3 or transparently via the MN. The procedure for transmitting the messages via the MN is the following:

SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information:

Figure 10.10.2-3: RRC Transfer procedure for SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information

Figure 10.10.2-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information from the UE:

1.	When the UE sends an SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information it sends it to the MN in a container called ULInformationTransferMRDC message as specified in TS 38.331 [4].

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received SN measurement report, failure information, SN UE assistance information, intra-SN CPC execution completion or IAB other information as an octet string.

This procedure is e.g. used for transmitting MeasurementReport to the SN, a very similar case as transmitting MeasurementReportAppLayer message to the SN.

The procedures for transmitting SN messages transparently via the MN were defined so that the procedures work also for the cases where the MN and the SN are from different vendors or of different RATs.

For QoE, the configuration of the UE using a transparent RRCReconfiguration message via the MN was agreed and included in the running RRC CR by a small update in the field description saying that the RRCReconfiguration message from the SN may also include the configuration of QoE. The corresponding UL procedure can be added by minor updates in RRC, allowing the MeasurementReportAppLayer message to be included in the transfer messages:

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR-DC dedicated information (e.g. for transferring the NR or E-UTRA RRC MeasurementReport message, the MeasurementReportAppLayer message, the FailureInformation message, the UEAssistanceInformation message, the RRCReconfigurationComplete message, the IABOtherInformation message or the NR or E-UTRA RRC MCGFailureInformation message).

Signalling radio bearer: SRB1, SRB3, SRB4

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

- ULInformationTransferMRDC message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERMRDC-START

ULInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

ulInformationTransferMRDC                   ULInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransferMRDC-IEs::=           SEQUENCE {

ul-DCCH-MessageNR                           OCTET STRING                    OPTIONAL,

ul-DCCH-MessageEUTRA                        OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, MeasurementReportAppLayer, RRCReconfigurationComplete, UEAssistanceInformation, FailureInformation, and IABOtherInformation messages when sent via SRB1 and to transfer the NR MCGFailureInformation message when sent via SRB3.

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport message when sent via SRB1 and to transfer the E-UTRA MCGFailureInformation message when sent via SRB3.

In RAN2#119bis, RAN2 agreed to use SRB4 as baseline for the QoE reporting and this is an implementation of that agreement. Using the existing procedures for QoE guarantees that QoE works also in multi-vendor (and multi-RAT) scenarios and it also simplifies the implementation. It also means that reports associated to the SN are correctly forwarded to the SN, without any new mechanism having to be introduced.

Reusing existing DC procedures for QoE guarantees support in multi-vendor and multi-RAT scenarios and decreases the complexity of the feature.

A MeasurementReportAppLayer message to the SN can be sent embedded in ULInformationTransferMRDC to the MN.

ULInformationTransferMRDC can be sent using SRB4.

In RAN2#121, the following agreement was taken:

1: RRC configuration determines to which node UE sends the QoE report.  It is possible to change the reporting leg via RRC signalling after it has been configured.

The agreement says that the RRC configuration determines to which node the UE sends the QoE report. There are three possibilities to configure the UE:

- The QoE configuration is included in RRCReconfiguration from the MN.

- The QoE configuration is included in RRCReconfiguration from the SN, the SN RRCReconfiguration is embedded in an MN RRCReconfiguration (SN modification with MN involvement, see above).

- The QoE configuration is included in RRCReconfiguration from the SN, sent via SRB3.

The corresponding default reporting options are (following the legacy principles):

- The QoE report is included in MeasurementReportAppLayer to the MN.

- The QoE report is included in MeasurementReportAppLayer to the SN, where the SN MeasurementReportAppLayer is embedded in an MN ULInformationTransferMRDC (RRC transfer procedure of SN message, see above).

- The QoE report is included in MeasurementReportAppLayer to the SN, sent via SRB5.

It is proposed to clarify these default options for QoE to avoid different interpretations. As mentioned above, reusing existing procedures guarantees that QoE works also in multi-vendor (and multi-RAT) scenarios and it also simplifies the implementation.

The following are the default options for QoE reporting, used unless the UE is instructed otherwise:

- If the QoE configuration was included in RRCReconfiguration from the MN, the QoE report is included in MeasurementReportAppLayer to the MN.

- If the QoE configuration was included in RRCReconfiguration from the SN, the SN RRCReconfiguration embedded in an MN RRCReconfiguration, the QoE report is included in MeasurementReportAppLayer to the SN, where the SN-bound MeasurementReportAppLayer is embedded in an ULInformationTransferMRDC to the MN. 

- If the QoE configuration was included in RRCReconfiguration from the SN, sent via SRB3, the QoE report is included in MeasurementReportAppLayer to the SN, sent via SRB5.

It was also agreed that the reporting leg can be changed for an ongoing measurement via RRC signalling. The new recipient node needs to acknowledge that it accepts receiving the reports before such a change can be done. The other node may e.g. not be able to receive reports due to overload or lack of support of QoE. This was agreed in RAN3:

The node that currently receives the QoE reports via the Uu can send a request to the peer node, asking that the QoE reporting leg is switched to the peer node.

The leg switch for QoE reporting needs to be approved by both nodes serving the UE.

In RRC, a reporting instruction to the UE needs to be defined. The reporting instruction needs to include the possible options for QoE reporting, i.e. report to MN via SRB4, report to SN transparently via SRB4 and report to SN directly via SRB5. If the network includes a reporting instruction in the RRC message, the UE follows that instruction. If there is no instruction included, the default reporting option is used. The reporting instruction is optional in the RRC message, as it is not needed when the default reporting option is used.

The RAN3 agreement copied above also means that the UE cannot send the reports to another node than the default node, if the network didn’t explicitly configure another reporting option. This is important as the other node may e.g. be overloaded, may be of a different vendor or have some other reason why it cannot receive QoE reports which the UE isn’t aware of. Therefore, the following is proposed:

Network configuration is needed for the UE to transmit reports to a different node than the default option for reporting.

The RRC reporting instruction for QoE reporting can include the options: 
- report to MN via SRB4, 
- report to SN transparently via SRB4, and 
- report to SN directly via SRB5.

The reporting options need to be configured per QoE configuration, as one reason for the instruction is that the node carrying the session should also receive the RVQoE reports. This is likely different for different sessions and different for different QoE configurations. Also, both the MN and the SN may configure the UE with QoE and the reporting instruction is likely different for the different configurations. If RAN3 agrees that the QoE and RVQoE reports can be sent to different nodes, the reporting instruction may need to be even further detailed, but that can await the outcome in RAN3.

The reporting instruction is configured at least per QoE configuration, i.e. per measConfigAppLayerId. FFS on separate indications for QoE and RVQoE reports.

If the SRB to be used for the default reporting option is not configured and the UE didn’t receive any instruction to report in another way, the configuration can be considered as erroneous and the UE may discard the reports. One example of this is if the network configured QoE using SRB3, but didn’t configure SRB5 and didn’t instruct the UE to report via SRB4.

If the default reporting option is not available and the network didn’t indicate any other reporting option, the UE may discard the QoE reports.

## 2.2	SCG deactivated or released

When a UE is configured with dual connectivity, the network may reconfigure the UE to single connectivity for various reasons, e.g. depending on the amount of data being transmitted. If the SN provided a QoE configuration to the UE while it was in dual connectivity, that configuration needs to be released when the SCG is released. Otherwise, the measurements may remain hanging in the UE.

QoE configurations configured by the SN are released in the UE when the SCG is released.

The SCG may be deactivated when the UE has a QoE report to send. In other cases when the UE has data to send and the SCG is deactivated, the UE sends a UEAssistanceInformation message to the network, indicating that it has UL data to transmit. This indication can be reused when the UE has a QoE report to transmit.

The UE sends a UEAssistanceInformation message indicating that it has UL data to send (according to existing procedures), if the SCG is deactivated when the UE has a QoE report to send.

Some companies brought up that it may be unnecessary to activate the SCG just to send a QoE report. However, encapsulated QoE reports may be very large and large reports would be reason enough to activate the SCG. RVQoE reports on the other hand are not large. It could be considered if the UE should indicate that it has a QoE/RVQoE report to send, in addition to indicating UL data. The network could then take proper actions and ask the other node if it is able to receive the reports instead and then tell the UE to use a different reporting option.

Discuss if the UE should indicate in UEAssistanceInformation that it is a QoE report that the UE has to transmit.

# Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Observation 1	Reusing existing DC procedures for QoE guarantees support in multi-vendor and multi-RAT scenarios and decreases the complexity of the feature.

Proposal 1	A MeasurementReportAppLayer message to the SN can be sent embedded in ULInformationTransferMRDC to the MN.

Proposal 2	ULInformationTransferMRDC can be sent using SRB4.

Proposal 3	The following are the default options for QoE reporting, used unless the UE is instructed otherwise:  - If the QoE configuration was included in RRCReconfiguration from the MN, the QoE report is included in MeasurementReportAppLayer to the MN.  - If the QoE configuration was included in RRCReconfiguration from the SN, the SN RRCReconfiguration embedded in an MN RRCReconfiguration, the QoE report is included in MeasurementReportAppLayer to the SN, where the SN-bound MeasurementReportAppLayer is embedded in an ULInformationTransferMRDC to the MN.   - If the QoE configuration was included in RRCReconfiguration from the SN, sent via SRB3, the QoE report is included in MeasurementReportAppLayer to the SN, sent via SRB5.

Proposal 4	Network configuration is needed for the UE to transmit reports to a different node than the default option for reporting.

Proposal 5	The RRC reporting instruction for QoE reporting can include the options:  - report to MN via SRB4,  - report to SN transparently via SRB4, and  - report to SN directly via SRB5.

Proposal 6	The reporting instruction is configured at least per QoE configuration, i.e. per measConfigAppLayerId. FFS on separate indications for QoE and RVQoE reports.

Proposal 7	If the default reporting option is not available and the network didn’t indicate any other reporting option, the UE may discard the QoE reports.

Proposal 8	QoE configurations configured by the SN are released in the UE when the SCG is released.

Proposal 9	The UE sends a UEAssistanceInformation message indicating that it has UL data to send (according to existing procedures), if the SCG is deactivated when the UE has a QoE report to send.

Proposal 10	Discuss if the UE should indicate in UEAssistanceInformation that it is a QoE report that the UE has to transmit.

# References

RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom