3GPP TSG-RAN WG2 #122	R2-2305384

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	 7.14.3 Rel-17 leftover topics for QoE

Source:	Ericsson

Title:	Event based RVQoE reporting

Document for:	Discussion

# 1	Introduction

As some of the features were not finalized in release 17, they are currently of interest as release 17 leftovers defined in the WID Enhancement on NR QoE management and optimizations for diverse services [1]. The current status of the leftovers is summarized as the latest RAN2 and RAN3 agreements and points for further discussions below.

In RAN3#118 meeting the following was agreed:

Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting.

Do not introduce the threshold-based trigger for reporting playout delay for media startup.

The final list of topics that are to be discussed in Rel-18:

RVQoE value (pending SA4 reply).

Assistance information for handling of QoE reporting upon RAN overload.

DU activation/deactivation/pause/resume of RVQoE reporting over F1.

DU participation in assembling the RVQoE configuration.

Event-based RVQoE reporting trigger.

In RAN2 #119bis-e the following was agreed:

For easy agreement:

Proposal 1: From RAN2’s perspective, there is no further work for slice-based QoE measurement.

Proposal 2: RAN2 can wait for RAN3 progress on enhancement to per-slice RAN visible QoE.

Proposal 3: RAN2 needs to wait for the progress of RAN3 on RVQoE value.

Proposal 10: The enhancement on UAI message to express the UE’s preference on QoE reporting configurations is not pursued.

Proposal 11: QoE reporting via unlicensed band is out of the WID scope.

For further discussion remain:

Proposal 4: It is proposed RAN2 to discuss possible options for event-based RVQoE, including benefits, spec impacts, and complexities.

Proposal 5: Whether to add the QoS flow ID in the RVQoE report is FFS.

Proposal 6: RAN2 discuss whether to send a LS to CT1/SA4 (related to P5).

Proposal 7: RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3).

Proposal 8: FFS on whether to send the priority information to UE.

Proposal 9: To wait for RAN3 decision on granularity of priority.

One of the other aspects addressed in this meeting in the contributions R2-2209837 and R2-2209845 was which of the layers, UE AS or UE application layer, should be evaluating whether the RVQoE events have happened.

In this contribution we focus of the event-triggered RVQoE reporting, RVQoE event evaluation and their importance beyond the context of release 17 leftovers.

# 2	Discussion

## 2.1	Event-based RVQoE

RAN visible QoE (RVQoE) provides a possibility of enabling a real-time radio resource optimization to improve a user’s quality of experience while the user is still in an ongoing application session. Currently, the RVQoE reporting is associated with the reporting of the legacy QoE reporting. However, the purposes of legacy QoE and RVQoE are different: the former is meant for the post-session analysis, while the latter is meant for real-time analysis and radio resource optimization (QoE reports are meant for evaluation of the quality of experience in such an entity as an OAM after the session usually is over, while the RVQoE reports are mostly useful for real-time improvement of the quality of experience for users in a current session by readily optimising the radio resources). Therefore, RVQoE reporting does not have to be the same as the QoE reporting with the only difference of the report recipient. By enabling and enhancing the event triggered RVQoE, a faster and more efficient optimization of the radio resources can be achieved directly for users in ongoing sessions enabling improved quality of experience “on the go”. In this section we discuss the importance of event triggered RVQoE reporting.

Extending RVQoE with the events to trigger the reporting, may reduce the report size as the reporting as RVQoE measurements do not have to be continuously logged and stored in the UE AS. Event based RVQoE additionally enables RVQoE to be more independent from the OAM configured QoE measurements and allow real-time optimization of radio resources to improve the quality of experience for the users. Without event triggered RVQoE the reports may continue to contain excessive data in the RVQoE reports that may not be useful to RAN at the time of the report arrival and may result in more difficult and slower metrics’ analysis.

The following options were discussed in the RAN2#119bis-e meeting:

- Option 1: gNB can configure an event to UE for triggering RVQoE measurement reporting, and it can be:

  - new Event for BufferLevel reporting: The BufferLevel is below a configured threshold. FFS which layer performs RVQoE event evaluation

- Option 2: gNB can configure an event to UE for triggering RVQoE measurement reporting, and it can be:

  - new Event for PlayoutDelay reporting: The PlayoutDelay is above a configured threshold. FFS which layer performs RVQoE event evaluation

- Option 3: RAN2 study what is the trigger event need to be defined for RVQoE, such as location, time, and some threshold.

- Option 4: Consider using RVQoE metrics fulfilling certain conditions, as triggers for RVQoE reporting.

- Option 5: Others (some companies propose enhancements but there are no details, so more inputs would be needed for understanding)

- Option 6: No enhancements

In the RAN3#118 meeting the following was agreed: Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting.
This agreement is like option 1 from RAN2119bis, suggesting introduction of a new event for RVQOE reporting, where the trigger for the event is the BufferLevel below a configured threshold.

Option 4, i.e., using RVQoE metrics fulfilling certain conditions as triggers for RVQoE reporting, encompasses options 1 to 3 as well and provides easier extensions of the list of trigger events in the future.

Option 6 may result in limited usefulness of RVQoE to RAN for real-time radio resources’ optimization.

Option 4 can be promoted as it includes such conditions as the buffer level falling below a certain level, more importantly, allows including additional RVQoE metrics as event triggers in the future.

Consider using RVQoE metrics fulfilling certain conditions, as triggers for RVQoE reporting.

Option 6 should not be considered as it imposes limitations on usefulness of RVQoE for RAN and on future development of RVQoE.

As the metrics of application layer measurements are sent to the UE AS layer, it could potentially be both the UE AS and the UE application layer, which, upon instruction from the network evaluates whether the conditions for the event-triggered are met. RAN may configure a UE to report when e.g., the buffer level becomes too low, hence the UE AS would know about the condition that has to be met to report back to the network.

However, if the UE application layer is allowed to evaluate whether the RVQoE was fulfilled, it could send only those RVQoE metric values which meet the defined threshold conditions or events directly to UE AS, immediately discarding values which do not meet those conditions, this way reducing the amount of reports collected by the UE AS. UE AS in its turn could directly forward the RVQOE data received from the UE application layer before sending a report to the RAN. This motivation is further strengthened by the reply LS from SA4 [2], stating that there already exists a mechanism for the application layer to be configured for QoE reporting, which could be reused for threshold-based RVQoE reporting. Additionally, the application layer can make a threshold-based decision and submit the QoE reports in a timelier manner than UE AS layer, leading to smaller delay between a threshold occurrence and the next scheduled QoE report. With the above discussion in mind, we propose the following:

The UE application layer evaluates whether conditions for the event- or threshold- triggered RVQoE reporting are met before the RVQoE report is sent to RAN.

There is also a multitude of other events in the UE AS layer, which can reflect the changed, that is worsened or improved, quality of experience for a user, to be considered as trigger events for the event-triggered RVQoE reporting. Considering the following UE mobility, handover and NR-DC aspects specified in the Release 18, list of the RVQoE events could be even further expanded to be able to learn in more detail how these aspects affect the quality of experience, as RVQoE would enable a real-time evaluation of quality of experience under rapidly changing radio conditions. Examples of such events are handovers, radio link failure, change in reporting path in dual connectivity etc. These events can be considered as viable trigger events for RVQoE reporting.

Events in UE AS layer can trigger event-based RVQoE reporting. FFS on the type of events.

# Conclusion

RAN2 is kindly asked to discuss the following proposals:

Observation 1	In the RAN3#118 meeting the following was agreed: Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting. This agreement is like option 1 from RAN2119bis, suggesting introduction of a new event for RVQOE reporting, where the trigger for the event is the BufferLevel below a configured threshold.

Observation 2	Option 4, i.e., using RVQoE metrics fulfilling certain conditions as triggers for RVQoE reporting, encompasses options 1 to 3 as well and provides easier extensions of the list of trigger events in the future.

Observation 3	Option 6 may result in limited usefulness of RVQoE to RAN for real-time radio resources’ optimization.

Proposal 1	Consider using RVQoE metrics fulfilling certain conditions, as triggers for RVQoE reporting.

Proposal 2	The UE application layer evaluates whether conditions for the event- or threshold- triggered RVQoE reporting are met before the RVQoE report is sent to RAN.

Proposal 3	Events in UE AS layer can trigger event-based RVQoE reporting. FFS on the type of events.

# References

RP-223488, Revised WID: Enhancement on NR QoE management and optimizations for diverse services, China Unicom https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_120/Docs/R2-2213054.zip

S4-230684, Reply LS on buffer level threshold-based RVQoE reporting, 3GPP TSG-SA WG4 Meeting #123-e, 17-21 April 2023