3GPP TSG-RAN WG2 #122	R2-2305386

Incheon, Korea, 2023-05-22- 2023-05-26

Agenda Item:	7.4.4 CHO including target MCG and candidate SCGs for CPC CPA in NR-DC

Source:	Ericsson

Title:	CHO with associated CPC/CPA

Document for:	Discussion, Decision

# 1	Introduction

The rel-18 work on further NR mobility enhancements is described in the WID in [1]. The WID includes an objective related to specifying CHO configuration including both a target MCG and target candidate SCGs for CPAC (objective 4):

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

  - CHO including target MCG and target SCG is used as the baseline

In RAN2#119-e the following agreements were reached for this objective:

Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17.

CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported.

FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

In RAN2#120 one more agreement was made:

Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation)

In RAN2#121 the following agreements were made:

RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18

The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

In RAN2#121bis the following agreements were made:

- For the CHO+CPC case:

When both CHO and CPC conditions are met, both CHO and CPC cell change is executed.

Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).

Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

In this contribution different aspects and open issues related to objective 4 in the WID on rel-18 work on further NR mobility enhancements ([1]) are discussed.

# 2	Discussion

## 2.1	CHO including target MCG and associated CPC/CPA

### 2.1.1	Fulfilment of execution conditions

In RAN2#121bis it was agreed that both the execution conditions for CHO and the execution conditions for CPC/CPA need to be fulfilled before the UE executes the conditional reconfiguration, in case CHO with associated CPC/CPA has been configured. There is still an FFS related to whether the UE in some cases could trigger CHO if only the CHO conditions are fulfilled and whether the UE could continue monitoring the CPC/CPA conditions after the CHO configuration has been applied.

When dual connectivity has been configured, the MCG configuration may be different for different SCGs and therefore the MCG configuration and the SCG configuration cannot be assumed to be configurations that can be handled separately. Therefore, it is not possible to trigger UE autonomous actions related to separating the MCG and the SCG configurations. If e.g. the CHO conditions would be fulfilled, but not the CPC conditions, the UE cannot apply only the MCG configuration and release the SCG on its own. If an SCG is released, the network in most cases needs to reconfigure the bearers in the UE to adjust the configuration to an MCG-only configuration. The UE cannot make such a reconfiguration without a reconfiguration message received from the network. This is also the reason why it was agreed that the network needs to include the release of any associated SCG in the CHO configurations in rel-16, as CHO with SCG is not supported in rel-16. The network has the option to configure a CHO-only configuration in case it wants to ensure that the UE will always execute CHO when the CHO conditions are fulfilled.

A UE cannot autonomously release the SCG as the bearers likely need to be reconfigured at an SCG release.

Do not consider the FFS that “the UE in some cases could trigger CHO if only the CHO conditions are fulfilled”.

The network has the option to configure a CHO-only configuration (with or without a (fixed) SCG) in case it wants to ensure that the UE will always execute CHO when the CHO conditions are fulfilled. Another option would be that the network configures the current PSCell as a candidate PSCell so that the radio condition of the current PSCell is also taken into account. Such a configuration would increase the probability of fulfilled CPC conditions when the CHO conditions are fulfilled. The conditional events may need to be updated to support such a configuration.

Immediate execution of CHO when the CHO conditions are fulfilled can be achieved by network configuration.

Investigate if the current PSCell can be configured as a candidate PSCell in configuration of CHO with candidate SCGs for CPC/CPA.

If the network provided a CHO-only configuration and the UE applied that configuration, e.g. in the case that the CPC/CPA conditions were not fulfilled, it may be desired that the UE continues monitoring the CPC/CPA conditions after the CHO was executed. The UE cannot continue monitoring execution conditions related to the source MN after it has performed a handover to the target MN, assuming the execution conditions for CHO with associated CPC/CPA relate to the source MN measConfig, see section 2.1.2. However, the network may provide CPC/CPA configurations within the CHO configuration that is applied upon execution and these CPC/CPA configurations may relate to the target MN measConfig. The UE can then start monitoring the execution conditions for CPC/CPA directly after the CHO was executed and a possible CPC/CPA can be executed quickly. The only change that is needed for this is to remove the restriction that a conditional reconfiguration cannot contain another conditional reconfiguration.

condRRCReconfig

The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config.

This update resolves the FFS from RAN2#121bis “IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.”.

Remove the restriction that a conditional reconfiguration cannot contain a conditional reconfiguration.

### 2.1.2	Configuration of execution conditions

In RAN2#121, RAN2 agreed on simultaneous evaluation of CHO with associated CPC/CPA configuration. RAN2 also agreed that the UE does not need to decode the message that is to be applied when the CHO conditions are fulfilled, before the conditions actually are fulfilled.

For CHO with associated CPC/CPA, the CHO conditions will relate to the source MN measConfig, but for the candidate SCG measurements configured by the target MN, i.e. CPA or MN-initiated CPC, the measurements will relate to the target MN measConfig. However, the UE is not required to decode the target MN measConfig, at least not if it is included inside the message that is applied when the CHO conditions are fulfilled.

It would be possible to extract the target MN measConfig in the network side and send it outside the message that is applied when the CHO conditions are fulfilled, i.e. including it in the message where the conditional reconfiguration for CHO is included. This still is a bit complex for the UE, as the UE would have to perform measurements for separate MCG measConfigs, the source MN measConfig and a measConfig for each candidate target MN. Also, the source MN would have to check each target MN measConfig to ensure that the UE capabilities are not exceeded in total.

It may be easier for the UE if the source MN merges the measurement configuration for CPA/CPC (to be evaluated together with CHO) from the different target MNs into the source MN measConfig, so that the UE only gets one MCG measConfig. The source MN may not be able to merge everything that the target MNs request for, as the UE capabilities may be exceeded or the capabilities of the source MN may be exceeded. In such case the source MN needs to reject the configuration of certain SCG candidates.

The source MN merges the source MN measConfig and the measurement configuration for CPA/CPC (to be evaluated together with CHO) from each target MN measConfig so that only one overall MCG measConfig is sent to and applied by the UE.

### 2.1.3	Configuration of SN-initiated CPC

The configuration of CPC associated with a CHO candidate is in principle MN-initiated, as the SN is not yet connected to the candidate MN and has no information that CHO with candidate SCG is being configured. In a discussion for NR-DC with selective activation, the use of A3/A5 events related to the PSCell for MN-initiated CPC has been discussed. Several companies think it is not needed, as SN-initiated CPC would solve the problem. In the case of CHO with candidate SCG, the MN would have to trigger the SN-initiated CPC, so that the SN gets the information that it should configure SN-initiated CPC. The conditions for the SN-initiated CPC can then refer to the serving SN measConfig. Alternatively, A3/A5 events related to the PSCell could be supported for MN-initiated CPC, i.e. the same as has been proposed for NR-DC with selective activation.

The MN can trigger the serving SN to configure SN-initiated CPC for CHO with CPC candidates.

Alternatively, A3/A5 events related to the serving PSCell can be configured for MN-initiated CPC.

### 2.1.4	Signalling of CHO with CPC/CPA configurations

In legacy, the UE applies an RRCReconfiguration message when the associated execution condition(s) are fulfilled. At CHO with SCG, the RRCReconfiguration contains both the configuration of an MCG and the configuration of an SCG in an included RRCReconfiguration message. As discussed above, the MCG configuration may be different for different SCGs and therefore the MCG configuration and the SCG configuration cannot be assumed to be configurations that can be handled separately. Therefore, it is preferred to follow the legacy principle where the network provides one RRCReconfiguration message (including another RRCReconfiguration message for the SCG) for each target CHO and CPC/CPA combination.

One RRCReconfiguration message containing the configuration of both a target MCG and a target SCG is applied by the UE when the execution conditions for CHO with CPC/CPA are fulfilled.

In Annex A, two different TPs are shown. In a first option, there is one condReconfigToAddMod instance for each candidate PSCell. This means that there can be multiple condReconfigToAddMod instances for one PCell, one for each candidate PSCell.

In the second option, there is one condReconfigToAddMod instance for one PCell and that instance can contain multiple PSCell candidate configurations. The second option has the benefit of consuming less condReconfigIds. There is also always a CHO-only (rel-16 or rel-17) target configuration included for each PCell for the case that the none of the associated CPC/CPA are fulfilled.

RAN2 to decide on a preferred TP for CHO with associated SCG.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A UE cannot autonomously release the SCG as the bearers likely need to be reconfigured at an SCG release.

Observation 2	Immediate execution of CHO when the CHO conditions are fulfilled can be achieved by network configuration.

Based on the discussion in the previous sections the following is proposed:

Proposal 1	Do not consider the FFS that “the UE in some cases could trigger CHO if only the CHO conditions are fulfilled”.

Proposal 2	Investigate if the current PSCell can be configured as a candidate PSCell in configuration of CHO with candidate SCGs for CPC/CPA.

Proposal 3	Remove the restriction that a conditional reconfiguration cannot contain a conditional reconfiguration.

Proposal 4	The source MN merges the source MN measConfig and the measurement configuration for CPA/CPC (to be evaluated together with CHO) from each target MN measConfig so that only one overall MCG measConfig is sent to and applied by the UE.

Proposal 5	The MN can trigger the serving SN to configure SN-initiated CPC for CHO with CPC candidates.

Proposal 6	Alternatively, A3/A5 events related to the serving PSCell can be configured for MN-initiated CPC.

Proposal 7	One RRCReconfiguration message containing the configuration of both a target MCG and a target SCG is applied by the UE when the execution conditions for CHO with CPC/CPA are fulfilled.

Proposal 8	RAN2 to decide on a preferred TP for CHO with associated SCG.

# References

RP-221799, Revised WID on Further NR mobility enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #96, June 6-9, 2022

# Annex A – Text Proposal

- Below is a Text Proposal to TS 38.331, 17.3.0

- Option 1:

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

associatedCPACExecutionCond-r18  CHOICE {

condExecutionCond-r18            SEQUENCE (SIZE (1..2)) OF MeasId,

condExecutionCondSCG-r18         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17)

}                                                                          OPTIONAL   -- Need M

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

- Option 2:

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

associatedCPACConfigList-r18         SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF AssociatedCPACConfig-r18                OPTIONAL   -- Need M

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

AssociatedCPACConfig-r18	::=     SEQUENCE {

condExecutionCond-r18            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL, -- Need M

condExecutionCondSCG-r18         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL, -- Need M

condRRCReconfig-r18              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,

...

}

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP