3GPP TSG RAN WG2#122	           R2-2305392

Incheon, Korea, 22nd – 26th May 2023

Agenda item:	7.2.2

Source: 	Lenovo

Title: 	On SL Positioning Architecture Aspects

Document for:	Discussion

# Introduction

The Rel-18 NR expanded and improved positioning WID was approved upon during the RAN#98-e [1] meeting:

During the RAN2#121 [2] meeting, the following SL Positioning agreements were made:

Agreement: Use “SLPP” (without hyphen) as the name of the new protocol. Agreements: Regarding the structure of SLPP, e.g. general part, procedure part , Information Element Abstract Syntax Definition, the structure of LPP (TS 37.355) can be used as baseline for further discussion. The content of each section will be added in accordance with future agreements, not based on LPP legacy directly. FFS on procedure description in the field description as LPP. Regarding the ASN.1 part of SLPP, follow NR RRC approach, e.g. - FFS on Need code (e.g. how to support no UL/DL), support of delta signalling - Define ASN.1 elements for common UE capabilities in a dedicated section (i.e. “UE capability information elements”); FFS whether any positioning method specific capability IEs should be grouped by positioning method. - Common section for constraints - “nonCriticalExtension” at message level - Fields in the field description are sorted based on alphabetical order - FFS on whether setup release structure should be introduced in SLPP Agreement: PC5-U is used for transport of SLPP. Agreement: With respect to the overall signaling procedure for PC5-only positioning (including at least IC and OOC; FFS if there are differences for PC), it is proposed to agree that the sidelink positioning procedure comprises the following series of steps as a baseline, between the LMF/positioning server UE/NG-RAN/candidate Anchor UE(s) and Target UE(s): - Triggering event - Sidelink positioning capability exchange 3. Sidelink positioning assistance data transfer 4. SL Positioning Request Location Information 5. Measurement of SL-PRS 6. Location calculation 7. SL Positioning Provide Location Information Some steps may have dependencies on SA2 and can be revisited in this light. The order is subject to further discussion. FFS if discovery and selection of anchor UEs and/or server UE are part of the positioning layer in RAN2 scope. LS to SA2 to ask for confirmation and guidance on the SA2 aspects. Agreement: RAN2 do not intend to discuss assistant UE functionality in Rel-18. To be indicated in the LS to SA2 in discussion [409]. Direct ranging between two UEs with the server providing assistance to both sides is not precluded. Agreement: RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.

Further progress was made during the RAN2#121bis-e [3] meeting, the following SL Positioning agreements were made:

Agreement:

LS on group positioning is postponed for a reply from next meeting.

WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.

WA: At least part of the group management for group positioning is performed at upper/application layer.

Agreements:

Sidelink Positioning Architecture in Figure 1-4 on R2-2304301 is taken as baseline in TS 38.305 for further discussion.

RAN2 understanding is that there is no impact to LTE specs from this objective.

Agreement:

Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2.  Server UE can be further

discussed at least for the case that the server UE is separate from the target and anchor.

Agreement:

Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario.

Agreement:

Anchor UE selection can be included in the sidelink positioning procedures at least for out of coverage scenario.

Agreement:

RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection

of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles.  FFS how much

information is indicated about anchor UEs (e.g., knowledge of location).

The UE role information is indicated in the discovery SLPP metafield.  FFS if this applies to both discovery modes and

which messages.

Agreement:

R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location

related measurements/location estimates, to transfer assistance data, or to exchange of capabilities.

RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in

case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request.

TP in R2-2304005 is postponed.

RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction

ID in order to associate messages with one another (e.g., request and response)”

RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

In addition, various RAN2 secondary objectives were also defined, which require some degree of coordination with RAN1 and RAN3. This contribution provides a detailed discussion into the various areas to develop in order to support the necessary functionality and procedures to perform SL positioning.

# SL Positioning Architecture

During the RAN2#121bis-e meeting [4], the following architecture was agreed upon as baseline for TS38.305:

Figure 1: SL Positioning baseline architecture

The Sidelink positioning architecture in Figure 1 aims to cover different coverage conditions, including in-coverage, and out-of-coverage scenarios. UE-A is defined to be an in-coverage dual RAT UE with connection to an ng-eNB and gNB, UE-B is defined to be an NR-only UE, UE-C and UE-D are meant to be out-of-coverage UEs. To address the NR-only partial coverage scenario, an FFS has been added to the link between UE-C and UE-B. The SL positioning should encompass all scenarios including the partial coverage scenario.

Proposal 1: RAN2 to confirm the NR PC5 link between UE-C and UE-B as part of the NR-only partial coverage scenario in the SL Positioning baseline architecture.

Since UE-B is deemed to be an NR-only UE, the LTE Uu link between the ng-eNB and UE-B is not required. Therefore, the “FFS LTE-Uu” link between UE-A and ng-eNB may be removed.

Proposal 2: RAN2 to remove the “FFS LTE-Uu” link between UE-B and ng-eNB.

The general architecture and signaling procedures have been studied for two operational scenarios, i.e., PC5-only positioning and combination of Uu- and PC5-based positioning. SA2 has similarly aligned two types of operation including UE-only and Network-assisted operation.

The Uu positioning architecture is primarily based on the involvement of the LMF and as such can only operate in in-coverage scenarios with a network deployment supporting an LMF. In the case of SL positioning, the use cases span across different vertical applications (e.g., V2X, Public Safety, IIoT and Commercial ranging), which have different requirements in terms of coverage scenario operation. The proposed SL positioning architecture should also support partial and out-of-coverage scenarios. This aspect was extensively studied during the RAN study captured in TR 38.845, where the SL positioning framework should cater to all coverage scenarios.

Furthermore, due to the distributed nature of SL positioning there are multiple entities which may act as configuration and positioning calculation entities, e.g., RSU, gNB, target-UE, anchor-UE, positioning reference units, etc., which is different from the Uu operation where the LMF was responsible for both positioning configuration and positioning calculation. It is, therefore, important to consider the involvement of such entities within the SL positioning architecture.

SA2 has also further concluded that the LMF may be involved in the SL positioning procedures between a Target UE and Anchor UE, which may be a standalone extension of the LPP, SLPP or both. RAN2 has further agreed such aspects including whether or not to extend the existing protocols may be best considered during the normative work. Based on the LMF involvement, the UE may need to support both LPP and SLPP. This may be inherently limiting for UEs wishing to only perform ranging with respect to other UEs in the network, which only support SLPP.

Observation 1: For UEs wishing to only perform ranging in-coverage and without LPP capabilities, LMF involvement may be unnecessary.

Proposal 3: RAN2 to support both LMF-dependent (network-assisted operation as defined by SA2) (e.g., UEs supporting both LPP and SLPP) for in-coverage scenarios and partial coverage scenarios and LMF-independent (UE-only operation as defined by SA2) (e.g., UEs only supporting SLPP) SL positioning architectures for all coverage scenarios including in-coverage, partial coverage and out-of-coverage.

Specifically, the partial coverage scenario can be further classified into two cases: 1) target UE is in coverage and at least one of the anchor UEs is out of coverage; 2) target UE is out of coverage and at least one of the anchor UEs is in coverage.

For case 1, the signalling and procedures to support positioning functionalities can partially reuse the design of in-coverage scenario except that LMF may get to the OOC anchor UEs via target UE if needed for LMF-dependent SL positioning architecture.  While for case 2, target and anchor UEs may perform SL positioning for both LMF-dependent and LMF-independent architectures. For LMF-independent architecture, the signalling and procedures can reuse the design of out-of-coverage scenario. For LMF-dependent architecture, we noticed that following use cases are noteworthy:

Network triggered positioning for OOC target UE: SA2 addresses the issue on network triggered positioning for OOC target UE. AMF initiates the location request to LMF with the information of IC candidate anchor UEs. LMF may request candidate anchor UEs to discover target UE and provide necessary information.

Anchor UE relocation initiated from SL positioning: Assuming that OOC target UE is performing standalone SL positioning, the IC anchor UE may be required to relocate itself via Uu positioning to eliminate the positioning error caused by the inaccurate position of anchor UEs when some positioning errors are detected.

Limited UE capability on location estimate: The capability of OOC target UE is restricted and cannot support the position calculation or location estimate. It may request assistance from LMF via in coverage anchor UEs.

Considering above use cases show strong motivation to support the partial coverage scenario in which target UE is out of coverage and at least one of the anchor UEs is in coverage, it is suggested that RAN2 to discuss the signalling and procedures related to this.

Proposal 4: It is suggested that RAN2 to discuss the procedures and signalling design for partial coverage case in which a target UE is out-of-coverage and at least one of the anchor UEs is in coverage.

## SL Positioning Discovery

RAN2 also made some agreements regarding the SL Positioning Discovery procedures, whereby discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE and server UE roles. This enables the higher-layers to be aware of the types of UEs involved in a SL positioning session including Anchor UEs, Server UEs and target-UEs. An issue of further discussion includes whether or not a “Located UE” (defined by SA2) should be indicated to the higher-layer discovery procedures. From RAN2 perspective no such distinction may be required as “Located UE” is a subset of an anchor UE (“Reference UE” with known location as defined by SA2). The location of the anchor UE may or may not be provided to the higher-layers for discovery purposes as part of the SLPP metadata field provided that there are no privacy concerns of providing such location information to the higher-layer discovery procedures.

Proposal 5: From RAN2 perspective, there is no separate “Located UE” to be discovered as the anchor UE may choose to provide its own location to the higher-layer discovery procedures, if required provided that there are no privacy concerns. The anchor UE (Reference UE) location may be optionally provided in the SLPP metadata field.

Indication of SLPP support may also be considered as useful high-level information to enable efficient discovery procedures of suitable SL Positioning UEs. This enables the higher-layers to only consider UEs with SLPP support as part of SL positioning discovery procedures.

Proposal 6: RAN2 to confirm indication of SLPP support as part of the SLPP metadata field, in addition to the UE roles.

Further discussion would also be needed on the model discovery framework for the different SL positioning entities, i.e., Model A or Model B discovery methods defined in TS 23.303. Depending on the type of discovery model, candidate SL positioning UEs may be provided from the higher-layers, whereby further filtration of UEs, e.g., Anchor may be performed in the AS layer based on the real-time radio conditions and resource availability.

## Identification and Selection of Anchor UEs

In general, SL Positioning targets both absolute and relative positioning requirements. For each of these cases, it is expected that anchor UEs with relative stable mobility patterns and known UE locations, may be configured to participate in SL positioning/ranging session. This is especially required for positioning techniques requiring 2 or more anchor nodes for enhanced positioning performance, e.g., in the case of SL-TDoA.

Observation 2: Anchor nodes with known locations such as UEs and RSUs can enable certain SL positioning techniques such as SL-TDoA.

One of the key responsibilities of anchor nodes, would be to transmit/receive SL-PRS, perform SL positioning measurements and reporting, as well as any other key positioning-related tasks. In the case of anchor UEs, there are additional issues worth studying including the selection, triggering and configuration of anchor nodes in the AS layer, to perform the aforementioned tasks. This is required since higher-layer may provide a candidate set of anchor UEs, wherein the suitability of the anchor nodes depends on the anchor UE position, radio/channel conditions between Anchor UE and Target-UE and so forth.

Furthermore, this should be studied for both LMF-dependent SL positioning architecture and LMF-independent SL positioning architecture, to support in coverage, partial coverage and out of coverage scenario. For the LMF-dependent SL positioning architecture, the LMF may need to perform Anchor/Server UE management to the anchor nodes in addition to legacy target UE, while for LMF-independent SL positioning architecture, anchor node management (e.g., including (re)selection, triggering and configuration) may be controlled by gNB if in-coverage or determined by an UE that starts/initiates the SL positioning session in a distributed fashion, and therefore the corresponding coordination procedures need to be studied.

There are certain Radio parameters which can be utilised to select optimal anchor UEs from a candidate list of anchor UEs provided by the discovery methods based on:

- Reported LOS/NLOS indication, which implies the selection of anchor UEs classified with LOS links

- Reported SL measurements, e.g., SL PRS RSRP, other SL RS RSRP metrics, which indicates the link quality between target-UE and anchor UEs.

- Coverage scenario of the potential anchor UEs, e.g., in-coverage, partial coverage and/or out-of-coverage

- Synchronization related information, e.g., RTD offset, synchronization source information, which helps minimize synchronization errors amongst candidate anchor UEs as well ensure anchor UEs have the same synchronization source.

- Specific UE type, e.g., RSU, or UEs with specific velocity, direction/ angle, or UEs supports specific positioning methods, etc.

Proposal 7: RAN2 is recommended to support procedures related to anchor UE(s) including triggering, (re)selection, and configuration when performing sidelink positioning procedures.

Proposal 8: Anchor UE selection radio-based criteria may be based on at least reported LOS/NLOS indication, reported SL measurements, e.g., SL PRS RSRP, coverage of anchor UEs, synchronization related information, specific UE type. FFS any further Anchor UE selection criteria.

## SL Group Positioning

SA3 [5] and SA2 [6] have provided their respective replies on the issue of groupcast and broadcast positioning use cases and security issues for SL positioning.

SA3 Reply [5]: SA3 has preliminarily agreed that unprotected broadcast/groupcast signaling has security issue, therefore ciphering and integrity protection are required for the signaling transferred using groupcast/broadcast. Currently, ciphering and integrity protection are not supported for broadcast/groupcast on sidelink. SA3 has also preliminarily agreed that it is feasible to work on the corresponding requirements and solutions for protecting broadcasted/groupcasted SL positioning signaling when the basic groupcast/broadcast procedure is stable. Therefore, SA3 would appreciate more information (e.g. user cases for broadcast/groupcast, detailed operations) from RAN2 for developing feasible solutions, if any. In addition, SA3 would also appreciate feedback from SA2 on group management for SL positioning signalling groupcast. SA2 Reply [6]: Regarding the questions from RAN2 on the use cases for groupcast/broadcast signalling for SL positioning, SA2 would like to point out that for the V2X use cases, for example those documented in TR 38.845, there are use cases of involving Road Side Unit (RSU) and multiple vehicles, or multiple vehicles in a platoon, which would benefit from groupcast/broadcast signalling support. Additionally, the MCX service that relies on group communication would also benefit from groupcast/broadcast signalling support. In addition, for Ranging based services, as documented in TR 22.855, there are multiple use cases that involve groups of UEs performing Ranging/Sidelink positioning services, e.g. Museum Tour, etc. Furthermore, SA2 would like to also point out that SA2 has concluded group management in the attached pCR S2-2301782.

SA3 has determined the need for ciphering and integrity protection for SL positioning broadcast and groupcast messages. SA3 may further work on security management of the SL positioning groupcast and broadcast message pending stability of the SL positioning groupcast/broadcast procedures. In this case, RAN2 can start work on SL positioning signalling and procedure for groupcast/broadcast. More specifically, two use cases for groupcast/broadcast can be further studied:

- One target UE and multiple anchor UEs in one group: as captured in TR 38.859, in addition to unicast, SL positioning capability, assistance data etc. can be sent by groupcast/broadcast.

- Multiple target UEs and one or multiple anchor UEs in one group: i.e. SL group positioning, with multiple UEs performing Ranging/Sidelink positioning services, e.g. Museum Tour, etc. In line with SA2, the group positioning involving the collection of location estimates for multiple target UEs (absolute positioning) or multiple UE pairs (ranging/relative positioning) in each LCS request.

SA2 has indicated that there are indeed use cases that may benefit from performing SL positioning within a group. They are also further concluded in the attached pCR [6], that the group management can be performed at the application layer, however it has also been noted that group management at the SLPP layer is also out-of-scope of SA2. There is therefore sufficient motivation and use cases to enable support of group positioning. In addition, it is up to RAN2 to decide on the need for group management at the SLPP layer, and there are benefits of performing group management at the SLPP layer, including but not limited to: 1) selection/addition/removal of anchor nodes, 2) SL radio resource management including resource allocation and 3) Dynamic group management based on the number of available supported SL positioning/ranging UEs. It can be noted that such management functionality can be transparent to the Application layer.

During RAN2#121bis-e meeting [3], two working assumptions were made to progress on the positioning of multiple target UEs within a group. The first working assumption details the understanding of group positioning involving the collection of location estimates for numerous target UEs (absolute positioning) or multiple UE pairs (ranging/relative positioning) in each LCS request, aligned with SA2’s understanding, while the second working assumption deals with at least part of the group management for group positioning is performed at upper/application layer.

Proposal 9: RAN2 to confirm the first and second working assumption made during RAN2#121bis-e on group positioning involving multiple target UEs.

Proposal 10: RAN2 to further discuss the remaining aspects, which are in scope of the SLPP Layer Group management, e.g., addition/re-selection/removal of SL positioning UEs.

## SL Positioning ID Management

SA2 has also requested the RAN WGs input for several RAN parameters in their earlier LS [7], including the type of identifiers required to support SL positioning. SL communications makes self-generated source-IDs and higher-layer generated (e.g., application-layer) destination IDs, in order to support different communication cast types. Similarly, in order to identify the different types of positioning nodes/UEs involved, RAN2 is tasked to study whether existing SL UE ID management is sufficient or whether additional SL positioning ID management needs to be supported in the new positioning layer, e.g., to identify and differentiate between anchor UEs and server UEs by means of separate IDs.

Proposal 11: RAN2 to further study whether existing/new SL ID management needs to be supported for SL positioning to support new UE-types such as anchor UEs and/or server UEs.

## SL Positioning UE Roles

### SL Positioning Server UE

SA2 has clarified that the role of a SL Positioning Server UE is applicable for partial coverage and out-of-coverage scenarios and only in cases that a constrained UE is not able to support all the SL Positioning/Ranging features. In addition, such UEs may be discoverable and further selected for result calculation purposes only.

The need for a separate (SL Positioning) Server UE is not so clear from RAN perspective, since it caters to only certain limited scenarios in SL positioning based on coverage situations and UE capabilities. It is assumed that a target UE and anchor UEs have sufficient capabilities to perform UE-only SL Positioning including, configuration, measurements and result calculation across all coverage scenarios. The scope of introducing additional discovery and selection procedures as well as additional resource coordination for a separate UE may increase the complexity of the SL positioning framework. However, the aforementioned issues may be resolved, if the SL Positioning Server UE are considered part of the Anchor UE or Target-UE.

Proposal 12: RAN2 to consider support of “SL Positioning Server UE” as a functionality part of an Anchor UE or Target-UE.

## SL Positioning Location Services and QoS Parameters

The types of location services for SL positioning will impact the supported positioning modes, i.e., how the SL UE-based or SL UE-assisted positioning is defined. Although, the types of location services are to be specified by SA2, the corresponding service will directly impact the type of SL positioning modes specified under different coverage scenarios.  The extent to which other network entities such as AMF and GMLC will impact each of the SL positioning location services need to be also further considered, especially in-coverage and partial coverage scenarios. SA2 has provided supporting procedures for SL-MT-LR [8] and SL-MO-LR [9]. RAN2 may further confirm with SA2 on the supported location services for SL positioning including NI-LR, Immediate Location requests and Deferred location requests which can be re-used as in Uu positioning. Further discussion may be required on the applicability of these request types under different coverage scenarios, e.g., in-coverage, partial coverage and out-of-coverage.

Proposal 13: RAN2 to confirm with SA2 on the supported location services for SL positioning including NI-LR, Immediate Location requests and Deferred location requests which can be re-used as in Uu positioning. FFS the applicability of these location service requests under different coverage scenarios.

SA2 has concluded that the 5GC authorizes and provisions the PQI to the UE with a mapping to the Ranging/SL positioning service when PC5-U is used as the SLPP transport layer. The assumption in this case is that the PC5 user plane architecture is used as the SLPP transport layer.

The positioning (LCS) QoS parameters, including accuracy and latency of direction and distance, may also be authorized and provisioned according to SA2. They further expect that the RAN WGs to evaluate if that's a correct understanding of the positioning QoS parameters.

From RAN2 perspective, the positioning QoS parameters may include and have already been communicated to SA2:

- Horizontal/Vertical Absolute Positioning Accuracies, verticalCoordinateRequest (in meters)

- Horizontal/Vertical Relative Positioning Accuracies (in meters)

- Ranging for Distance Accuracy (in meters)

- Ranging for Direction Accuracy (in degrees or radians)

- Response Time for absolute/relative position and ranging

- Velocity request.

RAN1 had sent a reply LS [10], confirming mentioning that in the case of relative positioning and ranging, only relative velocity w.r.t another UE can be estimated.

Proposal 14: RAN2 to mention in SA2 reply LS that RAN1 has confirmed the relative velocity QoS parameter.

# SL Positioning Techniques

## SL RAT-dependent Positioning techniques

The potential SL positioning/ranging techniques should be able to support different scenarios, especially given that Anchor UEs (whose location may or may not be known), Target-UEs, and possibly SL Positioning Server UEs can be considered given the distributed nature of SL communications. The SL RAT-dependent positioning techniques to be supported may be summarized as follows:

- SL timing-based positioning methods

  - SL-TDoA

  - SL-RTT (including one-way and two-way RTT methods)

- SL angular-based positioning methods

  - SL-AoA

RAN1 has already agreed on a few supported SL positioning methods including SL-RTT, SL-AoA and SL-TDoA. Furthermore, measurements such as SL PRS RSRP, SL PRS RSRPP, SL RTOA, have been agreed during the RAN1#112 meeting [11]. The measurement configuration and reporting of such measurements may be signalled using the agreed functionality of SLPP Request Location Information and SLPP Provide Location Information.

Proposal 15: RAN2 to support the signalling design for the agreed RAN1 SL Positioning measurements including measurement configuration using SLPP Request Location Information and measurement reporting using SLPP Provide Location Information Reporting.

    - Hybrid Uu and SL Positioning

RAN2 has confirmed combination of Uu-and PC5-based positioning/hybrid Uu and PC5 based positioning is one of the supported operation scenarios. A typical scenario for hybrid positioning is shown in Figure 2, where a target-UE obtains measurements or transmits the reference signals for positioning on both Uu and PC5 interfaces.

Figure 2: A typical scenario for hybrid Uu and SL positioning

Hybrid Uu and SL positioning can be initiated from standalone Uu or sidelink (SL) positioning when current Uu or SL positioning is not possible or cannot satisfy the requirements of the positioning/location service. Further, the hybrid positioning may be initiated during any specific time in the process of current standalone Uu or sidelink positioning procedure. Specifically, when the signal quality of existing Uu/PC5 link is lower than a pre-defined threshold during a period of time, when the received location information reports indicates related failure cause, or when the position estimate results cannot satisfy the QoS of positioning service, the hybrid Uu and PC5-based positioning may be initiated to enhance the performance. Correspondingly, related signalling and protocol to support hybrid Uu and SL positioning are also suggested to be discussed. In addition, it would be also beneficial to understand the coordination between performing Uu and SL positioning measurements, given the potentially different measurement and processing behaviours for Uu and SL. However, further RAN1 input may be required on this aspect.

Proposal 16: Hybrid Uu and SL positioning can be initiated from standalone Uu positioning or standalone SL positioning when either Uu or SL positioning is not possible or cannot satisfy the positioning requirements/QoS. RAN1 feedback may be required for any measurement and processing impacts from hybrid Uu and SL positioning.

# SL Positioning Configuration and Resource Allocation

## General

In the context of the Uu positioning, the LMF is a key central entity coordinating the PRS/SRS resource configuration with the serving and neighbouring gNBs via the NRPPa interface, while in SL the gNB is responsible for the configuration and resource allocation of all SL UEs, while in-coverage. In order to avoid additional specification efforts, it is recommended that SL positioning resource allocation mechanism reuses many of the principles that already exist to enable SL communications.

Observation 3: For Uu positioning, the LMF coordinates the transmission configuration of PRS/SRS, while for in-coverage SL UEs, the gNB is the key configuration entity.

## Configuration

The SL positioning configuration may comprise of all necessary parameters required to support a SL positioning/ranging session. This may include not only time-frequency resources or other physical-layer parameters, but also the type of configured positioning techniques, selected and configured anchor nodes, QoS information, and additional assistance information that may assist the configuration node with the SL positioning configuration.

During the RAN1#112 meeting [11], the following aspects were agreed upon in relation to SL Positioning Resource allocation.

- Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH). Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further). Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Therefore, it is recommended that RAN2 confirm some high-level principles regarding the resource configuration of UEs performing SL Positioning. Unlike Uu positioning, the LMF is not involved in resource coordination as already agreed by RAN1. Therefore tight coordination is required between SL positioning UEs and the gNB in the case of Mode 1 resource allocation. In general, different aspects regarding SL positioning configuration for one or more UEs via UE-specific signalling or broadcast signalling should be further discussed.

Proposal 17: RAN2 to further study the SLPP transmission of the SL positioning configuration depending on the type of configuration node and SL positioning mode (e.g., UE-based or UE-assisted), e.g., gNB, RSU/Anchor-UE, Target-UE by considering at least:

- UE-specific (via dedicated signalling) or common (via broadcast/groupcast signalling, e.g., SIB) SL positioning configurations.

- UE assistance information to assist the configuration node, e.g., gNB, LMF, UE in supporting SL positioning procedures.

## Resource Allocation Mode 1 and Mode 2

It should be possible to extend the current Mode 1 and Mode 2 resource allocation modes in SL to configure SL PRS in all coverage scenarios, i.e., in-coverage, partial coverage and out-of-coverage as already agreed by RAN1. The following resource allocation-related agreements were made by RAN1 during the RAN1#112bis-e meeting [12]:

Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool. Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Modification 6: For the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - Option 3: Provided by higher layers with values TBD - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%): - Options TBD - Modification 8: For the pre-emption of the reserved resources: - Options TBD - Note 1: Other potential modifications and/or other options within each modification are not precluded - Note 2: Multiple options for each potential modification may be supported Agreement In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

Scheme 1 offers a centralized approach to configure SL PRS for in-coverage and partial coverage scenarios, while Scheme 2 enables SL PRS configuration by UEs in out-of-coverage scenarios.

According to this resource allocation mode, the gNB may configure the SL PRS resources in a centralised manner depending on the configured positioning technique. For techniques requiring relative position estimation amongst multiple pair of UEs (e.g., between an anchor node UE and target-UE), gNB-based resource coordination should be sufficient. The gNB may also coordinate SL PRS for multiple anchor node UEs when performing triangulation using timing-based methods such as SL-TDoA or angular-based position estimation, for the purposes of absolute location estimation.

In addition, depending on the type of location request the gNB may configure one of the following:

- Dynamic grants for one shot SL positioning estimates (based on one shot SL PRS transmissions) (e.g., in the case of immediately triggered requests).

- Configured grant schemes in the case that periodic location requests are required, e.g., Type 1 and 2 CG grants (based on periodic SL PRS transmissions).

Proposal 18: RAN2 to further study the impacts of Scheme 1 coordination & signalling of SL PRS resources for one or more UEs participating in a SL positioning session (e.g., one or more anchor UEs and a target-UE) in line with RAN1’s agreement.

Mode 2 is beneficial for scenarios not requiring network infrastructure, where the Initiator UE (UE starting/initiating the SL positioning session) may autonomously select and reserve resource for the purposes of SL positioning. This relies on a pre-configuration of resource pools within a UE. Given the distributed nature of this scheme, Mode 2 can ensure stability of SL PRS for a given period by selecting and reserving resources for SL PRS transmission. However, this may depend on the type of SL PRS configuration (e.g., bandwidth, number of symbols, etc.).

The resource selection triggers may be different in the case of SL PRS when compared to the existing schemes for SL data (TB) transmission, where the selection is dependent on the generation of a new TB or a pre-emption mechanism depending on the SL traffic priority. Similarly, in the case of SL Positioning, the QoS may also have an impact on the resource selection mechanism in Mode 2. The sensing window should also be able to accommodate a SL PRS transmission based on the accuracy requirements of the SL positioning service request.

Proposal 19: RAN2 to further study Scheme 2 coordination of SL PRS resources in line with RAN1’s agreement for one or more UEs participating in a SL positioning session e.g., one or more anchor UEs and a target-UE) including at least:

- The use of pre-configured resources in out-of-coverage scenarios.

- Initiator UE (e.g., target UE) indicates the SL-PRS resource to other UEs involved in SL Positioning (e.g., one or more anchor UEs) over sidelink.

# SL Measurement and Reporting

SL positioning measurements can be performed by the target-UE in a similar fashion to the current Uu positioning framework. The target-UE is usually configured separately to report certain measurements based on the positioning method and can also be extended to the SL positioning framework. In the context of relative positioning, it can be expected that the SL positioning measurements may be performed either by the Initiating or Responding UEs, depending on the type of configured positioning methods, e.g., the two-way SL-RTT requires that both the Initiator and Responder UE perform the timestamp measurements for enhanced accuracy.

It can be noted that depending on the scenario, one or multiple entities may have different responsibilities in terms of transmitting the SL positioning configuration, performing measurements, reporting (if required, e.g., UE-based methods would require optional reporting) and finally the position calculation. It can be further observed that the measurement node/entity may receive the SL-PRS transmissions and then performs the associated SL positioning measurements.

Proposal 20: RAN2 to consider the supported signalling and procedures depending on which node/entity:

- Performs the SL positioning measurements and associated reporting (e.g., if needed for UE-based like positioning), e.g., RSU/Anchor-UE, Target-UE.

- Performs the SL positioning calculation e.g., gNB, LMF, RSU/Anchor-UE, Target-UE.

NOTE: This has a dependency on the SL Positioning architecture as well as positioning modes (e.g., UE-assisted, UE-based).

The SL Positioning framework should also be capable of supporting different types of reporting of both SL positioning measurements and absolute/relative location estimate. Table 1 shows the various reporting types depending on the reporting configuration.

Table 1: Types of SL Positioning Reporting

| SL Positioning Reporting Types | Overview |
| --- | --- |
| One-shot | - The UE/device performing measurements may report to the positioning calculation entity in a one-shot manner to support immediate reporting. - The UE (positioning calculation) entity may report the absolute/relative location estimate to the requesting node/network entity, e.g., UE, LMF, etc. |
| Triggered | - The UE/device performing measurements may report to the positioning calculation entity based on defined event-based criteria, e.g., based on a location change/cell change. - The UE/device performing positioning calculation may report the location estimate to the requesting entity upon trigger of a configured event. |
| Periodic | - The UE/device may provide periodic positioning measurement or location estimate reports with the same time interval between two consecutive reports. |

Proposal 21: Support different SL Positioning reporting types including one-shot, triggered and periodic reports.

# Conclusion

The following observations regarding potential SL positioning solutions have been noted as part of the discussion:

Observation 1: For UEs wishing to only perform ranging in-coverage and without LPP capabilities, LMF involvement may be unnecessary.

Observation 2: Anchor nodes with known locations such as UEs and RSUs can enable certain SL positioning techniques such as SL-TDoA.

Observation 3: For Uu positioning, the LMF coordinates the transmission configuration of PRS/SRS, while for in-coverage SL UEs, the gNB is the key configuration entity.

The proposals relating to the SL Positioning framework are summarized as follows:

Proposal 1: RAN2 to confirm the NR PC5 link between UE-C and UE-B as part of the NR-only partial coverage scenario in the SL Positioning baseline architecture.

Proposal 2: RAN2 to remove the “FFS LTE-Uu” link between UE-B and ng-eNB.

Proposal 3: RAN2 to support both LMF-dependent (network-assisted operation as defined by SA2) (e.g., UEs supporting both LPP and SLPP) for in-coverage scenarios and partial coverage scenarios and LMF-independent (UE-only operation as defined by SA2) (e.g., UEs only supporting SLPP) SL positioning architectures for all coverage scenarios including in-coverage, partial coverage and out-of-coverage.

Proposal 4: It is suggested that RAN2 to discuss the procedures and signalling design for partial coverage case in which a target UE is out-of-coverage and at least one of the anchor UEs is in coverage.

Proposal 5: From RAN2 perspective, there is no separate “Located UE” to be discovered as the anchor UE may choose to provide its own location to the higher-layer discovery procedures, if required provided that there are no privacy concerns. The anchor UE (Reference UE) location may be optionally provided in the SLPP metadata field.

Proposal 6: RAN2 to confirm indication of SLPP support as part of the SLPP metadata field, in addition to the UE roles.

Proposal 7: RAN2 is recommended to support procedures related to anchor UE(s) including triggering, (re)selection, and configuration when performing sidelink positioning procedures.

Proposal 8: Anchor UE selection radio-based criteria may be based on at least reported LOS/NLOS indication, reported SL measurements, e.g., SL PRS RSRP, coverage of anchor UEs, synchronization related information, specific UE type. FFS any further Anchor UE selection criteria.

Proposal 9: RAN2 to confirm the first and second working assumption made during RAN2#121bis-e on group positioning involving multiple target UEs.

Proposal 10: RAN2 to further discuss the remaining aspects, which are in scope of the SLPP Layer Group management, e.g., addition/re-selection/removal of SL positioning UEs.

Proposal 11: RAN2 to further study whether existing/new SL ID management needs to be supported for SL positioning to support new UE-types such as anchor UEs and/or server UEs.

Proposal 12: RAN2 to consider support of “SL Positioning Server UE” as a functionality part of an Anchor UE or Target-UE.

Proposal 13: RAN2 to confirm with SA2 on the supported location services for SL positioning including NI-LR, Immediate Location requests and Deferred location requests which can be re-used as in Uu positioning. FFS the applicability of these location service requests under different coverage scenarios.

Proposal 14: RAN2 to mention in SA2 reply LS that RAN1 has confirmed the relative velocity QoS parameter.

Proposal 15: RAN2 to support the signalling design for the agreed RAN1 SL Positioning measurements including measurement configuration using SLPP Request Location Information and measurement reporting using SLPP Provide Location Information Reporting.

Proposal 16: Hybrid Uu and SL positioning can be initiated from standalone Uu positioning or standalone SL positioning when either Uu or SL positioning is not possible or cannot satisfy the positioning requirements/QoS. RAN1 feedback may be required for any measurement and processing impacts from hybrid Uu and SL positioning.

Proposal 17: RAN2 to further study the SLPP transmission of the SL positioning configuration depending on the type of configuration node and SL positioning mode (e.g., UE-based or UE-assisted), e.g., gNB, RSU/Anchor-UE, Target-UE by considering at least:

- UE-specific (via dedicated signalling) or common (via broadcast/groupcast signalling, e.g., SIB) SL positioning configurations.

- UE assistance information to assist the configuration node, e.g., gNB, LMF, UE in supporting SL positioning procedures.

Proposal 18: RAN2 to further study the impacts of Scheme 1 coordination & signalling of SL PRS resources for one or more UEs participating in a SL positioning session (e.g., one or more anchor UEs and a target-UE) in line with RAN1’s agreement.

Proposal 19: RAN2 to further study Scheme 2 coordination of SL PRS resources in line with RAN1’s agreement for one or more UEs participating in a SL positioning session e.g., one or more anchor UEs and a target-UE) including at least:

- The use of pre-configured resources in out-of-coverage scenarios.

- Initiator UE (e.g., target UE) indicates the SL-PRS resource to other UEs involved in SL Positioning (e.g., one or more anchor UEs) over sidelink.

Proposal 20: RAN2 to consider the supported signalling and procedures depending on which node/entity:

- Performs the SL positioning measurements and associated reporting (e.g., if needed for UE-based like positioning), e.g., RSU/Anchor-UE, Target-UE.

- Performs the SL positioning calculation e.g., gNB, LMF, RSU/Anchor-UE, Target-UE. NOTE: This has a dependency on the SL Positioning architecture as well as positioning modes (e.g., UE-assisted, UE-based).

Proposal 21: Support different SL Positioning reporting types including one-shot, triggered and periodic reports.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, RAN#98-e, Dec. 2022.

- R2-2301904, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), RAN2#121, Feb. 2023.

- R2-2304204, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), RAN2#121bis-e, Apr. 2023.

- R2-2304301, “Report of [AT121bis-e][428][POS] Sidelink positioning stage 2 (CATT)”, RAN2#121bis-e, Apr. 2023.

- R2-2300084, “Reply LS to LS on SL positioning groupcast and broadcast”, RAN2#121, SA3, Feb. 2023.

- R2-2300079, “Reply LS on SL positioning groupcast and broadcast”, RAN2#121, SA2, Feb. 2023.

- R2-2302255, “Reply LS on RAN dependency for Ranging & Sidelink Positioning”, RAN2#121, Feb. 2023.

- S2-2305748, Support of MT-LR for Ranging and Sidelink Positioning, CR, SA2#156e, Apr. 2023.

- S2-2305749, Ranging and Sidelink Positioning MO-LR procedure, CR, SA2#156e, Apr. 2023.

- R1-2304152, “Reply LS on RAN dependency for Ranging & Sidelink Positioning”, RAN1#112bis-e, Apr. 2023.

- R1-2302065, “Session notes for 9.5 Expanded and Improved NR Positioning”, Ad-Hoc Chair (Huawei), RAN1#112, Feb. 2023.

- R1-2302065, “Session notes for 9.5 Expanded and Improved NR Positioning”, Ad-Hoc Chair (Huawei), RAN1#112bis-e, Apr. 2023.