3GPP TSG-RAN WG2 Meeting #122	R2-2305416

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.25.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Reply LS proposal for C1-232944/R2-2304606 (LS on NAS-AS interaction in terms of NS-AoS)

WID/SID:	eNS_Ph3 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

CT1 sent an LS in C1-232944 / R2-2304606 on NAS-AS interaction in terms of NS-AoS. This paper proposes a reply to this CT1 LS.

# 2	Discussion

According to the LS in C1-232944 / R2-2304606 on NAS-AS interaction in terms of NS-AoS CT1 is working on stage 3 implementation in terms of NS-AoS specified in TS 23.501. This feature requires the NAS layer of the UE to know whether the UE is in an NS-AoS or not.

Observation 1: The UE NAS should be aware whether the UE is inside or outside of NS-AoS in all RRC states.

Two alternative solutions were discussed in CT1:

Alternative 1

The NAS layer forwards the S-NSSAI location availability information to the AS layer. Based on the S-NSSAI location availability and the UE’s serving cell, the AS layer indicates whether the UE is inside or outside an NS-AoS to the NAS layer.

Alternative 2

The NAS layer receives information of the UE’s serving cell from the AS layer.

In the current specifications NAS in the UE is not aware of the cell level location information of the UE. Therefore alternative 2 requires introducing new interactions between the AS and NAS layers in the UE to make the NAS layer aware of the serving/camping cell of the UE.

Observation 2: Alternative 2 requires the AS in the UE to inform the NAS when the UE mobility (e.g., cell reselection) results in a change in the serving or camping cell.

The current functional division between NAS and AS layers in the UE is that AS layer receives the necessary information form NAS that should be observed (e.g., list of TACs of the RA) during mobility, and only notifies the NAS about mobility when NAS level actions are needed (e.g., when the UE moves in a TA that is not part of the current RA). We think that alternative 1 is more aligned with current functional division between NAS and AS in the UE. Furthermore, Alternative 1 requires less frequent interactions between the NAS and AS layers.

Proposal 1: RAN2 replies to CT1 that from RAN2 perspective alternative 1 is the preferred option. (Draft Reply LS can be found in the Annex.)

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: The UE NAS should be aware whether the UE is inside or outside of NS-AoS in all RRC states.

Observation 2: Alternative 2 requires the AS in the UE to inform the NAS when the UE mobility (e.g., cell reselection) results in a change in the serving or camping cell.

Proposal 1: RAN2 replies to CT1 that from RAN2 perspective alternative 1 is the preferred option. (Draft Reply LS can be found in the Annex.)

# Annex:	Draft Reply LS

3GPP TSG-RAN WG2 Meeting #122	DRAFT R2-230xxxx

Incheon, South Korea, 22– 26 May 2023

Title:	[DRAFT] Reply LS on NAS-AS interaction in terms of NS-AoS

Response to:	C1-232944 / R2-2304606

Release:	Release 18

Work Item:	eNS_Ph3

Source:	[TSG RAN WG2]

To:	CT WG1

Cc:	SA WG2

Contact Person:

#### Name:	György Wolfner

E-mail Address:	gyorgy<dot>wolfner<at>nokia<dot>com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN WG2 thanks for the LS on NAS-AS interaction in terms of NS-AoS (C1-232944 / R2-2304606).

RAN2 has discussed the issue and concluded that alternative 1 is the preferred option from RAN2 perspective, as it is more aligned with the current functional division between NAS and AS layers in the UE.

2. Actions:

To CT WG1

ACTION: 	RAN2 respectfully asks CT1 to take the above into considerations.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#123	from 2023-08-21	to 2023-08-25		Toulouse, FR

RAN2#123bis	from 2023-10-09	to 2023-10-13		Xiamen, CN