3GPP TSG-RAN WG2 Meeting #122	R2-2305417

# Incheon, South Korea, 22– 26 May 2023													(R2-2302863)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | 0124 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR M3 measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Immediate MDT in NR , measurement M3 has been voided. Also stage 3 specification TS32.422 defines: - M3 is not supported by this release However, in Measurement collection triggers, a guideline remeins to collect M3 till the end of measurement collection period: For M3: - End of measurement collection period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement collection trigger for NR M3 is voided Impact analysis Impacted functionality: Immediate MDT Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no interoperability issue. - If the UE is implemented according to the CR and the network is not, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the changes are not approved, stage 2 and stage 3 specifications are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.4.1.1	Measurements and reporting triggers for Immediate MDT

Measurements to be performed for Immediate MDT purposes involve reporting triggers and criteria utilized for RRM. In addition, there are associated network performance measurements performed in the gNB.

In particular, the following measurements shall be supported for Immediate MDT performance:

Measurements:

⁻	M1: DL signal quantities measurement results for the serving cell and for intra-frequency/Inter-frequency/inter-RAT neighbour cells, including cell/beam level measurement for NR cells only, TS 38.215 [19].

⁻	M2: Power Headroom measurement by UE, TS 38.213 [20].

⁻	M3: Void.

⁻	M4: PDCP SDU Data Volume measurement separately for DL and UL, per DRB per UE, see TS 28.552 [17].

⁻	M5: Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, see TS 28.552 [17].

⁻	M6: Packet Delay measurement separately for DL and UL, per DRB per UE, TS 28.552 [17] and TS 38.314 [18].

NOTE 0:	UL PDCP Excess Packet Delay measurement can be configured with a threshold as specified in TS 38.331 [15].

⁻	M7: Packet loss rate measurement separately for DL and UL, per DRB per UE, TS 28.552 [17] and TS 38.314 [18].

⁻	M8: RSSI measurement by UE (for WLAN/Bluetooth measurement) see TS 38.331 [15].

⁻	M9: RTT Measurement by UE (for WLAN measurement) see TS 38.331 [15].

NOTE 1:	Void

NOTE 1a:	M5 ~ M7 can apply to MR-DC and EN-DC SN terminated MCG/split bearers and MN terminated SCG/split bearers.

Measurement collection triggers:

-	For M1:

-	Event-triggered measurement reports according to existing RRM configuration for events A1, A2, A3, A4, A5, A6, B1 or B2.

-	Periodic, A2 event-triggered, or A2 event triggered periodic measurement report according to MDT specific measurement configuration.

-	For M2:

-	Reception of Power Headroom Report (PHR) according to existing RRM configuration.

NOTE 2:	PHR is carried by MAC signalling. Thus, the existing mechanism of PHR transmission applies, see TS 38.321 [21].

-	For M3:

-	Void.

-	For M4:

-	End of measurement collection period.

-	For M5:

-	End of measurement collection period.

NOTE 3:	If transmission of a data burst is ongoing at the boundary of the measurement collection period, T1 and T2 in throughput evaluations are set to the end and the start of the measurement period, respectively.

-	For M6:

-	End of measurement collection period.

-	For M7:

-	End of measurement collection period.

-	For M8:

-	Associated to M1 and/or M6 related measurement reporting triggers.

-	For M9:

-	Associated to M1 and/or M6 related UE measurement reporting triggers.

End of Changes