3GPP TSG-RAN WG2 Meeting #122	R2-2305419

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.9.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on reply LSs on RSRP issues (R1-2304211 / R2-2304617 and R4-2306366 / R2-2304637)

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

Both RAN1 and RAN4 provided reply LSs (R1-2304211/R2-2304617 and R4-2306366/R2-2304637) on comparison of SL-RSRP and SD-RSRP (LS sent in R2-2302234).

This paper discusses the consequences of the replies received from RAN1 and RAN4 and proposes a way forward.

# 2	Discussion

In R1-2304211/R2-2304617 RAN1 provide the following answer to RAN2:

RAN1’s understanding is that comparison of SL-RSRP and SD-RSRP measurement cannot be used for the purposes of triggering a measurement report at least due to the above outlined issues, and the decision on whether to use comparison of SL-RSRP and SD-RSRP measurement is up to RAN2.

Moreover, RAN1 explained that comparison of SL-RSRP and SD-RSRP measurement is affected by at least the following issues:

- Unicast is subject to transmit power control based on SL pathloss if parameter sl-P0-PSSCH-PSCCH is provided, while broadcast is not subject to this power control mechanism.

- Due to CBR-based power control, if configured, maximum transmit power may depend on the priority of the transmission. The priority for discovery messages may be different from the priority used for the transmissions over which SL-RSRP is measured.

- Transmission of discovery messages can take place in dedicated discovery pools; these may be configured differently, e.g. with respect to power control settings, from the pool(s) on which SL-RSRP is measured.

- Note: Sidelink carrier aggregation, for which an objective was added to Rel-18 NR_SL_enh2 at RAN#99, may additionally impact SL transmit power. 
Note: RAN1 has not started work on SL carrier aggregation.

In R4-2306366/R2-2304637 RAN4 provide the following answer to RAN2:

Comparison of SL-RSRP and SD-RSRP cannot be used to trigger a measurement report due to transmission power difference between the reference signals on which the UE is measuring SL-RSRP and SD-RSRP, unless the difference caused by transmission power control mechanism can be properly handled.

It is clear from both RAN1 and RAN4 replies that the transmit power control in unicast based on SL pathloss as opposed to no transmit power control in sidelink discovery broadcast message provides difference between SL-RSRP and SD-RSRP. Additionally, the transmit power of discovery message may vary due to CBR-based and/or resource pool based maximum TX power.

As indicated in the replies from either working group (especially the technical issues listed in the RAN1 reply LS) the introduction of a new Z2 event that is based on the comparison of SL-RSRP and SD-RSRP measurements is not straightforward and thus would require additional considerations and may require some cooperation with RAN1 and/or RAN4.

Observation 1: Specifying a Z2 event is not trivial due to the power imbalance between SL-RSRP and SD-RSRP measurements.

Proposal 1: RAN2 discuss how the comparison of SL-RSRP and SD-RSRP considering the different transmission powers can be handled in the new Z2 event.

The technical issues related to the comparison of SL-RSRP and SD-RSRP measurements are not limited to the introduction of Z2 event. The problems are also valid when relay selection or reselection happen in UE-to-UE relay and in UE-to-NW scenarios. Therefore, it would be reasonable to consider the issues above generally during RAN2 Rel-18 work and avoid any solutions that require the comparison of SL-RSRP and SD-RSRP measurements without handling the differences coming from the different transmission power.

Proposal 2: RAN2 agrees that the comparison of SL-RSRP and SD-RSRP measurements without handling the differences coming from the different transmission power should not be used during the UE-to-UE relay selection and reselection.

Proposal 3: RAN2 agrees that the comparison of SL-RSRP and SD-RSRP measurements without handling the differences coming from the different transmission power should not be used during the UE-to-NW relay selection and reselection.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Specifying a Z2 event is not trivial due to the power imbalance between SL-RSRP and SD-RSRP measurements.

Proposal 1: RAN2 discuss how the comparison of SL-RSRP and SD-RSRP considering the different transmission powers can be handled in the new Z2 event.

Proposal 2: RAN2 agrees that the comparison of SL-RSRP and SD-RSRP measurements without handling the differences coming from the different transmission power should not be used during the UE-to-UE relay selection and reselection.

Proposal 3: RAN2 agrees that the comparison of SL-RSRP and SD-RSRP measurements without handling the differences coming from the different transmission power should not be used during the UE-to-NW relay selection and reselection.