3GPP TSG-RAN WG2 Meeting #122	R2-2305423

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	SPR and SHR related enhancements

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# Introduction

At RAN2#121 the following agreements with respect to SPR were made:

- UE includes available location information in SPR.

- UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched

- At least the following options are needed for releasing SPR report:

  - New SPR is initiated

  - Upon retrieval of SPR

  - Detach is initiated

- In SPR, reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

Additionally, in RAN2#120 agreed the following with respect to SPR configuration and retrieval:

- Only MN can retrieve the SPR from the UE.

- Regarding the configuration of the UE, RAN2 agree following options depending on which of nodes initiates SPR, i.e.:

  - For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

  - For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

  - T304 trigger needs to be configured by the target SN node.

Furthermore, some more SPR enhancements (other than LS-related discussions) were agreed as well:

Agreements:

- UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

- UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

- Only the latest successful PSCell change/addition is reported by the UE.

- Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

- UE records/reports PCell SHR and PSCell SPR separately

The following content was also agreed for SPR in RAN2#119bis:

UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

B.SPR is fetched via UE Information Request/Response procedure.

In this contribution, we discuss enhancements for SPR/SHR regarding the generation of the reports and SPR reporting mechanism to the network, and enhancements for SPR content that enable effective root cause analysis.

# 2	Discussion

## 2.1	SPR generation trigger

The application of SPR for conditional PSCell addition and change (CPAC) was considered in the discussion at the last meeting RAN2#120 with focus on optimization of the conditional reconfiguration of a PSCell addition/change.

When conditional PSCell change is configured, the UE starts evaluation of the prepared target PSCells according to the configured execution condition. Within the RLM process of the serving PSCell the signal quality is monitored by means of lower layer indications. If signal quality deteriorates during the evaluation phase that out-of-sync indications from lower layers are triggered and timer T310 is started, the mobility procedure is jeopardized to fail. If the execution condition of the conditional PSCell change is met before T310 expires, the UE avoids RLF. The conditional PSCell change was successful without making the network aware how risky cell change was. For such cases, SHR has been agreed as addition means for MRO, and SPR for the PSCell changes in NR-DC context is the natural equivalent to SHR.

Since only crucial issues during the cell change procedure should be addressed with SPR (as it is for SHR), a certain degree of outage, which is measured by exceeding a percentage threshold of the timer resulting in an RLF with its expiry; e.g., the parameter thresholdPercentageT310, triggers the logging of a risky maneuver in the SPR.

Observation 1.1: Outages exceeding a certain percentage of failure criterion during the mobility procedure are triggering an SPR.

### 2.1.1	SPR for “almost too late” CPC execution

The case that the UE escapes with CPC execution from a RLF while T310 running (as depicted in Figure 1) is the intended case from MRO enhancement perspective. SPR was triggered by passing thresholdPercentageT310 (here exemplarily is configured with 40% of T310 duration) and CPC execution happens before T310 expires. Even though the CPC was successful, the SPR provides information of an “almost too late” execution, which can be used for MRO refinements.

Observation 1.2: SPR of outages where CPC execution rescued the UE are relevant MRO refinements.

Figure 1: SPR triggered while UE rescued by CPC execution and successful handover

However, it can also happen that RLM stops the timer T310 itself, since radio signal quality returns to reliable in-sync condition (i.e., lower layers are in-sync indications) by passing the Qin threshold after the SPR was triggered by passing thresholdPercentageT310. In this case, the UE has a robust link quality at the moment of the CPC execution (see Figure 2) and, therefore the SPR was not created when RLF almost happened as above. Therefore, the reported SPR will be wrongly interpreted and categorized as “almost too late” execution.

Observation 1.3: A SPR is created even though T310 is stopped by RLM and might be therefore wrongly interpreted.

Observation 1.4: The current SPR (also SHR) does not allow a distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Figure 2: SPR has been triggered but not related to the cell change

Proposal 1.1: RAN2 agrees that current SPR procedure (as well as SHR procedure) does not provide means to distinguish the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Proposal 1.2: RAN2 agrees to means how SPR (as well as SHR procedure) should be enhanced to enable the distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

## 2.2	SPR reporting mechanism

According to RAN2#119bis-e agreement the UEInformationRequest/UEInformationResponse mechanism will be used to fetch the SPR from the UE. The UE shall send the SPR report if available in the UEInformationResponse only when triggered by the UEInformationRequest. The UEInformationRequest with respect to SPR is sent to the UE only if UE has previously indicated the availability of SPR to the network.

SPR availability could be indicated using legacy mechanism such in the case of SHR, where the report availability is sent to the network via the RRCReconfigurationComplete message. However, in the dual connectivity scenario (as depicted in Fig. 3) during a PSCell change the RRCReconfigurationComplete message is sent to MN right after receiving the RRCConnectionReconfiguration and before the PSCell is accessed (successfully or not). At that stage the UE cannot know in advance if any of the configured thresholds for SPR generation have already been exceeded and thus cannot generate SPR.

Figure 3: SN change procedure – MN initiated (TS 37.340 Fig. 10.5.2-1)

Observation 2: SPR availability cannot be indicated to the network by using RRCReconfigurationComplete message as the latter is sent to MN too early.

Proposal 2: RAN2 discusses a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network.

## 2.3	SPR content enhancements

### 2.3.1	Root cause analysis

PSCell changes for the UE may be initiated by either the MN or the SN. However, which node should be in charge of carrying out root cause analysis in case any problems arise is still FFS in RAN3. Following the principles defined in Rel-17, we believe it is reasonable that the initiating node should also carry out the root cause analysis following a (near) failure that is captured in the SPR.

Proposal 3.1: The initiating node of PSCell change should also carry out the root cause analysis based on the contents of the SPR.

In the case of SN-initiated PSCell change, the source SN configures the SPR and the MN will signal this to the UE. A further RAN2#120 agreement then mandates the MN to retrieve the SPR once generated by the UE. The next logical step would then be for the MN to forward the retrieved SPR to the SN for further root cause analysis. In case there was also an MN change, the Target MN may need to forward the SPR to the source MN forehand.

Proposal 3.2: MN may forward the SPR to the initiating node for root cause analysis.

In order for the MN to correctly decide whether and to which node it needs to forward the retrieved SPR, it needs to keep track of the initiating node for each PSCell change or determine this based on available information (e.g., UE context). This may not always be possible, i.e., the MN does not record the initiating node or the UE context may be released and the SPR retrieved by a different MN and then forwarded to the source MN. In the latter case, the retrieving MN (Target MN) may not know either to which MN (source MN) it needs to forward the SPR.

Proposal 3.3: RAN2 to discuss solutions for determining the initiating node for PSCell change associated with an SPR to enable the forwarding of the SPR to the initiating node for root cause analysis.

### 2.3.2	User plane interruption time

In Rel-17 for Successful Handover Report (SHR) it was standardized that user plane interruption time is to be recorded, but only for DAPS handovers. Moreover, in RAN2 meetings in Rel-18 it has been further proposed by companies (see R2-2208177) that SHR should also contain user plane interruption time measurements for legacy or conditional handovers, which can help the network in decisions such as for instance to identify whether to configure DAPS bearers or not.

Following a similar discussion, user plane interruption time can also occur when the UE is operating in dual connectivity, for instance during a PSCell change, where there might happen that the UE is experiencing user plane interruption due to this change. The user plane interruption time during a PSCell change can occur on data radio bearers of different cell group types, as various possible cell group types can be configured at the UE e.g., MCG, SCG and split bearers. In turn, each of these cell group types can be SN or MN terminated. Therefore, if user plane interruption time is not reported correctly at the network side, MRO algorithms would not possess the adequate inputs, which can further lead to wrong decisions with respect to the bearers UE should be configured with in the future.

Observation 3: Current SPR content does not allow the network to accurately obtain information of user plane interruption time on a per cell group type granularity during a PSCell change.

Proposal 3.4: RAN2 to discuss the possibility of including user plane interruption time measurements on a per cell group type in SPR.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1.1: Outages exceeding a certain percentage of failure criterion during the mobility procedure are triggering an SPR.

Observation 1.2: SPR of outages where CPC execution rescued the UE are relevant MRO refinements.

Observation 1.3: A SPR is created even though T310 is stopped by RLM and might be therefore wrongly interpreted.

Observation 1.4: The current SPR (also SHR) does not allow a distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Proposal 1.1: RAN2 agrees that current SPR procedure (as well as SHR procedure) does not provide means to distinguish the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Proposal 1.2: RAN2 agrees to means how SPR (as well as SHR procedure) should be enhanced to enable the distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Observation 2: SPR availability cannot be indicated to the network by using RRCReconfigurationComplete message as the latter is sent to MN too early.

Proposal 2: RAN2 discusses a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network.

Proposal 3.1: The initiating node of PSCell change should also carry out the root cause analysis based on the contents of the SPR.

Proposal 3.2: MN may forward the SPR to the initiating node for root cause analysis.

Proposal 3.3: RAN2 to discuss solutions for determining the initiating node for PSCell change associated with an SPR to enable the forwarding of the SPR to the initiating node for root cause analysis.

Observation 3: Current SPR content does not allow the network to accurately obtain information of user plane interruption time on a per cell group type granularity during a PSCell change.

Proposal 3.4: RAN2 to discuss the possibility of including user plane interruption time measurements on a per cell group type in SPR.